3GPP TSG-RAN WG4 Meeting # 102-e	R4-2206821

# Electronic Meeting, 21st February – 3rd March, 2022

Agenda item:	5.1.3

Source:	Intel Corporation

Title:	WF on maintenance to R16 POS requirements

Document for:	Approval

- Maintenance to RRM Core requirements

Issue 1-1-1: If the SRS and PRS proximity condition is not met, how does RAN4 specify the UE measurement and report requirements (Core)?

Agreement:

- If the SRS and PRS proximity condition is not met, UE Rx-Tx measurement and reporting requirements do not apply.

  - It is UE implementation whether the UE transmit the UE Rx-Tx measurement results to LMF or not

Issue 1-1-2: If the SRS and PRS proximity condition is not met, what about accuracy requirements (Perf.)?

Agreement:

- If the SRS and PRS proximity condition is not met, UE Rx-Tx measurement accuracy requirements do not apply.

Issue 1-2-1: When SRS is reconfigured without cell change during the measurement period, UE shall restart the UE Rx-Tx time difference measurement after the SRS reconfiguration on the target cell is complete.

Agreement:

- When SRS is reconfigured without serving cell change during the measurement period, UE shall restart the UE Rx-Tx time difference measurement after the SRS reconfiguration is complete.

- Maintenance to RRM Perf. Requirements

Issue 2-1-1: Group delay calibration margin for RSTD measurement accuracy in FR1

Agreement:

- Group delay calibration margin for RSTD measurement accuracy in FR1:

| PRS BW (MHz) | Margin (Tc) |
| --- | --- |
| ≥ 5 | TBD |
| ≥ 10 | TBD |
| ≥ 20 | [36] |
| ≥ 50 | [16] |
| ≥ 100 | [12] |

Issue 2-1-2: Group delay calibration margin for RSTD measurement accuracy in FR2

Agreement:

- Group delay calibration margin for RSTD measurement accuracy in FR2

| PRS BW (MHz) | Margin (Tc) |
| --- | --- |
| ≥ 20 | TBD |
| ≥ 50 | [32] |
| ≥ 100 | [16] |
| ≥ 200 | [12] |

Issue 2-1-3: Frequency drift margin for RSTD measurement accuracy requirements

GTW Agreement:

- Specify a frequency drift margin of 32 Tc for RSTD that applies for a maximum time offset of 160 msec between the PRS resource instances used to calculate the RSTD measurement for the case of a single PFL in FR1 and FR2.

- Specify a frequency drift margin of [256 Tc] for RSTD that applies for a maximum time offset of [1.28 second] between the PRS resource instances used to calculate the RSTD measurement for the case of multiple PFLs.

Issue 2-2-1: Group delay calibration margin for Rx-Tx measurement accuracy in FR1

Agreement:

- Group delay calibration margin for Rx-Tx measurement accuracy in FR1

| Min(PRS BW, SRS BW) (MHz) | Margin (Tc) |
| --- | --- |
| ≥ 5 | TBD |
| ≥ 10 | [80] |
| x≥ 20 | [56] |
| ≥ 50 | [24] |
| ≥ 100 | [24] |

Issue 2-2-2: Group delay calibration margin for Rx – Tx measurement accuracy in FR2

Agreement:

- Group delay calibration margin for Rx -– Tx measurement accuracy in FR2

| Min(PRS BW, SRS BW) (MHz) | Margin (Tc) |
| --- | --- |
| ≥ 20 | [76] |
| ≥ 50 | [32] |
| ≥ 100 | [24] |
| ≥ 200 | [20] |

Issue 2-2-3: Applicability of Rx-Tx accuracy requirements with autonomous timing adjustment

GTW Tentative agreements:

- Applicability of Rx-Tx accuracy requirements with autonomous timing adjustment is defined as:

  - If the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment

    - Option 1:

      - If the autonomous timing adjustment is below threshold X

        - UE Rx-Tx measurement accuracy requirements shall apply

      - Otherwise

        - UE Rx-Tx measurement accuracy requirements shall apply for a cell, which is also the downlink reference cell (defined in section 7.1.1)

        - UE Rx-Tx measurement accuracy requirements shall not apply for a cell, which is not the downlink reference cell (defined in section 7.1.1) for SRS transmission. UE shall restart the measurement period in such case

    - Option 2:

      - UE Rx-Tx measurement accuracy requirements shall apply for a cell, which is also the downlink reference cell (defined in section 7.1.1)

      - UE Rx-Tx measurement accuracy requirements shall not apply for a cell, which is not the downlink reference cell (defined in section 7.1.1) for SRS transmission. UE [may or shall] restart the measurement period in such case

Issue 2-3-1: PRS-RSRP accuracy under extreme condition

GTW Agreement:

- Introduce PRS-RSRP accuracy requirements for extreme conditions in Rel-16

- The margin for PRS-RSRP accuracy requirements under extreme conditions are:

  - [4.5] dB for absolute accuracy for FR1.

  - [3.0] dB for absolute accuracy for FR2.

  - [1.5] dB for relative accuracy for FR1.

  - [3.0] dB for relative accuracy for FR2.

Issue 2-4-3: whether to specify new UE based DL-TDOA test cases

whether to specify new UE based DL-TDOA test cases.

- Specify new test case for UE-based DL-TDOA measurements:

  - New test case for UE-based DL-TDOA reporting delay is added with limited additional effort, leveraging the existing test case setup for UE-assisted

    - Detailed test configuration is FFS

  - New test case for UE-based DL-TDOA measurement accuracy is added

    - Detailed test configuration is FFS

Tentative agreements: None.

Candidate options:

- Option 1: both of the above tests are added

- Option 2: none of the above test is added