3GPP TSG-RAN WG4 Meeting # 102-e 										R4-2206943

Electronic Meeting, Feb 21st - Mar 3rd, 2022

Agenda item:			10.19.3.1

Source:	Intel

Title:	WF on FeMIMO RRM impact for unified TCI state

Document for:	Approval

# Introduction

This contribution is to capture the agreements for the email discussion for Rel-17 FeMIMO RRM impact for Unified TCI state in RAN4 #102-e meeting.

# Agreements in GTW session

In GTW session, the following bullets have been agreed

Issue 1-3-1 Known cell condition for TCI state switch associated with different PCI

- Agreements

  - Define separate known cell condition for TCI state switch associated with different PCI and known TCI state condition

  - Known cell condition for TCI state switch associated with different PCI

    - Re-use known condition specified for inter-cell beam measurements

  - Known TCI state condition

    - Re-use legacy known TCI state condition

Issue 1-3-3 Whether to define TCI state switch delay requirement for unknown cell case

- Agreements

  - RAN4 will NOT specify the requirements for unknown cell case for TCI switching delay for a cell with different PCI from serving cell in Rel-17

Issue 1-4-2 Whether common TCI state switching delay requirement is defined for all CC or per CC

- Agreement

- Common TCI state switching delay requirement is defined for all CC:

- If the same/single RS (indicated by a common TCI state ID) is used to provide beam information for the set of configured CCs

- Re-use requirement for single-CC.

- The SCS should be the smallest SCS within all CCs;

- take a note in the spec for TCI switching delay requirement in CA case:

    - The requirements of Rel-17 unified TCI switching delay are applicable to CA cases based on the rule of reference BWP/CC selection in TS38.214.

- FFS: if the RS in the TCI state provides QCL-TypeD

- FFS: If different RS in CC set is used to provide beam information, or TCI states involve QCL-A or QCL-C/QCL-TypeB, the requirement be defined per CC respectively.

# Way Forward on Unified TCI state

### 2.1 Sub-topic 1-1: Definition of beam alignment

Issue 1-1-1 Beam alignment assumption if PL-RS is included in UL TCI or joint TCI

- PL-RS is identical to the source RS in UL or joint TCI

Issue 1-1-2 Beam alignment assumption if PL-RS is associated with UL TCI or joint TCI

- PL-RS and source RS in UL or joint TCI are QCL-Type D

### 2.2 Sub-topic 1-2 Switching delay requirements for serving cell

Issue 1-2-1 Requirement applicability of DCI based DL and UL TCI state switching delay

- target TCI state is known

- Note: UL TCI known state will be further clarified

- target TCI state is in active TCI state list for DL and joint TCI switch

- FFS: target TCI state is maintained for UL and joint TCI state switch.

Issue 1-2-2 Define DCI based DL and UL TCI state switching delay as

- Exact wording will be discussed in “DraftCR on DCI based DL and UL TCI switching delay requirements”

Issue 1-2-3 MAC CE based UL TCI state switch when UL TCI switch and RL-RS switch are activated in different MAC CE

- No separate PL-RS switching supported in Rel-17. PL-RS switching delay will be the same as UL TCI state switch delay.

Issue 1-2-4 TCI state-pair indication requirement

- Proposals

  - Option 1(Apple, MTK, Intel, ZTE, Huawei):

    - The TCI state switching delay requirement can be defined for UL TCI and DL TCI switching independently.

  - Option 2 (vivo):

    - Remove the issue here without conclusions since the intention is not clear.

  - Option 3(Samsung, Nokia):

    - No requirements for TCI state-pair indication

Issue 1-2-5 MAC-CE based UL TCI state switching delay when SSB is indicated as PL-RS in UL TCI state for FR2

- Proposals

  - Option 1(Huawei):

    - When a SSB is indicated as PL-RS in a UL TCI state, the scaling factor for beam sweeping needs to be introduced for PL-RS measurement time in FR2.

    - In FR2, the MAC-CE based UL TCI state switching delay need to be separately defined for SSB based PL-RS.

    - In FR2, when a SSB is indicated as PL-RS in a UL TCI state, the MAC-CE based UL TCI state switching delay for both known case and unknown case can be defined as:

    - THARQ + 3ms + NM*(5*TL1-RSRP_SSB + 2ms) with the assumption of M=1.

    - Where NM = 1, if the target PL-RS is not maintained by the UE, 0 otherwise.

  - Option 1a(Apple): The delay is

    - TFirstSSB + 39*TSSB

  - Option 2(Ericsson):

    - TL1-RSRP_SSB as (M*P)*TSSB

  - Option 3(Nokia, Samsung): Define generic requirement

  - Option 4: Further clarification is needed

Issue 1-2-6 TCI state list for PDCCH and PDSCH

- While re-using existing MAC-CE based TCI switching delay requirements for DL TCI switching delay requirements for PDCCH and PDSCH, the TCI state list considered should not be only for PDSCH.

Issue 1-2-7 Whether NM is allowed to be equal to 1 in Rel-17 specification

- RAN4 to agree that NM=1 is allowed in Rel-17 and shall define requirements for non-maintained case.

### 2.3 Sub-topic 1-3 Switching delay requirements when SSB is associated with cell with different PCI

Issue 1-3-2 TCI state switch delay requirement for known cell case

- For the case when the non-serving cell is known and the target TCI state is known, the same TCI state switch delay requirement as serving cell can be reused.

- For the case when the non-serving cell is known and the target TCI state is unknown, the same TCI state switch delay requirement as serving cell can be reused.

Issue 1-3-4 Whether introduce the interruption requirement due to TCI state switch associated with different PCI

- Not to consider this case. A UE assumes that the active BWPs of serving cell and non-serving cell is same for TCI switching.

Issue 1-3-5 Whether to only consider SSB based L1-RSRP measurement for cell with different PCI

- Only consider SSB based L1-RSRP measurement for RX beam refinement in Rel-17.

Issue 1-3-6 Update of known conditions for associated DL-RS

- The downlink TCI state is known if the following conditions are met:

-	During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target downlink TCI state to the completion of active downlink TCI state switch, where the RS resource for L1-RSRP measurement is the RS in target downlink TCI state or QCLed to the target downlink TCI state

-	Downlink TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement

-	The UE has sent at least 1 L1-RSRP report for the target downlink TCI state before the downlink TCI state switch command

-	The target downlink TCI state remains detectable during the downlink TCI state switching period

-	The SSB associated with the downlink TCI state remain detectable during the downlink TCI switching period

-	SNR of the downlink TCI state ≥ -3dB

-	The SSB can be associated with either the serving cell PCI or a PCI different from serving cell PCI.

### 2. 4 Sub-topic 1-4 Delay requirements for common TCI state switching in CA case

Issue 1-4-1 Known condition in CA scenario

- Proposals

  - Option 1(ZTE):

    - Reuse the existing known condition. Once the source RS of target TCI state is known for each CC in the intra-band CC group, which means the known condition is satisfied.

  - Option 2 (Intel, Nokia, vivo, Ericsson, Samsung, MTK, ZTE):

    - If the associated RS in common TCI state provides QCL-TypeD, the known condition can only consider whether the associated RS in the reference CC is known or not.

  - Option 3(Apple, ZTE):

    - The known condition should be dependent on shared RS or different RS.

### 2.5 Sub-topic 1-5 Requirements for PL-RS switching delay indicated by unified TCI

Issue 1-5-1 Whether to define MAC CE based PL-RS switching requirement when PL-RS is unknown

- Proposals

  - Option 1(Intel, Ericsson, ZTE):

    - Re-use MAC CE based UL TCI state switch delay of unknown case.

  - Option 2(Nokia):

    - Not to define PL-RS switching delay requirement when PL-RS is identical to the source RS in UL/Joint-TCI AND when the target PL-RS is unknown.

    - Apply MAC-CE based UL TCI switching delay requirement of known UL target TCI state,  when target PL-RS and source RS in UL/joint TCI are QCL-Type-D AND

- when the UL target TCI state is known but when the target pathloss reference signal is unknown  OR

- when the UL target TCI state is unknown but when the target pathloss reference signal is known.

  - Option 3: No additional requirements are defined or needed

Issue 1-5-3 DCI-based PL-RS switching delay requirements

- Apply DCI-based UL TCI switching delay requirement for DCI-based PL-RS switching delay requirements, when the target UL TCI state is known.

### 2.6 Sub-topic 1-6 TCI state list update delay

Issue 1-6-1 MAC CE based TCI state list update delay for serving cell

- Proposals

  - Option 1(vivo, Nokia, Ericsson):

    - For MAC CE based TCI state list update, specify requirements for the case when not all TCI states are known.

  - Option 2(Apple, MTK, Samsung):

    - Define MAC CE based TCI state list update requirement for known TCI state case

Issue 1-6-2 MAC CE based TCI state list update delay for cell with different PCI

- For DL TCI state list update requirements in FR2, T_first_SSB should be scale by the number of cells associated with the target UL TCIs whose SSBs for tracking are overlapped, i.e. Nmax+1, where Nmax=1.

- FFS: delay requirement when Nmax >1·

- For UL TCI state list update requirements in FR2, T_first_PL-RS and T_PL-RS should be scale by the number of cells associated with the target UL TCIs whose not-maintained PL-RSs are SSBs, and these SSBs are overlapped, i.e. Nmax+1, where Nmax=1.

- FFS: delay requirement when Nmax >1