3GPP TSG-RAN WG4 Meeting #102-e	R4-2207037

# Electronic Meeting,  21 February - 3 March 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | draft CR | rev | 1 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR: Introduction of channel quality report for NB-IoT supporting 16QAM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh4_LTE_eMTC6-Core |  |  |  |  |  | Date: |  |  | 2022-03-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduction of the channel quality report for NB-IoT (Cat-NB2) supporting 16QAM. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of the channel quality report for NB-IoT (Cat-NB2) supporting 16QAM. Correct the Code rate x 1024 for candidateRep-B. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear how Cat-NB2 UE detemines the channel quality. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.14.5 (new), 9.1.22.17 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS36.521-3 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of R4-2205090. Revise the reported value selection. Measure period is set in square brackets [].Correct the Code rate x 1024 for candidateRep-B |  |  |  |  |  |  |  |  |

----------------------------------------------------- Beginning of Change 1------------------------------------------------------------

### 8.14.5	Connected mode channel quality report for UE Category NB2 supporting 16QAM

The requirements in this clause shall apply for UE Category NB2 supporting 16QAM for unicast NPDSCH as defined in TS 36.331 [2] when triggered by the MAC-CE command as specified in TS 36.321 [17].

The DL channel quality provides the serving eNB with the following information with the parameters specified in Table 8.14.5-1:

-	The reported candidateRep should correspond to the minimum number of NPDCCH repetitions that achieves a hypothetical NPDCCH block error rate no larger than 1%, and correspond to the NPDSCH transport block size that achieves a NPDSCH block error rate no larger than 10%.

-	If the candidateRep entry corresponds to NPDCCH repetitions level X and NPDSCH transport block size Y, then no other candidate with NPDCCH repetitions level ≤ X and NPDSCH transport block size ≥ Y should satisfy the block error rate conditions for both NPDCCH and NPDSCH.

-	If there is no candidateRep entry to satisfy the condition, UE should report ‘noMeasurement’.

The report mapping is defined in Table 9.1.22.17-1.

Table 8.14.5-1: NPDCCH transmission parameters for downlink quality reporting

| Parameters | Values |
| --- | --- |
| DCI format | Format N1 |
| Number of information bits (excluding CRC) | 23bits |
| System bandwidth | 200kHz |
| Aggregation level | 2 |
| DRX | OFF |

The reported candidateRep value shall be derived from the channel quality measured from the time [UE finishes the decoding of Downlink Channel Quality report MAC CE] to [the end of NPDCCH period which carries the uplink grant of channel quality report for measurement of DL channel quality of the configured carrier].

------------------------------------------------------------- End of change 1------------------------------------------------------------

----------------------------------------------------- Beginning of Change 2------------------------------------------------------------

#### 9.1.22.17	Channel quality reporting for UE Category NB2 with 16-QAM

Table 9.1.22.17-1: Downlink channel quality measurement report mapping when the reception of unicast NPDSCH modulated with 16-QAM is configured

| Reported value | NPDCCH repetition level | NPDSCH transport block error probability not exceeding 0.1 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Modulation | Code rate x 1024 | Repetition | Efficiency |
| noMeasurement | No measurement reporting | Out of range |  |  |  |
| candidateRep-A | 1 | QPSK | 221 | 1 | 0.4316 |
| candidateRep-B | 2 | QPSK | 140 | 1 | 0.2737 |
| candidateRep-C | 4 | QPSK | 81 | 1 | 0.1579 |
| candidateRep-D | 8 | QPSK | 81 | 2 | 0.0789 |
| candidateRep-E | 16 | QPSK | 81 | 4 | 0.0395 |
| candidateRep-F | 32 | QPSK | 81 | 8 | 0.0198 |
| candidateRep-G | 1 | QPSK | 336.8 | 1 | 0.6579 |
| candidateRep-H | 1 | QPSK | 453.6 | 1 | 0.8860 |
| candidateRep-I | 1 | QPSK | 579.4 | 1 | 1.1316 |
| candidateRep-J | 1 | QPSK | 759 | 1 | 1.4825 |
| candidateRep-K | 1 | 16QAM | 487.3 | 1 | 1.9035 |
| candidateRep-L | 1 | 16QAM | 541.2 | 1 | 2.1140 |
| candidateRep-M | 1 | 16QAM | 658 | 1 | 2.5702 |
| candidateRep-N | 1 | 16QAM | 783.7 | 1 | 3.0614 |
| candidateRep-O | 1 | 16QAM | 837.6 | 1 | 3.2719 |

----------------------------------------------------- Beginning of Change 2------------------------------------------------------------