3GPP TSG-RAN WG4 Meeting #102-e	R4-2207198

# Electronic Meeting, 21st Feb – 3rd Mar, 2022

Title:	WF on NTN SAN demodulation requirements

Source:	Huawei, HiSilicon

Agenda item:			10.13.6.1

Document for:	Approval

Introduction

This WF capture all agreements and open issues for the following topics in [102-e][325] NR_NTN_Demod.

- Topic #2: Satellite Access Node demodulation requirements

  - Issue 2-1: General assumptions

  - Issue 2-2: PUSCH requirements

  - Issue 2-3: PUCCH requirements

  - Issue 2-4: PRACH requirements

The agreed WFs on NTN SAN demodulation requirements in previous meetings are listed as following.

- R4-2203043, RAN4#101bis-e

Topic #2: Satellite Access Node demodulation requirements

Issue 2-1: General assumptions

Issue 2-1-1: Doppler shift model

Agreements

- Consider 200Hz as the maximum Doppler shift for UL in service link

Candidate options

- Option 1: Do not consider the residual Doppler error for UL in feeder link

- Option 2: Consider the residual Doppler error for UL in feeder link. 0.5ppm is the worst case.

Issue 2-1-2: Delay spread model

Candidate options

- Option 1: Single delay spread

  - Option 1a: 100ns

  - Option 1b: 250ns

- Option 2: Different delay spread

  - Option 2a: 10ns/50ns/150ns

  - Option 2b: 10ns/50ns/250ns.

Issue 2-2: PUSCH requirements

Issue 2-2-1: Scope of PUSCH requirements

Agreements

- Not to consider the requirements for mapping Type B with non-slot transmission

Candidate options

- Option 1: Do not consider 2 step RACH case

- Option 2: Consider the 2 step RACH case

Issue 2-2-2: Channel model for PUSCH

Agreements

- Select NTN-TDL-A and NTN-TDL-C as the channel model for PUSCH requirements

Issue 2-2-3: SCS/CBW set for PUSCH requirements

Candidate options

- Option 1: 15kHz SCS: 5MHz/10MHz/20MHz, 30kHz SCS: 10MHz/20MHz

- Option 2: a few of PRBs for all SCS

Issue 2-2-4: Modulation order for PUSCH requirements

Agreements

- Select MCS4 for PUSCH requirements

Issue 2-2-5: Antenna configuration for PUSCH requirements

Agreements

- UE 1Tx

- SAN 2Rx

Candidate options

- FFS: SAN 1Rx, SAN 4Rx and SAN 8Rx

Issue 2-2-6: Test parameters for NTN PUSCH

For information, not agreement

- Interested companies are encouraged to provide initial simulation results in next meeting.

| Parameter |  | Value |
| --- | --- | --- |
| Channel model |  | NTN-TDL-A and NTN-TDL-C |
| MCS |  | MCS4 |
| Transform precoding |  | Disabled and Enabled |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0, group hopping and sequence hopping are disabled |
| Time domain | PUSCH mapping type | A, B |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
| Frequency domain resource | RB assignment | FFS |
| assignment | Frequency hopping | Disabled |
| TPMI index for 2Tx two-layer spatial multiplexing transmission |  | 0 |
| Code block group based PUSCH transmission |  | Disabled |

Issue 2-2-7: Test parameters for NTN UL timing adjustment

For information, not agreement

- Interested companies are encouraged to provide initial simulation results in next meeting.

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Channel bandwidth |  | FFS |
| MCS |  | MCS4 |
| Channel model |  | NTN-TDL-A |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | DM-RS position (l0) | 2 |
|  | Additional DM-RS position | pos2 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
| Time domain resource assignment | DM-RS sequence generation | NID0=0, nSCID =0 for moving UE NID0=1, nSCID =1 for stationary UE |
|  | PUSCH mapping type | Both A and B |
|  | Allocation length | 14 |
| Frequency domain resource assignment | RB assignment | FFS |
|  | Starting PRB index | FFS |
|  | Frequency hopping | Disabled |
| SRS resource allocation | Slots in which sounding RS is transmitted (Note 1) | slot #1 in radio frames |
|  | SRS resource allocation | FFS |
| NOTE 1. The transmission of SRS is optional. And the transmission comb and SRS periodic are configured as KTC = 2, and TSRS = 10 respectively. |  |  |

| Parameter | Scenario X |
| --- | --- |
| Channel model | Stationary UE: AWGN Moving UE: NTN-TDLA<Doppler>-<DS> |
| UE speed | 120 km/h |
| CP length | Normal |
| A | 15 kHz: 10 s 30 kHz: 5 s |
|  | 15 kHz: 0.04 s-1 30 kHz: 0.08 s-1 |

Issue 2-2-8: Test parameters for NTN PUSCH repetition type A

For information, not agreement

- Interested companies are encouraged to provide initial simulation results in next meeting.

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Channel model |  | NTN-TDL-A |
| MCS |  | 5 in Table 3 |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 3, 0, 3 [Note 2] |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | 0 |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type | A, B |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
|  | PUSCH aggregation factor | n2 |
| Frequency domain resource assignment | RB assignment | FFS |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| Note 2: The effective RV sequence is {0, 2, 3, 1} with slot aggregation. |  |  |

Issue 2-2-9: Test parameters for NTN msgA PUSCH for 2-step RA type

For information, not agreement

- Interested companies are encouraged to provide initial simulation results in next meeting.

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Channel bandwidth |  | FFS |
| Channel model |  | NTN-TDL-A |
| MCS |  | FFS |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | DM-RS position (l0) | 2 |
|  | Additional DM-RS position | pos2 or pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID = 0 |
| Time domain resource assignment | PUSCH mapping type | Both A and B |
|  | Allocation length | 14 |
| Frequency domain resource assignment | RB assignment | 2 PRBs |
|  | Starting PRB index | 0 |
|  | Frequency hopping | Disabled |
| Time offset (TO) Cycling (µs) | start:end | 15k SCS: FFS |
|  |  | 30k SCS: FFS |
| Test Metric | BLER | 0.01 |
| NOTE 1: A single requirement is defined that is applicable regardless of whether pos1 or pos2 is configured for the additional DM-RS position NOTE 2: The power ratio between preamble and msgA (msgA-DeltaPreamble) is set to be sufficient to achieve 100% preamble detection. The SNR for the requirement is defined on the msgA PUSCH |  |  |

Issue 2-3: PUCCH requirements

Issue 2-3-1: Scope of PUCCH requirements

Agreements

- In addition to PUCCH format 0/1/2/3/4, RAN4 to define NTN multi-slot PUCCH demodulation requirements

- Prioritize UCI with HARQ on PUCCH demodulation requirement

Issue 2-3-2: Channel model for PUCCH requirements

Agreements

- RAN4 to only consider NTN-TDLA channel model for PUCCH requirements definition

Issue 2-3-3: SCS/CBW set for PUCCH requirements

Agreements

- To follow the same SCS/CBW set as PUSCH as the start point

Candidate options

- Option 1: No need to reduce test

- Option 2: Need to reduce test cases (specify if any)

Issue 2-3-4: Antenna configuration for PUCCH

Agreements

- UE 1Tx

- SAN 2Rx

Candidate options

- FFS: SAN 1Rx, SAN 4Rx and SAN 8Rx

Issue 2-3-5: Test parameters for NTN PUCCH format 0/1/2/3/4

For information, not agreement

- Interested companies are encouraged to provide initial simulation results in next meeting.

| Parameter | Test |
| --- | --- |
| PUCCH format | 0 |
| Number of UCI information bits | 1 |
| Number of PRBs | 1 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | N/A for 1 symbol Enabled for 2 symbols |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 13 for 1 symbol 12 for 2 symbols |
| Channel model | NTN-TDLA |
| Number of OFDM symbols | FFS |
| Test metric | ACK missed detection probability <1% |

| Parameter | Test |
| --- | --- |
| PUCCH format | 1 |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (nrofPRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Cyclic Prefix | Normal |
| Channel model | NTN-TDLA |
| Test metric | NACK to ACK probability<0.1%, ACK missed detection probability <1% |

| Parameter | Value |
| --- | --- |
| PUCCH format | 2 |
| Modulation order | QSPK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | N/A |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Number of PRBs | 4 |
| Number of symbols | 1 |
| The number of UCI information bits | 4 |
| First symbol | 13 |
| DM-RS sequence generation | NID0=0 |
| Cyclic Prefix | Normal |
| Channel model | NTN-TDLA |
| Test metric | NACK to ACK probability<0.1%, ACK missed detection probability <1% |

| Parameter | Value |
| --- | --- |
| e | 2 |
| Modulation order | QSPK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| Frist PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Number of PRBs | 9 |
| Number of symbols | 2 |
| The number of UCI information bits | 22 |
| First symbol | 12 |
| DM-RS sequence generation | NID0=0 |
| Cyclic Prefix | Normal |
| Channel model | NTN-TDLA |
| Test metric | UCI block error probability<1% |

| Parameter | Value |
| --- | --- |
| PUCCH format | 3 |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of PRBs | FFS |
| Number of symbols | FFS |
| The number of UCI information bits | FFS |
| First symbol | 0 |
| Cyclic Prefix | Normal |
| Additional DM-RS configuration | FFS |
| Channel model | NTN-TDLA |
| Test metric | FFS |

| Parameter | Value |
| --- | --- |
| PUCCH format | 4 |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Number of PRBs | 1 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of symbols | 14 |
| The number of UCI information bits | FFS |
| First symbol | 0 |
| Length of the orthogonal cover code | n2 |
| Index of the orthogonal cover code | n0 |
| Cyclic Prefix | Normal |
| Additional DM-RS configuration | FFS |
| Channel model | NTN-TDLA |
| Test metric | FFS |

Issue 2-3-6:  Test parameters for NTN PUCCH multi-slot PUCCH format 1

For information, not agreement

- Interested companies are encouraged to provide initial simulation results in next meeting.

| Parameter | Test |
| --- | --- |
| PUCCH format | 1 (multi-slot) |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | disabled |
| Inter-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (nrofPRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Number of slots for PUCCH repetition | 2 |
| Cyclic Prefix | Normal |
| Channel model | NTN-TDLA |
| Test metric | NACK to ACK probability<0.1%, ACK missed detection probability <1% |

Issue 2-4: PRACH requirements

Issue 2-4-1: Channel model for PRACH

Agreements

- Define NTN SAN PRACH demodulation requirement for AWGN and NLOS multi-path channel.

Issue 2-4-2: Test parameters for NTN PRACH demodulation requirement

For information, not agreement

- Interested companies are encouraged to provide initial simulation results in next meeting.

| Parameter | Value |
| --- | --- |
| Preamble format | FFS |
| Antenna | FFS |
| SCS | FFS |
| Propagation | AWGN, NTN-TDLA |
| Frequency offset | FFS |
| Time error tolerance | FFS |

Reference

- R4-2207169, Email discussion summary for [102-e][325] NR_NTN_Demod, RAN4#102-e, Qualcomm Incorporated

- R4-2203043, WF on NTN SAN demodulation requirements, RAN4#101bis-e, Huawei, HiSilicon