3GPP TSG-RAN WG4 Meeting #102-e	R4-2207206

Electronic Meeting, 21 February – 3 March 2022

Agenda Item:	10.20.4

Source:	Ericsson

Title:	WF on RedCap demodulation and CQI reporting requirements

Document for:	Approval

# 1	Configuration for UE demodulation and CSI reporting requirements for RedCap

Number of Rx antennas, CBW, duplex mode, and frequency range

- 1Rx RedCap UE: RAN4 define new UE demodulation and CSI reporting requirements with the following configuration:

  - FDD SCS=15kHz in FR1: CBW=10MHz

  - TDD SCS=30kHz in FR1: CBW=20MHz

  - Not to define UE demodulation and CSI requirements with 1Rx for FR2 RedCap UE

UL/DL pattern used for FDD tests for 1Rx UE

- Assume Full-duplex FDD to derive the requirements, e.g., SNR to achieve 70% of maximum throughput.

- For the requirement specification,

  - Option 1: Consider Full-duplex FDD only

  - Option 2: Consider Full-duplex FDD and half-duplex FDD with applicability rule

Applicability rule for RedCap UE demodulation and CSI reporting requirements between 1Rx UE and 2Rx UE

- Define applicability rule for requirements for RedCap UE:

  - If RedCap UE supports only 1 Rx, only 1Rx test is executed

  - If RedCap UE supports only 2 Rx, only 2Rx test is executed

  - If RedCap UE supports both 1 Rx and 2 Rx, only 2 Rx test is executed

# 2	UE demodulation requirements

Interested companies are encouraged to provide the simulation results for the agreed test cases in RAN4#103-e.

## 2.1	PDSCH

### 2.1.1	Scope of demodulation requirements

Additional PDSCH demodulation requirements

- Option 1: Focus on definition of minimum set of requirements, discussed in Topic #2, to verify the mandatory features. RAN4 discuss other requirements once it is stable, and the performance part TU is allowed.

- Option 2: Not define the additional PDSCH demodulation requirements other than the candidates discussed in 2.1, in Rel-17 RedCap

Define 256QAM demodulation requirements or not

- Option 1: Specify 256QAM demodulation requirements for FR1 only

  - Candidate test setup:

    - TS38.101-4 5.2.2.1.1-3 Test 1-3 for FDD 15kHz

    - TS38.101-4 5.2.2.2.1-3 Test 1-3 for TDD 30kHz

  - FFS the MCS for 1Rx case

- Option 2: Not to specify 256QAM demodulation requirements.

- Option 3: Discuss whether to specify the requirements for FR1 once we stabilize the discussion on requirements for mandatory features.

### 2.1.2	Simulation assumption

Interested companies are encouraged to provide the simulation results with ‘New’. Interested companies can also provide the simulation results with ‘Reuse’ for reference.

#### 3.1.1.1	FDD 15kHz FR1

| 1Rx UE or 2Rx UE | CBW / SCS | MCS and rank | Propagation condition | Antenna configuration | Metric | Reference from TS38.101-4 Table | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 10MHz / 15kHz | QPSK 1/3 Rank 1 | TDLB400-400 | 2x1 low | 70% of peak rate | 5.2.2.1.1-3 Test 1-1 | New |
| 1Rx UE | 10MHz / 15kHz | 16QAM 0.48 Rank 1 | TDLC300-100 | 2x1 low | 70% of peak rate | 5.2.2.1.1-3 Test 1-4 | New |
| 1Rx UE | 10MHz / 15kHz | 64QAM 0.50 Rank 1 | TDLA30-10 | 2x1 low | 70% of peak rate | 5.2.2.1.1-4 Test 2-1 | New |
| 2Rx UE | 10MHz / 15kHz | QPSK 1/3 Rank 1 | TDLB400-400 | 2x2 ULA low | 70% of peak rate | 5.2.2.1.1-3 Test 1-1 | Reuse |
| 2Rx UE | 10MHz / 15kHz | 16QAM 0.48 Rank 1 | TDLC300-100 | 2x2 ULA low | 70% of peak rate | 5.2.2.1.1-3 Test 1-4 | New |
| 2Rx UE | 10MHz / 15kHz | 64QAM 0.5 Rank 2 | TDLA30-10 | 2x2 ULA low | 70% of peak rate | 5.2.2.1.1-4 Test 2-1 | Reuse |

#### 3.1.1.2	TDD 30kHz FR1

| 1Rx UE or 2Rx UE | CBW / SCS | MCS and rank | TDD UL/DL pattern | Propagation condition | Antenna configuration | Metric | Reference from TS38.101-4 Table | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 20MHz / 30kHz | QPSK 1/3 Rank 1 | FR1.30-1A | TDLB400-400 | 2x1 low | 70% of peak rate | 5.2.2.2.1-3 Test 1-1 | New |
| 1Rx UE | 20MHz / 30kHz | 16QAM 0.48 Rank 1 | FR1.30-1 | TDLC300-100 | 2x1 low | 70% of peak rate | 5.2.2.2.1-3 Test 1-4 | New |
| 1Rx UE | 20MHz / 30kHz | 64QAM 0.50 Rank 1 | FR1.30-1 | TDLA30-10 | 2x1 low | 70% of peak rate | 5.2.2.2.1-4 Test 2-1 | New |
| 2Rx UE | 20MHz / 30kHz | QPSK 1/3 Rank 1 | FR1.30-1A | TDLB400-400 | 2x2 ULA low | 70% of peak rate | 5.2.2.2.1-3 Test 1-1 | New |
| 2Rx UE | 20MHz / 30kHz | 16QAM 0.48 Rank 1 | FR1.30-1 | TDLC300-100 | 2x2 ULA low | 70% of peak rate | 5.2.2.2.1-3 Test 1-4 | New |
| 2Rx UE | 20MHz / 30kHz | 64QAM 0.50 Rank 2 | FR1.30-1 | TDLA30-10 | 2x2 ULA low | 70% of peak rate | 5.2.2.2.1-4 Test 2-1 | New |

#### 3.1.1.3	TDD 120kHz FR2

| 1Rx UE or 2Rx UE | CBW / SCS | MCS and rank | TDD UL/DL pattern | Propagation condition | Antenna configuration | Metric | Reference from TS38.101-4 | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2Rx UE | 100MHz / 120kHz | QPSK 1/3 Rank 1 | FR2.120-1A | TDLC60-300 | 2x2 ULA low | 70% of peak rate | 7.2.2.2.1-3 Test 1-1 | Reuse |
| 2Rx UE | 100MHz / 120kHz | 16QAM 0.48 Rank 2 | FR2.120-1 | TDLA30-300 | 2x2 ULA low | 70% of peak rate | 7.2.2.2.1-4 Test 2-2 | Reuse |
| 2Rx UE | 100MHz / 120kHz | 64QAM 0.46 Rank 2 | FR2.120-2 | TDLA30-75 | 2x2 ULA low | 70% of peak rate | 7.2.2.2.1-4 Test 2-6 | Reuse |

## 2.2	PDCCH

### 2.2.1	Scope of demodulation requirements

Aggregation level(s)

- Option 1: Specify PDCCH demodulation requirements with AL4 and AL8.

- Option 2: Specify PDCCH demodulation requirements with AL4 or AL8.

### 2.2.2	Simulation assumption

Interested companies are encouraged to provide the simulation results with ‘New’. Interested companies can also provide the simulation results with ‘Reuse’ for reference.

#### 2.2.2.1	Candidate options for FDD 15kHz FR1

| 1Rx UE or 2Rx UE | CBW (MHz) | RB size | duration | AL | Propagation condition | Antenna configuration | Metric (Pm-dsg) | Reference from TS38.101-4 | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 10 | 48 | 2 | 4 | TDLA30-10 | 1x1 | 1% | 5.3.2.1.1-1 Test 3 | New |
| 1Rx UE | 10 | 48 | 1 | 8 | TDLA30-10 | 2x1 low | 1% | 5.3.2.1.2-1 Test 3 | New |
| 2Rx UE | 10 | 48 | 2 | 4 | TDLA30-10 | 1x2 low | 1% | 5.3.2.1.1-1 Test 3 | Reuse |
| 2Rx UE | 10 | 48 | 1 | 8 | TDLA30-10 | 2x2 low | 1% | 5.3.2.1.2-1 Test 3 | Reuse |

#### 2.2.2.2	Candidate options for TDD 30kHz FR1

| 1Rx UE or 2Rx UE | CBW (MHz) | RB size | duration | AL | Propagation condition | Antenna configuration | Metric (Pm-dsg) | Reference from TS38.101-4 | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 20 | 48 | 2 | 4 | TDLC300-100 | 1x1 | 1% | 5.3.2.2.1-1 Test 2 | New |
| 1Rx UE | 20 | 48 | 2 | 8 | TDLC300-100 | 2x1 low | 1% | 5.3.2.2.2-1 Test 1 | New |
| 2Rx UE | 20 | 48 | 2 | 4 | TDLC300-100 | 1x2 low | 1% | 5.3.2.2.1-1 Test 2 | New |
| 2Rx UE | 20 | 48 | 2 | 8 | TDLC300-100 | 2x2 low | 1% | 5.3.2.2.2-1 Test 1 | New |

#### 2.2.2.3		Candidate options for TDD 120kHz FR2

| 1Rx UE or 2Rx UE | CBW (MHz) | RB size | duration | AL | Propagation condition | Antenna configuration | Metric (Pm-dsg) | Reference from TS38.101-4 | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2Rx UE | 100 | 60 | 1 | 4 | TDLA30-300 | 1x2 low | 1% | 7.3.2.2.1-1 Test 1-2 | Reuse |
| 2Rx UE | 100 | 60 | 1 | 8 | TDLA30-75 | 2x2 low | 1% | 7.3.2.2.2-1 Test 2-1 | Reuse |

## 2.4	PBCH

### 2.4.1	Scope of demodulation requirements

Whether to define 1Rx PBCH demodulation requirements in the case SS/PBCH block index is known

- Not to define 1Rx PBCH demodulation requirements in the case SS/PBCH block index is known

  - Define only 2Rx requirements for FR2

### 2.4.2	Simulation assumption

Interested companies are encouraged to provide the simulation results with ‘New’. Interested companies can also provide the simulation results with ‘Reuse’ for reference.

| 1Rx UE or 2Rx UE | BW / SSB SCS | Propagation condition | Antenna configuration | Metric (Pm-bch) | SS/PBCK block index | Reference from TS38.101-4 | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 10MHz / 15kHz | TDLC300-100 | 1x1 | 1% | Unknown | 5.4.2.1-2 Test 1 | New |
| 1Rx UE | 20MHz / 30kHz | TDLA30-10 | 1x1 | 1% | Unknown | 5.4.2.2-2 Test 1 | New |
| 2Rx UE | 10MHz / 15kHz | TDLC300-100 | 1x2 low | 1% | Unknown | 5.4.2.1-2 Test 1 | Reuse |
| 2Rx UE | 10MHz / 15kHz | TDLC300-100 | 1x2 low | 1% | Known | 5.4.2.1-3 Test 1 | Reuse |
| 2Rx UE | 20MHz / 30kHz | TDLA30-10 | 1x2 low | 1% | Unknown | 5.4.2.2-2 Test 1 | Reuse (*1) |
| 2Rx UE | 20MHz / 30kHz | TDLA30-10 | 1x2 low | 1% | Known | 5.4.2.2-3 Test 1 | Reuse (*1) |
| 2Rx UE | 100MHz / 120kHz | TDLA30-300 | 1x2 low | 1% | Unknown | 7.4.2.2-2 Test 1 | Reuse |
| 2Rx UE | 100MHz / 120kHz | TDLA30-300 | 1x2 low | 1% | Known | 7.4.2.2-3 Test 1 | Reuse |
| 2Rx UE | 100MHz / 240kHz | TDLA30-75 | 1x2 low | 1% | Unknown | 7.4.2.2-2 Test 2 | Reuse |
| 2Rx UE | 100MHz / 240kHz | TDLA30-75 | 1x2 low | 1% | Known | 7.4.2.2-3 Test 2 | Reuse |
| Note 1: RAN4 can keep the same SNR requirements but need to change the channel bandwidth configuration. |  |  |  |  |  |  |  |

## 2.4	Sustained data rate (SDR)

- Define SDR tests for RedCap 1Rx/2Rx UE

  - Define only 2Rx requirements for FR2.

# 4	CSI reporting requirements

## 4.1		CQI reporting requirements

### 4.1.1	Scope of CQI reporting requirements

Whether to define CQI reporting test with CQI table 2 for RedCap 2Rx UE supporting 256QAM in FR1 FDD

- Define test with only CQI table 1.

Test points for CQI reporting test with CQI table 1

- Set two SNR test points based on simulation results to cover SNR region with CQI report corresponding to:

  - 64QAM

  - QPSK or 16QAM

### 4.1.2	Simulation assumption

Interested companies are encouraged to provide the simulation results with ‘New’. Interested companies can also provide the simulation results with ‘Reuse’ for reference.

#### 4.1.2.1	FDD FR1

| 1Rx UE or 2Rx UE | Reference | CQI test | CBW / SCS | CQI table | Rank | Antenna configuration | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | Table 6.2.2.1.1.1-1 Test 1/2 | CQI report definition test | 10 / 15 | Table 1 | 1 | 2 x 1 with static channel specified in Annex B.1 in TS 38.101-4 | New |
| 1Rx UE | Table 6.2.2.1.2.1-1 Test 1/2 | Wideband CQI reporting test | 10 / 15 | Table 1 | 1 | 2 x 1 ULA high | New |
| 2Rx UE | Table 6.2.2.1.1.1-1 Test 1/2 | CQI report definition test | 10 / 15 | Table 1 | 2 | 2 x 2 with static channel specified in Annex B.1 in TS 38.101-4 | New |
| 2Rx UE | Table 6.2.2.1.2.1-1 Test 1/2 | Wideband CQI reporting test | 10 / 15 | Table 1 | 2 | 2 x 2 ULA high | New |

#### 4.1.2.2	TDD FR1

| 1Rx UE or 2Rx UE | Reference | CQI test | CBW / SCS | CQI table | Rank | Antenna configuration | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | Table 6.2.2.2.1.1-1 Test 1/2 | CQI report definition test | 20 / 30 | Table 1 | 1 | 2 x 1 with static channel specified in Annex B.1 in TS 38.101-4 | New |
| 1Rx UE | Table 6.2.2.2.2.1-1 Test 1/2 | Wideband CQI reporting test | 20 / 30 | Table 1 | 1 | 2 x 1 ULA high | New |
| 2Rx UE | Table 6.2.2.2.1.1-1 Test 1/2 | CQI report definition test | 20 / 30 | Table 1 | 2 | 2 x 2 with static channel specified in Annex B.1 in TS 38.101-4 | New |
| 2Rx UE | Table 6.2.2.2.2.1-1 Test 1/2 | Wideband CQI reporting test | 20 / 30 | Table 1 | 1 | 2 x 2 ULA high | New |

#### 4.1.2.3	TDD FR2

| 1Rx UE or 2Rx UE | Reference | CQI test | CBW / SCS | CQI table | Rank | Antenna configuration | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2Rx UE | Table 8.2.2.2.1.1-1 Test 1/2 | CQI report definition test | 100 / 120 | Table 1 | 2 | 2 x 2 ULA high | Reuse |
| 2Rx UE | Table 8.2.2.2.2.1-1 Tests 1/2 | Wideband CQI reporting test | 100 / 120 | Table 1 | 1 | 2 x 2 ULA high | Reuse |

## 4.2	PMI reporting requirements

### 4.2.1	Scope of PMI reporting requirements

Whether to define PMI reporting requirements for RedCap UEs

- Define PMI reporting requirements with 4Tx configuration for FR1 for both 1Rx and 2Rx UEs

- Define PMI reporting requirements with 2Tx configuration for FR2 for 2Rx UE.

### 4.2.2	Simulation assumption

Interested companies are encouraged to provide the simulation results with ‘New’. Interested companies can also provide the simulation results with ‘Reuse’ for reference.

#### 4.2.2.1	FDD FR1

| 1Rx UE or 2Rx UE | Reference | PMI test | CBW / SCS | Tx ports | Antenna Configuration | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- |
| 1Rx | Table 6.3.2.1.1-1 Test 1 | Single PMI, Type I, rank 1 | 10 / 15 | 4 | 4 x 1 ULA high | New |
| 2Rx | Table 6.3.2.1.1-1 Test 1 | Single PMI, Type I, rank 1 | 10 / 15 | 4 | 4 x 2 XP high | Reuse |

#### 4.2.2.2	TDD FR1

| 1Rx UE or 2Rx UE | Reference | PMI test | CBW / SCS | Tx ports | Antenna Configuration | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- |
| 1Rx | Table 6.3.2.2.1-1 Test 1 | Single PMI, Type I, rank 1 | 20 / 30 | 4 | 4 x 1 ULA high | New |
| 2Rx | Table 6.3.2.2.1-1 Test 1 | Single PMI, Type I, rank 1 | 20 / 30 | 4 | 4 x 2 XP high | New |

#### 4.2.2.2	TDD FR2

| 1Rx UE or 2Rx UE | Reference | PMI test | CBW / SCS | Tx ports | Antenna Configuration | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- |
| 2Rx | Table 8.3.2.2.1-1 Test 2 (TDD FR2.120-1) | Single PMI, Type I, rank 1 | 100 / 120 | 2 | 2 x 2 ULA low | Reuse |

## 4.3	RI reporting requirements for 2Rx UE

### 4.3.1	Scope of RI reporting requirements

Whether to define CQI reporting test with CQI table 2 for RedCap 2Rx UE supporting 256QAM in FR1 FDD

- Option 1: Define RI reporting tests for RedCap 2Rx UE.

- Option 2: Not to define RI reporting tests for RedCap 2Rx UE.

# Appendix: Spec structure for RedCap UE demodulation and CSI reporting requirements (For information)

Table 1	Specification structure for RedCap UE Demodulation performance requirements (FR1 - Conducted requirements).

| Section number | Section name | Note |
| --- | --- | --- |
| 5.1 | General |  |
| 5.1.1 | Applicability of requirements |  |
| 5.1.1.x | Applicability of requirements for RedCap UEs | New section |
| 5.2 | PDSCH demodulation requirements |  |
| 5.2.1 | 1RX requirements |  |
| 5.2.1.1 | FDD |  |
| 5.2.1.1.1 | Minimum requirements for RedCap UEs | New section |
| 5.2.1.2 | TDD |  |
| 5.2.1.2.1 | Minimum requirements for RedCap UEs | New section |
| 5.2.2 | 2RX requirements |  |
| 5.2.2.1 | FDD |  |
| 5.2.2.1.X | Minimum requirements for RedCap UEs | New section |
| 5.2.2.2 | TDD |  |
| 5.2.2.2.X | Minimum requirements for RedCap UEs | New section |
| 5.3 | PDCCH demodulation requirements |  |
| 5.3.1 | 1RX requirements |  |
| 5.3.1.1 | FDD |  |
| 5.3.1.1.1 | Minimum requirements for RedCap UEs | New section |
| 5.3.1.2 | TDD |  |
| 5.3.1.2.1 | Minimum requirements for RedCap UEs | New section |
| 5.3.2 | 2RX requirements |  |
| 5.3.2.1 | FDD |  |
| 5.3.2.1.X | Minimum requirements for RedCap UEs | New section |
| 5.3.2.2 | TDD |  |
| 5.3.2.2.X | Minimum requirements for RedCap UEs | New section |
| 5.4 | PBCH demodulation requirements |  |
| 5.4.1 | 1RX requirements |  |
| 5.4.1.1 | FDD | New section |
| 5.4.1.2 | TDD | New section |
| 5.4.2 | 2RX requirements |  |
| 5.4.2.1 | FDD | Add minimum requirement tables for RedCap UEs |
| 5.4.2.2 | TDD | Add minimum requirement tables for RedCap UEs |
| 5.5 | Sustained downlink data rate provided by lower layers |  |
| 5.5.1 | FR1 single carrier requirements | Add the applicability for RedCap UEs |
| 5.5A | Sustained downlink data rate provided by lower layers |  |
| 5.5A.1 | FR1 CA requirements | Update test parameters for RedCap UEs. |

Table 2	Specification structure for RedCap UE CSI reporting requirements (FR1 - Conducted requirements).

| Section number | Section name | Note |
| --- | --- | --- |
| 6.1 | General |  |
| 6.1.1 | Applicability of requirements |  |
| 6.1.1.x | Applicability of requirements for RedCap UEs | New section |
| 6.2 | Reporting of Channel Quality Indicator (CQI) |  |
| 6.2.1 | 1RX requirements |  |
| 6.2.1.1 | FDD |  |
| 6.2.1.1.1 | CQI reporting definition under AWGN conditions for RedCap UEs | New section |
| 6.2.1.1.2 | CQI reporting under fading conditions for RedCap UEs | New section |
| 6.2.1.2 | TDD |  |
| 6.2.1.2.1 | CQI reporting definition under AWGN conditions for RedCap UEs | New section |
| 6.2.1.2.2 | CQI reporting under fading conditions for RedCap UEs | New section |
| 6.2.2 | 2RX requirements |  |
| 6.2.2.1 | FDD |  |
| 6.2.2.1.X | CQI reporting definition under AWGN conditions for RedCap UEs | New section |
| 6.2.2.1.X | CQI reporting under fading conditions for RedCap UEs | New section |
| 6.2.2.2 | TDD |  |
| 6.2.2.2.X | CQI reporting definition under AWGN conditions for RedCap UEs | New section |
| 6.2.2.2.X | CQI reporting under fading conditions for RedCap UEs | New section |
| 6.3 | Reporting of Precoding Matrix Indicator (PMI) |  |
| 6.3.1 | 1RX requirements |  |
| 6.3.1.1 | FDD |  |
| 6.3.1.1.1 | Single PMI with 4 TX TypeI-SinglePanel Codebook for RedCap UEs | New section |
| 6.3.1.2 | TDD |  |
| 6.3.1.2.1 | Single PMI with 4 TX TypeI-SinglePanel Codebook for RedCap UEs | New section |
| 6.3.2 | 2RX requirements |  |
| 6.3.2.1 | FDD |  |
| 6.3.2.1.X | Single PMI with 4 TX TypeI-SinglePanel Codebook for RedCap UEs | New section |
| 6.3.2.2 | TDD |  |
| 6.3.2.2.X | Single PMI with 4 TX TypeI-SinglePanel Codebook for RedCap UEs | New section |

Table 3	Specification structure for RedCap UE Demodulation performance requirements (FR2 - Radiated requirements).

| Section number | Section name | Note |
| --- | --- | --- |
| 7.1 | General |  |
| 7.1.1 | Applicability of requirements |  |
| 7.1.1.x | Applicability of requirements for RedCap UEs | New section |
| 7.2 | PDSCH demodulation requirements |  |
| 7.2.2 | 2RX requirements |  |
| 7.2.2.2 | TDD |  |
| 7.2.2.2.X | Minimum requirements for RedCap UEs | New section |
| 7.3 | PDCCH demodulation requirements |  |
| 7.3.2 | 2RX requirements |  |
| 7.3.2.2 | TDD |  |
| 7.3.2.2.X | Minimum requirements for RedCap UEs | New section |
| 7.4 | PBCH demodulation requirements |  |
| 7.4.2 | 2RX requirements |  |
| 7.4.2.2 | TDD | Add minimum requirement tables for RedCap UEs |
| 7.5 | Sustained downlink data rate provided by lower layers |  |
| 7.5.1 | FR2 single carrier requirements | Add the applicability for RedCap UEs |
| 7.5A | Sustained downlink data rate provided by lower layers |  |
| 7.5A.1 | FR2 CA requirements | Update test parameters for RedCap UEs. |

Table 4	Specification structure for RedCap UE CSI reporting requirements (FR2 - Radiated requirements).

| Section number | Section name | Note |
| --- | --- | --- |
| 8.1 | General |  |
| 8.1.1 | Applicability of requirements |  |
| 8.1.1.x | Applicability of requirements for RedCap UEs | New section |
| 8.2 | Reporting of Channel Quality Indicator (CQI) |  |
| 8.2.2 | 2RX requirements |  |
| 8.2.2.2 | TDD |  |
| 8.2.2.2.X | CQI reporting definition under AWGN conditions for RedCap UEs | New section |
| 8.2.2.2.X | CQI reporting under fading conditions for RedCap UEs | New section |
| 8.3 | Reporting of Precoding Matrix Indicator (PMI) |  |
| 8.3.2 | 2RX requirements |  |
| 8.3.2.2 | TDD |  |
| 8.3.2.2.X | Single PMI with 2 TX TypeI-SinglePanel Codebook for RedCap UEs | New section |