3GPP TSG-RAN WG4 Meeting #102-e															R4-2207231

Electronic Meeting, February 21st - March 3rd, 2022

Agenda item:			10.9.4.1

Source:	Intel Corporation

Title:	Simulation assumption for PDSCH requirement

Document for:	Approval

# Background

This contribution is to capture the agreed PDSCH simulation assumptions to define HST FR2 PDSCH demodulation performance requirements.

The detailed discussions can be found in the e-mail discussion summary R4-2207433

# PDSCH simulation assumptions

Simulation assumptions for results alignment and further requirements definition are summarized in Table 1.

Table 1. PDSCH Simulation assumptions

| Parameter |  |  | Value |
| --- | --- | --- | --- |
| Carrier frequency |  |  | 30 GHz |
| Duplex mode |  |  | TDD |
| SCS |  |  | 120 |
| CBW |  |  | 200 MHz |
| Antenna configuration |  |  | 2x2 |
| TDD pattern |  |  | DDDSU, S: 10D + 2G + 2U Schedule PDSCH in special slots |
| PDSCH DMRS configuration | DMRS type |  | Type 1 |
|  | Number of additional DMRS |  | 2 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | Starting symbol (S) |  | 1 |
|  | Length (L) |  | Specific to each Reference channel |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
| CSI-RS for tracking | Resource set #1 | First subcarrier index in the PRB used for CSI-RS (k0) | 0 for CSI-RS resource 1,2,3,4 |
|  |  | First OFDM symbol in the PRB used for CSI-RS | l0 = 5 for CSI-RS resource 1 and 2 |
|  |  |  | l0 = 9 for CSI-RS resource 3 and 4 |
|  |  | CSI-RS periodicity | 80 for CSI-RS resource 1,2,3,4 |
|  |  | CSI-RS offset | 2 for CSI-RS resource 1 and 2 |
|  |  |  | 3 for CSI-RS resource 3 and 4 |
|  |  | QCL info | TCI state #4 |
|  |  | Frequency Occupation | Start PRB 0 |
|  |  |  | Number of PRB =ceil(BWP size/4)*4 |
|  | Resource set #2 | First subcarrier index in the PRB used for CSI-RS (k0) | 0 for CSI-RS resource 5,6,7,8 |
|  |  | First OFDM symbol in the PRB used for CSI-RS | l0 = 4 for CSI-RS resource 5 and 6 |
|  |  |  | l0 = 8 for CSI-RS resource 7 and 8 |
|  |  | CSI-RS periodicity | 80 for CSI-RS resource 5,6,7,8 |
|  |  | CSI-RS offset | 2 for CSI-RS resource 5 and 6 |
|  |  |  | 3 for CSI-RS resource 7 and 8 |
|  |  | QCL info | TCI state #5 |
|  |  | Frequency Occupation | Start PRB 0 |
|  |  |  | Number of PRB =ceil(BWP size/4)*4 |
|  | Resource set #3 | First subcarrier index in the PRB used for CSI-RS (k0) | 1 for CSI-RS resource 9,10,11,12 |
|  |  | First OFDM symbol in the PRB used for CSI-RS | l0 = 5 for CSI-RS resource 9 and 10 |
|  |  |  | l0 = 9 for CSI-RS resource 11 and 12 |
|  |  | CSI-RS periodicity | 80 for CSI-RS resource 9,10,11,12 |
|  |  | CSI-RS offset | 2 for CSI-RS resource 9 and 10 |
|  |  |  | 3 for CSI-RS resource 11 and 12 |
|  |  | QCL info | TCI state #6 |
|  |  | Frequency Occupation | Start PRB 0 |
|  |  |  | Number of PRB =ceil(BWP size/4)*4 |
|  | Resource set #4 | First subcarrier index in the PRB used for CSI-RS (k0) | 1 for CSI-RS resource 13,14,15,16 |
|  |  | First OFDM symbol in the PRB used for CSI-RS | l0 = 4 for CSI-RS resource 13 and 14 |
|  |  |  | l0 = 8 for CSI-RS resource 15 and 16 |
|  |  | CSI-RS periodicity | 80 for CSI-RS resource 13,14,15,16 |
|  |  | CSI-RS offset | 2 for CSI-RS resource 13 and 14 |
|  |  |  | 3 for CSI-RS resource 15 and 16 |
|  |  | QCL info | TCI state #7 |
|  |  | Frequency Occupation | Start PRB 0 |
|  |  |  | Number of PRB =ceil(BWP size/4)*4 |
|  | Resource set #13 (Note2) | First subcarrier index in the PRB used for CSI-RS (k0) | 2 for CSI-RS resource 17,18,19,20 |
|  |  | First OFDM symbol in the PRB used for CSI-RS | l0 = 5 for CSI-RS resource 17 and 18 |
|  |  |  | l0 = 9 for CSI-RS resource 19 and 20 |
|  |  | CSI-RS periodicity | 80 for CSI-RS resource 17,18,19,20 |
|  |  | CSI-RS offset | 2 for CSI-RS resource 17 and 18 |
|  |  |  | 3 for CSI-RS resource 19 and 20 |
|  |  | QCL info | TCI state #12 |
|  |  | Frequency Occupation | Start PRB 0 |
|  |  |  | Number of PRB =ceil(BWP size/4)*4 |
|  | Resource set #14 (Note2) | First subcarrier index in the PRB used for CSI-RS (k0) | 2 for CSI-RS resource 21,22,23,24 |
|  |  | First OFDM symbol in the PRB used for CSI-RS | l0 = 4 for CSI-RS resource 21 and 22 |
|  |  |  | l0 = 8 for CSI-RS resource 23 and 24 |
|  |  | CSI-RS periodicity | 80 for CSI-RS resource 21,22,23,24 |
|  |  | CSI-RS offset | 2 for CSI-RS resource 21 and 22 |
|  |  |  | 3 for CSI-RS resource 23 and 24 |
|  |  | QCL info | TCI state #13 |
|  |  | Frequency Occupation | Start PRB 0 |
|  |  |  | Number of PRB =ceil(BWP size/4)*4 |
|  | Resource set #15 (Note2) | First subcarrier index in the PRB used for CSI-RS (k0) | 3 for CSI-RS resource 25,26,27,28 |
|  |  | First OFDM symbol in the PRB used for CSI-RS | l0 = 5 for CSI-RS resource 25 and 26 |
|  |  |  | l0 = 9 for CSI-RS resource 27 and 28 |
|  |  | CSI-RS periodicity | 80 for CSI-RS resource 25,26,27,28 |
|  |  | CSI-RS offset | 2 for CSI-RS resource 25 and 26 |
|  |  |  | 3 for CSI-RS resource 27 and 28 |
|  |  | QCL info | TCI state #14 |
|  |  | Frequency Occupation | Start PRB 0 |
|  |  |  | Number of PRB =ceil(BWP size/4)*4 |
|  | Resource set #16 (Note2) | First subcarrier index in the PRB used for CSI-RS (k0) | 3 for CSI-RS resource 29,30,31,32 |
|  |  | First OFDM symbol in the PRB used for CSI-RS | l0 = 4 for CSI-RS resource 29 and 30 |
|  |  |  | l0 = 8 for CSI-RS resource 31 and 32 |
|  |  | CSI-RS periodicity | 80 for CSI-RS resource 29,30,31,32 |
|  |  | CSI-RS offset | 2 for CSI-RS resource 29 and 30 |
|  |  |  | 3 for CSI-RS resource 31 and 32 |
|  |  | QCL info | TCI state #15 |
|  |  | Frequency Occupation | Start PRB 0 |
|  |  |  | Number of PRB =ceil(BWP size/4)*4 |
| NZP CSI-RS for CSI acquisition | Resource set #5 | First subcarrier index in the PRB used for CSI-RS (k0) | 0 |
|  |  | First OFDM symbol in the PRB used for CSI-RS | l0 = 12 |
|  |  | CSI-RS periodicity | 160 |
|  |  | CSI-RS offset | 0 |
|  |  | QCL info | TCI state #0 |
|  | Resource set #6 | First subcarrier index in the PRB used for CSI-RS (k0) | 2 |
|  |  | First OFDM symbol in the PRB used for CSI-RS | l0 = 12 |
|  |  | CSI-RS periodicity | 160 |
|  |  | CSI-RS offset | 0 |
|  |  | QCL info | TCI state #1 |
|  | Resource set #7 | First subcarrier index in the PRB used for CSI-RS (k0) | 4 |
|  |  | First OFDM symbol in the PRB used for CSI-RS | l0 = 12 |
|  |  | CSI-RS periodicity | 160 |
|  |  | CSI-RS offset | 0 |
|  |  | QCL info | TCI state #2 |
|  | Resource set #8 | First subcarrier index in the PRB used for CSI-RS (k0) | 6 |
|  |  | First OFDM symbol in the PRB used for CSI-RS | l0 = 12 |
|  |  | CSI-RS periodicity | 160 |
|  |  | CSI-RS offset | 0 |
|  |  | QCL info | TCI state #3 |
|  | Resource set #17 (Note2) | First subcarrier index in the PRB used for CSI-RS (k0) | 0 |
|  |  | First OFDM symbol in the PRB used for CSI-RS | l0 = 13 |
|  |  | CSI-RS periodicity | 160 |
|  |  | CSI-RS offset | 1 |
|  |  | QCL info | TCI state #8 |
|  | Resource set #18 (Note2) | First subcarrier index in the PRB used for CSI-RS (k0) | 2 |
|  |  | First OFDM symbol in the PRB used for CSI-RS | l0 = 13 |
|  |  | CSI-RS periodicity | 160 |
|  |  | CSI-RS offset | 1 |
|  |  | QCL info | TCI state #9 |
|  | Resource set #19 (Note2) | First subcarrier index in the PRB used for CSI-RS (k0) | 4 |
|  |  | First OFDM symbol in the PRB used for CSI-RS | l0 = 13 |
|  |  | CSI-RS periodicity | 160 |
|  |  | CSI-RS offset | 1 |
|  |  | QCL info | TCI state #10 |
|  | Resource set #20 (Note2) | First subcarrier index in the PRB used for CSI-RS (k0) | 6 |
|  |  | First OFDM symbol in the PRB used for CSI-RS | l0 = 13 |
|  |  | CSI-RS periodicity | 160 |
|  |  | CSI-RS offset | 1 |
|  |  | QCL info | TCI state #11 |
| CSI-RS for beam refinement | Resource set #9 | First subcarrier index in the PRB used for CSI-RS | k0=0 for CSI-RS resource 1,2 |
|  |  | First OFDM symbol in the PRB used for CSI-RS | l0 = 8 for CSI-RS resource 1 l0 = 9 for CSI-RS resource 2 |
|  |  | CSI-RS periodicity | 160 |
|  |  | CSI-RS offset | 0 |
|  |  | QCL info | TCI state #0 |
|  | Resource set #10 | First subcarrier index in the PRB used for CSI-RS | k0=1 for CSI-RS resource 3,4 |
|  |  | First OFDM symbol in the PRB used for CSI-RS | l0 = 8 for CSI-RS resource 3 l0 = 9 for CSI-RS resource 4 |
|  |  | CSI-RS periodicity | 160 |
|  |  | CSI-RS offset | 0 |
|  |  | QCL info | TCI state #1 |
|  | Resource set #11 | First subcarrier index in the PRB used for CSI-RS | k0=2 for CSI-RS resource 5,6 |
|  |  | First OFDM symbol in the PRB used for CSI-RS | l0 = 8 for CSI-RS resource 5 l0 = 9 for CSI-RS resource 6 |
|  |  | CSI-RS periodicity | 160 |
|  |  | CSI-RS offset | 0 |
|  |  | QCL info | TCI state #2 |
|  | Resource set #12 | First subcarrier index in the PRB used for CSI-RS | k0=3 for CSI-RS resource 7,8 |
|  |  | First OFDM symbol in the PRB used for CSI-RS | l0 = 8 for CSI-RS resource 7 l0 = 9 for CSI-RS resource 8 |
|  |  | CSI-RS periodicity | 160 |
|  |  | CSI-RS offset | 0 |
|  |  | QCL info | TCI state #3 |
|  | Resource set #21 (Note2) | First subcarrier index in the PRB used for CSI-RS | k0=0 for CSI-RS resource 9,10 |
|  |  | First OFDM symbol in the PRB used for CSI-RS | l0 = 10 for CSI-RS resource 9 l0 = 11 for CSI-RS resource 10 |
|  |  | CSI-RS periodicity | 160 |
|  |  | CSI-RS offset | 1 |
|  |  | QCL info | TCI state #8 |
|  | Resource set #22 (Note2) | First subcarrier index in the PRB used for CSI-RS | k0=1 for CSI-RS resource 11,12 |
|  |  | First OFDM symbol in the PRB used for CSI-RS | l0 = 10 for CSI-RS resource 11 l0 = 11 for CSI-RS resource 12 |
|  |  | CSI-RS periodicity | 160 |
|  |  | CSI-RS offset | 1 |
|  |  | QCL info | TCI state #9 |
|  | Resource set #23 (Note2) | First subcarrier index in the PRB used for CSI-RS | k0=2 for CSI-RS resource 13,14 |
|  |  | First OFDM symbol in the PRB used for CSI-RS | l0 = 10 for CSI-RS resource 13 l0 = 11 for CSI-RS resource 14 |
|  |  | CSI-RS periodicity | 160 |
|  |  | CSI-RS offset | 1 |
|  |  | QCL info | TCI state #10 |
|  | Resource set #24 (Note2) | First subcarrier index in the PRB used for CSI-RS | k0=3 for CSI-RS resource 15,16 |
|  |  | First OFDM symbol in the PRB used for CSI-RS | l0 = 10 for CSI-RS resource 15 l0 = 11 for CSI-RS resource 16 |
|  |  | CSI-RS periodicity | 160 |
|  |  | CSI-RS offset | 1 |
|  |  | QCL info | TCI state #11 |
| TCI state #0 | Type 1 QCL information | CSI-RS resource | CSI-RS resource 1 from 'CSI-RS for tracking Resource set #1' configuration |
|  |  | QCL Type | Type A |
|  | Type 2 QCL information | CSI-RS resource | CSI-RS resource 1 from 'CSI-RS for tracking Resource set #1' configuration |
|  |  | QCL Type | Type D |
| TCI state #1 | Type 1 QCL information | CSI-RS resource | CSI-RS resource 5 from 'CSI-RS for tracking Resource set #2' configuration |
|  |  | QCL Type | Type A |
|  | Type 2 QCL information | CSI-RS resource | CSI-RS resource 5 from 'CSI-RS for tracking Resource set #2' configuration |
|  |  | QCL Type | Type D |
| TCI state #2 | Type 1 QCL information | CSI-RS resource | CSI-RS resource 9 from 'CSI-RS for tracking Resource set #3' configuration |
|  |  | QCL Type | Type A |
|  | Type 2 QCL information | CSI-RS resource | CSI-RS resource 9 from 'CSI-RS for tracking Resource set #3' configuration |
|  |  | QCL Type | Type D |
| TCI state #3 | Type 1 QCL information | CSI-RS resource | CSI-RS resource 13 from 'CSI-RS for tracking Resource set #4' configuration |
|  |  | QCL Type | Type A |
|  | Type 2 QCL information | CSI-RS resource | CSI-RS resource 13 from 'CSI-RS for tracking Resource set #4' configuration |
|  |  | QCL Type | Type D |
| TCI state #8 (Note2) | Type 1 QCL information | CSI-RS resource | CSI-RS resource 17 from 'CSI-RS for tracking Resource set #13' configuration |
|  |  | QCL Type | Type A |
|  | Type 2 QCL information | CSI-RS resource | CSI-RS resource 17 from 'CSI-RS for tracking Resource set #13' configuration |
|  |  | QCL Type | Type D |
| TCI state #9 (Note2) | Type 1 QCL information | CSI-RS resource | CSI-RS resource 21 from 'CSI-RS for tracking Resource set #14' configuration |
|  |  | QCL Type | Type A |
|  | Type 2 QCL information | CSI-RS resource | CSI-RS resource 21 from 'CSI-RS for tracking Resource set #14' configuration |
|  |  | QCL Type | Type D |
| TCI state #10 (Note2) | Type 1 QCL information | CSI-RS resource | CSI-RS resource 25 from 'CSI-RS for tracking Resource set #15' configuration |
|  |  | QCL Type | Type A |
|  | Type 2 QCL information | CSI-RS resource | CSI-RS resource 25 from 'CSI-RS for tracking Resource set #15' configuration |
|  |  | QCL Type | Type D |
| TCI state #11 (Note2) | Type 1 QCL information | CSI-RS resource | CSI-RS resource 29 from 'CSI-RS for tracking Resource set #16' configuration |
|  |  | QCL Type | Type A |
|  | Type 2 QCL information | CSI-RS resource | CSI-RS resource 29 from 'CSI-RS for tracking Resource set #16' configuration |
|  |  | QCL Type | Type D |
| TCI state #4 | Type 1 QCL information | SSB index | SSB #0 |
|  |  | QCL Type | Type C |
|  | Type 2 QCL information | SSB index | SSB #0 |
|  |  | QCL Type | Type D |
| TCI state #5 | Type 1 QCL information | SSB index | SSB #1 |
|  |  | QCL Type | Type C |
|  | Type 2 QCL information | SSB index | SSB #1 |
|  |  | QCL Type | Type D |
| TCI state #6 | Type 1 QCL information | SSB index | SSB #2 |
|  |  | QCL Type | Type C |
|  | Type 2 QCL information | SSB index | SSB #2 |
|  |  | QCL Type | Type D |
| TCI state #7 | Type 1 QCL information | SSB index | SSB #3 |
|  |  | QCL Type | Type C |
|  | Type 2 QCL information | SSB index | SSB #3 |
|  |  | QCL Type | Type D |
| TCI state #12 (Note2) | Type 1 QCL information | SSB index | SSB #4 |
|  |  | QCL Type | Type C |
|  | Type 2 QCL information | SSB index | SSB #4 |
|  |  | QCL Type | Type D |
| TCI state #13 (Note2) | Type 1 QCL information | SSB index | SSB #5 |
|  |  | QCL Type | Type C |
|  | Type 2 QCL information | SSB index | SSB #5 |
|  |  | QCL Type | Type D |
| TCI state #14 (Note2) | Type 1 QCL information | SSB index | SSB #6 |
|  |  | QCL Type | Type C |
|  | Type 2 QCL information | SSB index | SSB #6 |
|  |  | QCL Type | Type D |
| TCI state #15 (Note2) | Type 1 QCL information | SSB index | SSB #7 |
|  |  | QCL Type | Type C |
|  | Type 2 QCL information | SSB index | SSB #7 |
|  |  | QCL Type | Type D |
| PTRS configuration | Frequency density (KPT-RS) |  | 2 |
|  | Time density (TPT-RS) |  | 1 |
| MCS and Rank |  |  | MCS 17 and Rank 2 |
| Number of HARQ Processes |  |  | 8 |
| SSB and CSI-RS for tracking assumptions |  |  | For Test 1-1, SSB # (2k mod 8) , CSI-RS (for tracking) resource set # ((k mod 4)+1), CSI-RS (for CSI acquisition) resource set # ((k mod 4) + 5) and CSI-RS (for beam refinement) resource set # ((k mod 4) + 9) are transmitted by kth RRH; SSB # ((2k mod 8)+1) , CSI-RS (for tracking) resource set # ((k mod 4) + 13), CSI-RS (for CSI acquisition) resource set # ((k mod 4) + 17) and CSI-RS (for beam refinement) resource set # ((k mod 4) + 21) are transmitted by kth RRH For Test 1-2, SSB # (k mod 4) , CSI-RS (for tracking) resource set # ((k mod 4)+1), CSI-RS (for CSI acquisition) resource set # ((k mod 4) + 5) and CSI-RS (for beam refinement) resource set # ((k mod 4) + 9) are transmitted by kth RRH |
| SSB periodicity |  |  | 20ms |
| SSB slot offset |  |  | 0 for HST-DPS-FR2-UNI-A 0 and 1 for HST-DPS-FR2-BI-B |
| Propagation conditions |  |  | HST-DPS-FR2-BI-B HST-DPS-FR2-UNI-A |
| Maximum Doppler shift |  |  | 9722 |
| Test metric |  |  | SNR at 70% of maximum throughput |
| Note 1: Other remaining parameters can be found in TS 38.101-4 Table 7.2-1 and Table 7.2.2.2.1-2 Note 2: Only configured for Test 1-2 (HST-DPS-FR2-BI-B) Note 3: Test 1-1: HST-DPS-FR2-UNI-A, Test 1-2: HST-DPS-FR2-BI-B |  |  |  |