3GPP TSG-RAN WG4 Meeting #102-e	R4-2207254

Electronic Meeting, 21 February – 3 March 2022

Agenda Item:	10.6.5

Source:	Ericsson

Title:	Way forward for NR DL1024QAM demodulation and CQI reporting requirements

Document for:	Approval

# 1	PDSCH demodulation requirements

- Interested companies are encouraged to provide the PDSCH simulation results according to Table 1 in the next meeting to decide the SNR test points.

- Add 1dB additional margin on top of the average of companies’ impairment results for PDSCH 1024QAM.

Table 1	Simulation parameters for PDSCH demodulation for DL 1024QAM in FR1.

| Parameters | FDD | TDD |
| --- | --- | --- |
| SCS/CBW | 15kHz and 10MHz | 30kHz and 40MHz |
| TDD UL/DL configuration | N/A | 7D1S2U, with S=6:4:4 Schedule PDSCH in the special slots |
| MCS (MCS index table 4) | MCS23 |  |
| Rank | 1 |  |
| Antenna configuration | 2x2 with ULA Low 2x4 with ULA Low |  |
| Propagation channel | TDLD30-5 |  |
| PDSCH configuration | Type A mapping, Start symbol 2, Duration 12 |  |
| PDSCH DMRS configuration | Type 1, Single symbol, additional DMRS: pos1 |  |
| Tx EVM assumed for simulation | 2.5% |  |
| SSB configuration | Periodicity 20 ms, Allocated in first slot within 20ms |  |
| TRS configuration | 20 ms periodicity, 2 slots, Offset 10 ms |  |
| Number of HARQ processes | 4 for FDD 15kHz, 8 for TDD 30kHz |  |
| Transform precoding | CP-OFDM |  |
| Allocated RBs | Full BWP |  |
| PRB bundling | 2 |  |
| Precoding model | Random Precoding, per slot and per RB bundling size granularity (codebook configuration: Single panel Type 1) |  |
| Receiver type | MMSE-IRC |  |
| Test metric | 70% of maximum throughput |  |

# 2	SDR requirements

- Set the practical MCS indexes with table 4 (TS38.214 Table 5.1.3.1-4) for SDR test for 2Rx as follows:

  - Rank 1: MCS23

  - Rank 2: Not define SDR test

- Set the practical MCS indexes with table 4 (TS38.214 Table 5.1.3.1-4) for SDR test for 4Rx as follows:

  - Rank 1: MCS24

  - Rank 2: MCS23

- Set MCSreqj as shown in Table 2 and Table 3 from the formula:

  - MCSreqj= min⁡(MCSupperboundj, MCSpracticalj)

Table 2	MCS indexes for SDR test with 1024QAM for 2Rx

| vLayersj | Qmj | fj | MCSupperboundj | MCSpracticalj | MCSreqj |
| --- | --- | --- | --- | --- | --- |
| 1 | 10 | 1 | 25 | [23] | [23] |
| 1 | 10 | 0.8 | 21 |  | [21] |
| 1 | 10 | 0.75 | 19 |  | [19] |
| 1 | 10 | 0.4 | 9 |  | [9] |

Table 3	MCS indexes for SDR test with 1024QAM for 4Rx

| vLayersj | Qmj | fj | MCSupperboundj | MCSpracticalj | MCSreqj |
| --- | --- | --- | --- | --- | --- |
| 1 | 10 | 1 | 25 | [24] | [24] |
| 1 | 10 | 0.8 | 21 |  | [21] |
| 1 | 10 | 0.75 | 19 |  | [19] |
| 1 | 10 | 0.4 | 9 |  | [9] |
| 2 | 10 | 1 | 25 | [23] | [23] |
| 2 | 10 | 0.8 | 21 |  | [21] |
| 2 | 10 | 0.75 | 19 |  | [19] |
| 2 | 10 | 0.4 | 9 |  | [9] |

# 3	CQI reporting tests

- Discuss the final CQI definition requirements with CQI table 4 considering the impairment margin and SNR levels provided by TE with the assumption of Tx EVM 2.5%.

  - 2Rx UE:

    - This is the common understanding SNR corresponding to CQI index 14 for 1024QAM table is around 28-30dB for 2Rx UE without impairment margin.

      - Option 1: SNR = 28/29dB

      - Option 2: SNR = 29/30dB

  - 4Rx UE:

    - This is the common understanding SNR corresponding to CQI index 14 for 1024QAM table is around 26-28dB for 4Rx UE without impairment margin.

      - Option 1: SNR = 25/26dB

      - Option 2: SNR = 26/27dB

# 4	CR work split

- Volunteer companies are encouraged to submit the draft CR in RAN4#103-e.

| Clause | CR | Volunteer company |
| --- | --- | --- |
| 5.1.1.x 6.1.1.x B.2.1.1 | Applicability of demodulation requirements Applicability of CSI reporting requirements Delay profiles for FR1 (TDLD30-5) | Qualcomm |
| 5.2.2.1.x 5.2.3.1.x A.3.2.1.x | PDSCH demodulation requirements for FDD (2Rx and 4Rx) FRC for PDSCH FDD | MediaTek |
| 5.2.2.2.x 5.2.3.2.x A.3.2.2.x | PDSCH demodulation requirements for TDD (2Rx and 4Rx) FRC for PDSCH TDD | Apple |
| 6.2.2.1.x 6.2.2.2.x A.4 | CQI reporting definition under AWGN conditions for FDD (2Rx and 4Rx) CSI reference measurement channels | Huawei |
| 6.2.3.1.x 6.2.3.2.x 5.5A | CQI reporting definition under AWGN conditions for TDD (2Rx and 4Rx) Sustained downlink data rate provided by lower layers | Ericsson |