3GPP TSG-RAN WG4 Meeting # 102-e 		                       R4-2207337

Electronic Meeting, February 21 – March 3, 2022

Title: 	TP to TS 38.108: clause 4.3 requirement reference point

Source: 	CATT

Agenda item:	6.13.1.4

Document for:	Approval

- Introduction

This contribution provides a text proposal for TS 38.108: clause 4.3 requirement reference point [1]..

- Reference

[1] TR 38.108, 0.0.1

- Text proposal

---------------------------------------------------Start of Text proposal---------------------------------------------------------

4.3	Requirement reference points

- 4.3.1 SAN type 1-H

For SAN type 1-H, the requirements are defined for two points of reference, signified by radiated requirements and conducted requirements.

Figure 4.3.1-1: Radiated and conducted reference points for SAN type 1-H

Radiated characteristics are defined over the air (OTA), where the radiated interface is referred to as the Radiated Interface Boundary (RIB). Radiated requirements are also referred to as OTA requirements. The (spatial) characteristics in which the OTA requirements apply are detailed for each requirement.

Conducted characteristics are defined at individual or groups of TAB connectors at the transceiver array boundary, which is the conducted interface between the transceiver unit array and the composite antenna.

The transceiver unit array is part of the composite transceiver functionality receiving and transmitting modulated signal s to ensure radio links with ground base stations and users.

The satellite payload is composed by a transceiver unit array and a composite antenna array. The transceiver unit array contains an implementation specific number of transmitter units and an implementation specific number of receiver units.

The composite antenna contains a radio distribution network (RDN) and an antenna array. The RDN is a linear passive network which distributes the RF power generated by the transceiver unit array to the antenna array, and/or distributes the radio signals collected by the antenna array to the transceiver unit array, in an implementation specific way.

How a conducted requirement is applied to the transceiver array boundary is detailed in the respective requirement clause.

- 4.3.1 SAN type 1-O

For SAN type 1-O, the radiated characteristics are defined over the air (OTA), where the operating band specific radiated interface is referred to as the Radiated Interface Boundary (RIB). Radiated requirements are also referred to as OTA requirements. The (spatial) characteristics in which the OTA requirements apply are detailed for each requirement.

Figure 4.3.2-1: Radiated reference points for SAN type 1-O

Co-location requirements are specified at the conducted interface of the co-location reference antenna, the co-location reference antenna does not form part of the BS under test but is a means to provide OTA power levels which are representative of a co-located system, further defined in clause 4.9.

For a SAN type 1-O the transceiver unit array must contain at least 8 transmitter units and at least 8 receiver units. Transmitter units and receiver units may be combined into transceiver units. The transmitter/receiver units have the ability to transmit/receive parallel independent modulated symbol streams.

---------------------------------------------------End of Text proposal---------------------------------------------------------