3GPP TSG-RAN WG4 Meeting # 102-e                                                       R4-2207359			R4-22xxxxx

# Electronic Meeting, February 21 – March 3, 2022

Source:	ZTE Corporation

Title:	TP for TS 38.108 OTA output power dynamics(9.4)

Agenda Item:	10.13.3.1

Document for:	Approval

- Introduction

In terms of the work split in the approved WF[1]. This contribution provides a text proposal to TS38.108 [2] on sub-clause 9.4(OTA outpur power dynamics).

- Reference

[1] R4-2203080, Way Forward on NTN_solutions_Part1, THALES

[2] R4-2203086,  Draft skeleton for TS 38.101-8, THALES

- Text Proposal

----- Start of TP -----

9.4	OTA output power dynamics

9.4.1	General

Transmit signal quality (as specified in clause 9.6) shall be maintained for the output power dynamics requirements.

The OTA output power requirements are directional requirements and apply to the beam peak directions over the OTA peak directions set.

9.4.2	OTA RE power control dynamic range

9.4.2.1	General

The OTA RE power control dynamic range is the difference between the power of an RE and the average RE power for a SAN at maximum output power (Pmax,c,EIRP) for a specified reference condition.

This requirement shall apply at each RIB supporting transmission in the operating band.

9.4.2.2	Minimum requirement for SAN type 1-O

The OTA RE power control dynamic range is specified the same as the conducted RE power control dynamic range requirement for SAN type 1-H in table 6.3.2.2-1.

9.4.3	OTA total power dynamic range

9.4.3.1	General

The OTA total power dynamic range is the difference between the maximum and the minimum transmit power of an OFDM symbol for a specified reference condition.

This requirement shall apply at each RIB supporting transmission in the operating band.

NOTE 1:	The upper limit of the OTA total power dynamic range is the SAN maximum carrier EIRP (Pmax,c,EIRP) when transmitting on all RBs. The lower limit of the OTA total power dynamic range is the average EIRP for single RB transmission in the same direction using the same beam. The OFDM symbol carries PDSCH and not contain RS or SSB.

9.4.3.2	Minimum requirement for SAN type 1-O

OTA total power dynamic range minimum requirement for SAN type 1-O is specified such as for each NR carrier it shall be larger than or equal to the levels specified for the conducted requirement for SAN type 1-H in table 6.3.3.2-1.

----- End of TP -----