3GPP TSG-RAN4 Meeting #102-e                                                         R4-2207403

# Online, , 21st Feb 2022 - 3rd Mar 2022						(Revision of R4-2205043)

Source:	Ligado Networks

Title:	TP for TR 38.863: Unwanted emissions for NTN satellite UEs transmitting in 1626.5 to 1660.5 MHz

Agenda item:			10.13.4.1

Document for:	Approval

# 1. Introduction

This TP proposes updates the regulatory section of TR 38.863 [1] to include the regulatory aspects for NTN L-band n255.

# 2. Reference

- TR 38.863 v0.2.0, Solutions for NR to support non-terrestrial networks (NTN): Non-terrestrial networks (NTN) related RF and co-existence (Release 17)

- FCC Code of Federal Regulations

# 3. Text Proposal

<Start of Text Proposal>

##### 7.4.2.2.3 	Output power dynamics

For Transmit OFF power and Power control, the framework and requirements for existing TN UE can be reused for satellite UE. The transmit OFF power is -50 dBm for 20MHz.

Since handheld UE always transmits the maximum output power for satellite access scenario based on the NR NTN calibration summary in R4-2115628, FFS whether relaxed value is needed or not based on existing TN UE requirements

##### 7.4.2.2.4 	Frequency error

The UE basic measurement interval of modulated carrier frequency is 1 UL slot. The mean value of basic measurements of UE modulated carrier frequency shall be accurate to within ± 0.1 PPM observed over a period of 1 ms of cumulated measurement intervals compared to the carrier frequency received from the Satellite Access Node.

NOTE:	The requirements is applicable only when PVT ephemeris updated at least once [10] seconds.

##### 7.4.2.2.5 	Transmit modulation quality

[To be updated]

##### 7.4.2.2.6 	Spectrum emission mask

##### 7.4.2.2.7 	ACLR

[To be updated]

##### 7.4.2.2.8 	Spurious emissions

The spurious emissions limits specified in TS 38.101-1[17] sub-clause 6.5.3 would also be applicable to NTN satellite access UEs.

Considering FCC rule 47 C.F.R § 25.216 (h) [23], TN UE’s general spurious emission requirements will be inadequate for L-band NTN satellite UE operation in the US. A new NS value and a subclause with Table 7.4.2.2.8-1 needs to be specified for n255 for NTN satellite operations in the US.

Table 7.4.2.2.8-1: Additional requirements for NS_57

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit1 (dBW) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 5 MHz, 10 MHz, 15 MHz, 20 MHz |  |  |
| 1559≤ f ≤ 1605 | -80 | 700 Hz | Averaged over any 2 millisecond active transmission interval |
| 1605≤ f ≤ 1610 | -80 + 24/5 (f-1605) | 700Hz |  |
| 1559 ≤ f ≤ 1605 | -70 | 1MHz | Averaged over any 2 millisecond active transmission interval |
| 1605≤ f ≤ 1610 | -70 + 24/5 (f-1605) | 1MHz |  |
| NOTE 1: The EIRP requirement in regulation is converted to conducted requirement using a 0 dBi antenna. |  |  |  |

<End of Text Proposal>