3GPP TSG-RAN4 Meeting #102-e	    R4-2207474

# Electronic Meeting, 21 February – 3 March 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.105 | CR | 250 | rev | - | Current version: | 15.15.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Big CR for TS 37.105 Maintenance (Rel-15, CAT F) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15 |  |  |  |  |  | Date: |  |  | 2021-03-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This big CR merges endorsed draft CR to TS 37.105 in RAN4#102-e. The reason for change in endorsed draft CR is copied below: R4-2204446: BS OBUE requirements clarification, rel-15 In RAN4#101e, corrections of NOTE for OBUE requirement tables for NR specs were agreed. Similar corections are required for MSR specs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The summary of change in endorsed draft CR is copied below. R4-2204446: BS OBUE requirements clarification, rel-15 Added clarification text in NOTE in tables for OBUE requirements. Deleted unnecessary text in NOTE in tables for OBUE requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The consequences if not approved for endorsed draft CR are coppied below. R4-2204446: BS OBUE requirements clarification, rel-15 Without the clarification text, how to derive “cumulative sum” is not clear when measurement bandwidthes are different. Unnecessary text in the NOTE which is never applied could cause misunderstanding. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.4.3.3, 6.6.5.2.2, 6.6.5.2.3, 9.7.5.2.2, 9.7.5.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 37.145-1, 37.145-2 CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

Table 6.6.4.3.3-1: Basic Limits for spectrum emission mask values, Prated,c,cell-10*log10(NTXU,countedpercell)  34 dBm for 1,28 Mcps TDD

| Frequency offset of measurement filter centre frequency, f_offset | Basic Limit | Measurement bandwidth |
| --- | --- | --- |
| 0.8 MHz f < 1.0 MHz | -20 dBm | 30 kHz |
| 1.0 MHz f < 1.8 MHz |  | 30 kHz |
| (NOTE) | -28 dBm | 30 kHz |
| 1.8 MHz f fmax | -13 dBm | 1 MHz |
| NOTE: For a multi-band TAB connector with Inter RF Bandwidth gap less than 8MHz, the basic limit within the Inter RF Bandwidth gap is calculated as a cumulative sum of emissions from the two adjacent carriers on each side of the Inter RF Bandwidth gap, where the contribution from the far-end RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end RF Bandwidth. |  |  |

Table 6.6.4.3.3-2: Basic Limits for spectrum emission mask values, 26 dBm  Prated,c,cell-10*log10(NTXU,countedpercell) < 34 dBm for 1,28 Mcps TDD

| Frequency offset of measurement filter centre frequency, f_offset | Basic Limit | Measurement bandwidth |
| --- | --- | --- |
| 0.8 MHz f < 1.0 MHz | Prated,c,cell – 10*log10(NTXU,countedpercell) -54 dB | 30 kHz |
| 1.0 MHz f < 1.8 MHz | Prated,c,cell – 10*log10(NTXU,countedpercell) -54-10*(f_offset-1,015)dB | 30 kHz |
| (NOTE) | Prated,c,cell – 10*log10(NTXU,countedpercell) -62 dB | 30 kHz |
| 1.8 MHz f fmax | Prated,c,cell – 10*log10(NTXU,countedpercell) - 47 dB | 1 MHz |
| NOTE: For a multi-band TAB connector with Inter RF Bandwidth gap less than 8MHz, the basic limit within the Inter RF Bandwidth gap is calculated as a cumulative sum of emissions from the two adjacent carriers on each side of the Inter RF Bandwidth gap, where the contribution from the far-end RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end RF Bandwidth. |  |  |

Table 6.6.4.3.3-3: Basic Limits for spectrum emission mask values, Prated,c,cell-10*log10(NTXU,countedpercell) < 26 dBm for 1,28 Mcps TDD

| Frequency offset of measurement filter centre frequency, f_offset | Basic Limit | Measurement bandwidth |
| --- | --- | --- |
| 0.8 MHz f < 1.0 MHz | -28 dBm | 30 kHz |
| 1.0 MHz f < 1.8 MHz |  | 30 kHz |
| (NOTE) | -36 dBm | 30 kHz |
| 1.8 MHz f fmax | -21 dBm | 1 MHz |
| NOTE: For a multi-band TAB connector with Inter RF Bandwidth gap less than 8MHz, the basic limit within the Inter RF Bandwidth gap is calculated as a cumulative sum of emissions from the two adjacent carriers on each side of the Inter RF Bandwidth gap, where the contribution from the far-end RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end RF Bandwidth. |  |  |

<Next change>

Table 6.6.5.2.2-1a: WA BS OBUE in BC1 and BC3 bands ≤ 1 GHz applicable for: BS supporting NR and not supporting UTRA

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic Limit (Note 1, 2) | Measurement bandwidth (Note 4) |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -14 dBm | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -16 dBm (Note 5) | 100 kHz |
| NOTE 1: For MSR TAB connector supporting non-contiguous spectrum operation within any operating band, the basic limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the basic limit within sub-block gaps shall be -16dBm/100kHz. NOTE 2: For MSR multi band TAB connector with Inter RF Bandwidth gap < 2×ΔfOBUEthe basic limit within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base station RF Bandwidth on each side of the Inter RF Bandwidth gap. |  |  |  |

<Next change>

Table 6.6.5.2.2-2a: MR BS OBUE in BC1 bands applicable for: BS with maximum output power 31 < Prated,c,cell-10*log10(NTXU,countedpercell)  38 dBm, supporting NR, and not supporting UTRA

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic Limit (Note 1, 2) | Measurement bandwidth (Note 4) |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz | Prated,c,cell – 10*log10(NTXU,countedpercell) – 53 dB - (7/5)*(f_offset/MHz-0,05) dB | 100 kHz |
| 5 MHz f < min(10 MHz, Δfmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | Prated,c,cell – 10*log10(NTXU,countedpercell) - 60 dB | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | Min(Prated,c,cell – 10*log10(NTXU,countedpercell) - 60 dB, - 25 dBm) (Note 5) | 100 kHz |
| NOTE 1: For MSR TAB connector supporting non-contiguous spectrum operation within any operating band the basic limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the basic limit within sub-block gaps shall be Min(Prated,c,cell – 10*log10(NTXU,countedpercell) – 60 dB, -25 dBm) / 100 kHz. NOTE 2: For MSR multi band TAB connector with Inter RF Bandwidth gap < 2×ΔfOBUEthe basic limit within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap. |  |  |  |

<Next change>

Table 6.6.5.2.2-3a: MR BS OBUE in BC1 bands applicable for: BS with maximum output power Prated,c,cell-10*log10(NTXU,countedpercell)  31 dBm, supporting NR, and not supporting UTRA

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limit (Note 1, 2) | Measurement bandwidth (Note 4) |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, Δfmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -29 dBm | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -29 dBm (Note 5) | 100 kHz |
| NOTE 1: For MSR TAB connector supporting non-contiguous spectrum operation within any operating band the basic limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the basic limit within sub-block gaps shall be -29dBm/100kHz. NOTE 2: For MSR multi band TAB connector with Inter RF Bandwidth gap < 2×ΔfOBUEthe basic limit within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap. |  |  |  |

<Next change>

Table 6.6.5.2.3-1a: WA BS OBUE in BC2 bands ≤ 1 GHz applicable for: BS supporting NR, not operating NRin band n8, and not supporting UTRA

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limit (Note 1, 2) | Measurement bandwidth (Note 10) |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -14 dBm | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -16 dBm (Note 11) | 100 kHz |
| NOTE 1: For MSR TAB connector supporting non-contiguous spectrum operation within any operating band, the basic limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the basic limit within sub-block gaps shall be -16dBm/100kHz. NOTE 2: For MSR multi band TAB connector with Inter RF Bandwidth gap < 2×ΔfOBUEthe basic limit within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 3: For operation with an E-UTRA 1.4 or 3 MHz carrier adjacent to the Base Station RF Bandwidth edge, the limits in table 6.6.5.2.3-2 apply for 0 MHz f < 0.15 MHz. |  |  |  |

<Next change>

Table 6.6.5.2.3-3a: MR BS OBUE in BC2 bands applicable for: BS with maximum output power 31 < Prated,c,cell-10*log10(NTXU,countedpercell)  38 dBm, supporting NR, and not supporting UTRA

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limit (Note 1, 2) | Measurement bandwidth (Note 10) |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz | Prated,c,cell – 10*log10(NTXU,countedpercell) -53 dB - (7/5)*(f_offset/MHz - 0,05) dB | 100 kHz |
| 5 MHz f < min(10 MHz, Δfmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | Prated,c,cell – 10*log10(NTXU,countedpercell) - 60 dB | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | Min(Prated,c,cell – 10*log10(NTXU,countedpercell) - 60 dB, -25 dBm) (Note 11) | 100 kHz |
| NOTE 1: For MSR TAB connectors supporting non-contiguous spectrum operation within any operating band the basic limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the basic limit within sub-block gaps shall be Min(Prated,c,cell – 10*log10(NTXU,countedpercell) -60 dB, -25 dBm) / 100 kHz. NOTE 2: For MSR multi band TAB connector with Inter RF Bandwidth gap < 2×ΔfOBUE the basic limit within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 3: For operation with an E-UTRA 1.4 or 3 MHz carrier adjacent to the Base Station RF Bandwidth edge, the limits in table 6.6.5.2.3-5 apply for 0 MHz f < 0.15 MHz. |  |  |  |

<Next change>

Table 6.6.5.2.3-4a: MR BS OBUE in BC2 bands applicable for: BS with maximum output power Prated,c,cell-10*log10(NTXU,countedpercell)  31 dBm, supporting NR, and not supporting UTRA

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limit (Note 1, 2) | Measurement bandwidth (Note 10) |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, Δfmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -29 dBm | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -29 dBm (Note 11) | 100 kHz |
| NOTE 1: For MSR TAB connectors supporting non-contiguous spectrum operation within any operating band the basic limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the basic limit within sub-block gaps shall be -29dBm/100kHz. NOTE 2: For MSR multi band TAB connector with Inter RF Bandwidth gap < 2×ΔfOBUEthe basic limit within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 3: For operation with an E-UTRA 1.4 or 3 MHz carrier adjacent to the Base Station RF Bandwidth edge, the limits in table 6.6.5.2.3-5 apply for 0 MHz f < 0.15 MHz. |  |  |  |

<Next change>

Table 9.7.5.2.2-1a: WA BS OBUE in BC1 and BC3 bands ≤ 1 GHz applicable for: BS supporting NR and not supporting UTRA

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement (Note 1, 2) | Measurement bandwidth (Note 4) |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -5 dBm | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -7 dBm (Note 5) | 100 kHz |
| NOTE 1: For MSR RIB supporting non-contiguous spectrum operation within any operating band, the minimum requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the minimum requirement within sub-block gaps shall be -7dBm/100kHz. NOTE 2: For MSR multi band RIB with Inter RF Bandwidth gap < 2×ΔfOBUE the minimum requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base station RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 3: For operation with an E-UTRA 1.4 or 3MHz carrier adjacent to the Base Station RF Bandwidth edge, the limits in Table 6.6.2.2-2 apply for 0 MHz f < 0.15 MHz. |  |  |  |

<Next change>

Table 9.7.5.2.2-2a: MR BS OBUE in BC1 bands applicable for: BS with maximum output power 40 < Prated,c,TRP  47 dBm BS, supporting NR, and not supporting UTRA

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement (Note 1, 2) | Measurement bandwidth (Note 4) |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz | Prated,c,TRP – 53 dB - (7/5)*(f_offset-0,05) dB | 100 kHz |
| 5 MHz f < min(10 MHz, Δfmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | Prated,c,TRP – 60 dB | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | Min(Prated,c,TRP – 60 dB, -16 dBm) (Note 4) | 100 kHz |
| NOTE 1: For MSR RIB supporting non-contiguous spectrum operation within any operating band the minimum requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the minimum requirement within sub-block gaps shall be Min(Prated,c,TRP – 60 dB, -16 dBm)/100 kHz. NOTE 2: For MSR multi band RIB with Inter RF Bandwidth gap < 2×ΔfOBUE the minimum requriement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap. |  |  |  |

<Next change>

Table 9.7.5.2.2-3a: MR BS OBUE in BC1 bands applicable for: BS with maximum output power Prated,c,TRP  40 dBm, supporting NR, and not supporting UTRA

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement (Note 1, 2) | Measurement bandwidth (Note 4) |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz | -13 dBm – 7/5(f_offset/MHz – 0.05) dB | 100 kHz |
| 5 MHz f < min(10 MHz, Δfmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -20 dBm | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -20 dBm (Note 4) | 100 kHz |
| NOTE 1: For MSR RIB supporting non-contiguous spectrum operation within any operating band the minimum requriement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the minimum requirement within sub-block gaps shall be -20 dBm/100 kHz. NOTE 2: For MSR multi band RIB with Inter RF Bandwidth gap < 2×ΔfOBUE the minimum requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap. |  |  |  |

<Next change>

Table 9.7.5.2.3-1a: WA BS OBUE in BC2 bands ≤ 1 GHz applicable for: BS supporting NR, not operating NR in band n8, and not supporting UTRA

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement (Note 1, 2) | Measurement bandwidth (Note 10) |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz | 2 dBm – 7/5(f_offset/MHz – 0.05) dB | 100 kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -5 dBm | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -7 dBm (Note 11) | 100 kHz |
| NOTE 1: For MSR RIB supporting non-contiguous spectrum operation within any operating band, the minimum requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the minimum requirement within sub-block gaps shall be -7 dBm/100 kHz. NOTE 2: For MSR multi band RIB with Inter RF Bandwidth gap < 2×ΔfOBUE the minimum requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 3: For operation with an E-UTRA 1.4 or 3 MHz carrier adjacent to the Base Station RF Bandwidth edge, the limits in table 9.7.5.2.3-2 apply for 0 MHz f < 0.15 MHz. |  |  |  |

<Next change>

Table 9.7.5.2.3-3a: MR BS OBUE in BC2 bands applicable for: BS with maximum output power 40 < Prated,c,TRP  47 dBm, supporting NR, and not supporting UTRA,

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement (Note 1, 2) | Measurement bandwidth (Note 10) |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz | Prated,c,TRP – 53 dB - (7/5)*(f_offset-0,05) dB | 100 kHz |
| 5 MHz f < min(10 MHz, Δfmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | Prated,c,TRP – 60 dB | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | Min(Prated,c,TRP – 60 dB, -16 dBm) (Note 11) | 100 kHz |
| NOTE 1: For MSR RIBs supporting non-contiguous spectrum operation within any operating band the minimum requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the minimum requirement within sub-block gaps shall be Min(Prated,c,TRP - 60 dB, -16 dBm)/100 kHz. NOTE 2: For MSR multi band RIB with Inter RF Bandwidth gap < 2×ΔfOBUE the minimum requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 3: For operation with an E-UTRA 1.4 or 3 MHz carrier adjacent to the Base Station RF Bandwidth edge, the limits in table 9.7.5.2.3-5 apply for 0 MHz f < 0.15 MHz. |  |  |  |

<Next change>

Table 9.7.5.2.3-4a: MR BS OBUE in BC2 bands applicable for: BS maximum output power Prated,c,TRP  40 dBm, supporting NR, and not supporting UTRA

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement (Note 1, 2) | Measurement bandwidth (Note 10) |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz | -13 dBm – 7/5(f_offset/MHz – 0.05) dB | 100 kHz |
| 5 MHz f < min(10 MHz, Δfmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -20 dBm | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -20 dBm (Note 11) | 100 kHz |
| NOTE 1: For MSR RIBs supporting non-contiguous spectrum operation within any operating band the minimum requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block ga. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the minimum requirement within sub-block gaps shall be -20 dBm/100 kHz. NOTE 2: For MSR multi band TAB connector with Inter RF Bandwidth gap < 2×ΔfOBUE the minimum requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 3: For operation with an E-UTRA 1.4 or 3 MHz carrier adjacent to the Base Station RF Bandwidth edge, the limits in table 9.7.5.2.3-6 apply for 0 MHz f < 0.15 MHz. |  |  |  |

<End of change>