3GPP TSG-RAN WG4 Meeting #103-e	R4-2207629

9 – 20 May, 2022

Source:	Qualcomm Incorporated

Title:	Calculation of MSD for V2X_n5A-n47A, V2X_5A_n47A and V2X_n5A_47A and accompanying TP

Agenda item:	8.20.2

Work Item:	          NR_LTE_V2X_PC5_combos-Core

Document for:	Approval

# 1.	Introduction

In RAN#94e [1] new inter-band V2X combinations for V2X_n5A-n47A, V2X_5A_n47A and V2X_n5A_47A was introduced. In this paper we present the MSDs required for band 47 for these V2X band combinations.

# 2. 	Discussion

In [1] the inter-band combinations V2X_n5A-n47A, V2X_5A_n47A and V2X_n5A_47A were introduced. For these combinations the 7th harmonic of band 5 can fall into band 47 causing degradation in sensitivity. Previously, a similar band combination, 5-47 [2] had allocated 0.2 dB relaxation in ΔRIB,c and another 0.2dB relaxation in ΔTIB,c . Our proposal is to follow these same relaxations for V2X_n5A-n47A, V2X_5A_n47A and V2X_n5A_47A. Therefore, we propose that the ΔTIB,c   relaxation for V2X_n5A-n47A, V2X_5A_n47A and V2X_n5A_47A is 0.2 dB and the ΔRIB,c  relaxation for V2X_n5A-n47A, V2X_5A_n47A and V2X_n5A_47A is also 0.2 dB

Proposal 1: ΔTIB,c   relaxation for V2X_n5A-n47A, V2X_5A_n47A and V2X_n5A_47A is 0.2 dB

Table 1: ΔTIB,c  for V2X_n5A-n47A, V2X_5A_n47A and V2X_n5A_47A

| V2X con-current band combination | NR/E-UTRA operating band | ΔTIB,c [dB] |
| --- | --- | --- |
| V2X_n5A-n47A | n5 | 0.2 |
| V2X_5A_n47A | 5 | 0.2 |
| V2X_n5A_47A | n5 | 0.2 |

Proposal 2: ΔRIB,c   relaxation V2X_n5A-n47A, V2X_5A_n47A and V2X_n5A_47A is 0.2 dB

Table 2: ΔRIB,c  for V2X_n5A-n47A, V2X_5A_n47A and V2X_n5A_47A

| V2X con-current band combination | NR/E-UTRA band | ΔRIB,c [dB] |
| --- | --- | --- |
| V2X_n5A-n47A | n5 | 0.2 |
| V2X_5A_n47A | 5 | 0.2 |
| V2X_n5A_47A | n5 | 0.2 |

# Conclusion

We present the ΔTIB,c and ΔRIB,c relaxations for inter-band combinations V2X_n5A-n47A, V2X_5A_n47A and V2X_n5A_47A and make the following proposals.

Proposal 1: ΔTIB,c   relaxation for V2X_n5A-n47A, V2X_5A_n47A and V2X_n5A_47A is 0.2 dB

Table 1: ΔTIB,c  for V2X_n5A-n47A, V2X_5A_n47A and V2X_n5A_47A

| V2X con-current band combination | NR/E-UTRA operating band | ΔTIB,c [dB] |
| --- | --- | --- |
| V2X_n5A-n47A | n5 | 0.2 |
| V2X_5A_n47A | 5 | 0.2 |
| V2X_n5A_47A | n5 | 0.2 |

Proposal 2: ΔRIB,c   relaxation V2X_n5A-n47A, V2X_5A_n47A and V2X_n5A_47A is 0.2 dB

Table 2: ΔRIB,c  for V2X_n5A-n47A, V2X_5A_n47A and V2X_n5A_47A

| V2X con-current band combination | NR/E-UTRA band | ΔRIB,c [dB] |
| --- | --- | --- |
| V2X_n5A-n47A | n5 | 0.2 |
| V2X_5A_n47A | 5 | 0.2 |
| V2X_n5A_47A | n5 | 0.2 |

# Annex - TP for TR 37.875 for V2X_n5A-n47A, V2X_5A_n47A and V2X_n5A_47A

## --Start of changes

***********************************Start of the TP*************************************

# 1	Scope

The present document is the Technical Report on TR on band combinations for con-current operation of NR/LTE Uu bands/band combinations and one NR/LTE V2X band.

The purpose of the present document is to study the extension of the band combinations for V2X service to grow the NR V2X ecosystem. Operators propose new band combinations for con-current operation of NR/LTE Uu bands/band combinations and one NR/LTE V2X band. Whether to specify con-current operation of LTE/NR CA/DC band combinations and V2X band depends on requests in Rel-17. Specifically, the self-desensitization problem of con-current operation band combinations will be analysed including harmonics, IMD problem. Also, the candidate solutions will be studied to solve the self-desensitization problem.

Table 1-1: Release 17 NR V2X band combinations

| V2X Band combination | REL independent from |
| --- | --- |
| V2X_n1-n47 | Rel-16 |
| V2X_n5-n47 | Rel-16 |
| V2X_n8-n47 | Rel-16 |
| V2X_n39-n47 | Rel-16 |
| V2X_n40-n47 | Rel-16 |
| V2X_n41-n47 | Rel-16 |
| V2X_n78-n47 | Rel-16 |
| V2X_n79-n47 | Rel-16 |
| V2X_n1-n47 | Rel-16 |
| V2X_n8-n47 | Rel-16 |
| V2X_n5-47 | Rel-16 |
| V2X_n39_47 | Rel-16 |
| V2X_n40_47 | Rel-16 |
| V2X_n41_47 | Rel-16 |
| V2X_n78_47 | Rel-16 |
| V2X_n79_47 | Rel-16 |
| V2X_1_n47 | Rel-16 |
| V2X_5_n47 | Rel-16 |
| V2X_8_n47 | Rel-16 |
| V2X_3_n47 | Rel-16 |
| V2X_39_n47 | Rel-16 |
| V2X_40_n47 | Rel-16 |
| V2X_41_n47 | Rel-16 |

Note: All band combinations in table 1-1 that are release independent from Rel-16 are optional

# 6	Con-current operation with one Uu band and one PC5 band

## 6.1	Con-current operation between one LTE Uu band and one NR PC5 band

<<Unchanged section omitted>>

### 6.1.7	V2X_5A_n47A

#### 6.1.7.1	Operating bands for V2X_5A_n47A

The operating bands for V2X_5A_n47A are specified in table 6.1.7.1-1.

Table 6.1.7.1-1: Inter-band con-current V2X operating bands for V2X_5A_n47A

| V2X con-current configuration | E-UTRA / NR Operating Band | Interface | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| V2X_5A_n47A | 5 | Uu | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n47 | PC5 | 5855 MHz | – | 5925 MHz | 5855 MHz | – | 5925 MHz | HD |

#### 6.1.7.2	Channel bandwidths per operating band for V2X_5A_n47A

The channel bandwidths per operating band for V2X_5A_n47A are specified in table 6.1.7.2-1.

Table 6.1.7.2-1: V2X inter-band con-current configurations and bandwidth combination sets for V2X_5A_n47A

| V2X inter-band Configuration | E-UTRA / NR operating Band | SCS kHz | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V2X_5A_n47A | 5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  | 50 | 0 |
|  | n47 | 15 |  |  |  | Yes |  | Yes |  | Yes | Yes |  |  |  |
|  |  | 30 |  |  |  | Yes |  | Yes |  | Yes | Yes |  |  |  |
|  |  | 60 |  |  |  | Yes |  | Yes |  | Yes | Yes |  |  |  |

#### 6.1.7.3	UE co-existence studies

The UE co-existence studies specified for V2X_n5A-n47A in clause 6.2.8.3 are applicable to V2X_5A_n47A since band 5 and band n5 have the same frequency range.

#### 6.1.7.4	MSD, ∆TIB and ∆RIB values

The ΔTIB,c and ΔRIB,c relaxations are given in the following tables

Table 6.1.7.4-1: ΔTIB,c  for V2X_5A_n47A

| V2X con-current band combination | E-UTRA operating band | ΔTIB,c [dB] |
| --- | --- | --- |
| V2X_5A_n47A | 5 | 0.2 |

Table 6.1.7.4-2: ΔRIB,c  for V2X_5A_n47A

| V2X con-current band combination | E-UTRA band | ΔRIB,c [dB] |
| --- | --- | --- |
| V2X_5A_n47A | 5 | 0.2 |

## 6.2	Con-current operation between one NR Uu band and one NR PC5 band

<<Unchanged section omitted>>

### 6.2.8	V2X_n5A-n47A

#### 6.2.8.1	Operating bands for V2X_n5A-n47A

The operating bands for V2X_n5A-n47A are specified in table 6.2.8.1-1.

Table 6.2.8.1-1: Inter-band con-current V2X operating bands for V2X_n5A-n47A

| V2X con-current configuration | E-UTRA / NR Operating Band | Interface | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| V2X_n5A_n47A | n5 | Uu | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n47 | PC5 | 5855 MHz | – | 5925 MHz | 5855 MHz | – | 5925 MHz | HD |

#### 6.2.8.2	Channel bandwidths per operating band for V2X_n5A-n47A

The channel bandwidths per operating band for V2X_n5A-n47A are specified in table 6.2.8.2-1.

Table 6.2.8.2-1: V2X inter-band con-current configurations and bandwidth combination sets for V2X_n5A-n47A

| V2X inter-band Configuration | NR operating Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V2X_n5A-n47A | n5 | 15 | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 65 | 0 |
|  |  | 30 |  | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n47 | 15 |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |

#### 6.2.8.3	Coexistence studies

The harmonics analysis for V2X_n1A-n47A is specified in table 6.2.8.3-1. Up to 4th harmonics of band n1 are provided since the frequency range of the 5th harmonics is much higher than 5.9GHz. The harmonics of band n47 are not listed as the harmonics distributed in the frequency range much higher than 5.9GHz have no impact on GNSS and ISM bands. Based on the harmonics analysis, it is observed that the 3rd harmonic of band n1 falls on band n47.

Table 6.2.8.3-1: Harmonics analysis for V2X_n5A-n47A

| Operating Band | Band n5 |  | Band n47 |  |
| --- | --- | --- | --- | --- |
| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| UL frequency (MHz) | 824 | 849 | 5855 | 5925 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | No effect |  |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 |  |  |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | No effect |  |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 |  |  |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | No effect |  |
| 4th harmonics frequency limits (MHz) | 3296 | 3396 |  |  |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | No effect |  |
| 5th harmonics frequency limits (MHz) | 4120 | 4245 |  |  |
| 6th harmonics frequency limits | 6*fx_low | 6*fx_high | No effect |  |
| 6th harmonics frequency limits (MHz) | 4944 | 5094 |  |  |
| 7th harmonics frequency limits | 7*fx_low | 7*fx_high | Has Impact |  |
| 7th harmonics frequency limits (MHz) | 5768 | 5943 |  |  |

The IMD analysis for V2X_n5A-n47A is specified in table 6.2.8.3-2. Up to the 5th order IMDs of band n5 and band n47 are provided. Based on the IMD analysis, it is observed that no IMD products fall into the associated bands.

| Operating Band | Band n5 |  | Band n47 |  |
| --- | --- | --- | --- | --- |
| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| UL frequency (MHz) | 824 | 849 | 5855 | 5925 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 11710 | 11850 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 17565 | 17775 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5006 | 5101 | 6679 | 6774 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 4277 | 4157 | 10861 | 11026 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7503 | 7623 | 12534 | 12699 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3453 | 3308 | 16716 | 16951 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8327 | 8472 | 18389 | 18624 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 10202 | 10012 | 13358 | 13548 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 22876 | 22571 | 2459 | 2629 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 24244 | 24549 | 9151 | 9321 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 16127 | 15867 | 9163 | 9378 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 19213 | 19473 | 14182 | 14397 |

Table 6.2.8.3-2: IMD analysis for V2X_n5A-n47A

The harmonics and intermodulation products should be evaluated when V2X inter-band con-current operating UE coexists with other systems such as GNSS and ISM. The harmonics and IMD analysis of V2X_n5A-n47A for GNSS and ISM bands is shown in table 6.2.8.3-3. Based on the analysis for GNSS and ISM bands, band n47 and 7th harmonics have an impact on the ISM band (5GHz).

Table 6.2.8.3-3: Harmonic and IMDs analysis of V2X_n5A-n47A UE for GNSS and ISM bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 7th harmonic n5, Band n47 |
|  | 5150 | - | 5350 | Yes | Europe |  |
|  | 5470 | - | 5725 | Yes |  |  |
|  | 5150 | - | 5825 | Yes | Asia | 7th harmonic n5 |

#### 6.2.8.4	MSD, ∆TIB and ∆RIB values

The ΔTIB,c and ΔRIB,c relaxations are given in the following tables

Table 6.2.8.4-1: ΔTIB,c  for V2X_n5A_n47A

| V2X con-current band combination | NR operating band | ΔTIB,c [dB] |
| --- | --- | --- |
| V2X_n5A_n47A | n5 | 0.2 |

Table 6.2.8.4-2: ΔRIB,c  for V2X_n5A_n47A

| V2X con-current band combination | NR band | ΔRIB,c [dB] |
| --- | --- | --- |
| V2X_n5A_n47A | n5 | 0.2 |

## 6.3	Con-current operation between one NR Uu band and one LTE PC5 band

<<Unchanged section omitted>>

### 6.3.8	V2X_n5A_47A

#### 6.3.8.1	Operating bands for V2X_n5A_47A

The operating bands for V2X_n5A_47A are specified in table 6.3.5.1-1.

Table 6.3.8.1-1: Inter-band con-current V2X operating bands for V2X_n5A_47A

| V2X con-current configuration | E-UTRA / NR Operating Band | Interface | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| V2X_n5A_47A | n5 | Uu | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | 47 | PC5 | 5855 MHz | – | 5925 MHz | 5855 MHz | – | 5925 MHz | HD |

#### 6.3.8.2	Channel bandwidths per operating band for V2X_n5A_47A

The channel bandwidths per operating band for V2X_n5A_47A are specified in table 6.3.8.2-1.

Table 6.3.8.2-1: V2X inter-band con-current configurations and bandwidth combination sets for V2X_n1A_47A

| V2X inter-band Configuration | E-UTRA / NR operating Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V2X_n5A_47A | n5 | 15 | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  | 45 | 0 |
|  |  | 30 |  | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | 47 | 15 |  | Yes |  | Yes |  |  |  |  |  |  |  |  |

#### 6.3.8.3	UE co-existence studies

The UE co-existence studies specified for V2X_n5A-n47A in clause 6.2.8.3 are applicable to V2X_n5A_47A since band 47 and band n47 have the same frequency range.

#### 6.3.7.4	MSD, ∆TIB and ∆RIB values

The ΔTIB,c and ΔRIB,c relaxations are given in the following tables

Table 6.3.7.4-1: ΔTIB,c   for V2X_n5A_47A

| V2X con-current band combination | NR operating band | ΔTIB,c [dB] |
| --- | --- | --- |
| V2X_n5A_47A | n5 | 0.2 |

Table 6.3.7.4-2: ΔRIB,c  for V2X_n5A_47A

| V2X con-current band combination | NR band | ΔRIB,c [dB] |
| --- | --- | --- |
| V2X_n5A_47A | n5 | 0.2 |

--End of changes

# References

[1] RP-213297, Revised WID: Band combinations for concurrent operation of NR/LTE Uu bands/band combinations and one NR/LTE V2X PC5 band, RAN#94-e, Dec. 2021

[2] TS 36.101, User equipment radio transmission and reception, V17.5.0, Mar. 2022