3GPP TSG-RAN WG4 Meeting #103-e	R4-2207725

Electronic Meeting, 09th May – 20th May 2022

Source:	AT&T

Title:	TP for TR 38.841 Update for n77 PC2 UL and Addition of n77 PC1.5 UL for CA_n30-n77

Agenda item:			8.24.2

Document for:	Approval

# 1. Introduction

A revised text proposal for TR 38.841 to update CA_n30-n77 to address missing MSD due to harmonic mixing and cross band isolation for PC2 Case B and to complete single uplink PC1.5 requirements.

# 2. Text Proposal

---Start of changes---

## 5.17	CA_n30-n77

### 5.17.1	Configurations

Table 5.17.1-1: NR CA configurations and bandwidth combinations sets for supporting power class 2 and single uplink PC1.5

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n30A-n77A | n778,9 CA_n30A-n77A8 | n30 | 5, 10 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n30A-n77(2A) | n778,9 CA_n30A-n77A8 | n30 | 5, 10 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 5.17.2	Maximum output power

Table 5.17.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_n30A-n77A | CA power class | Carrier n12 power class | Carrier n77 power class |
| --- | --- | --- | --- | --- |
| CA_n30A-n77A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

The tolerance +2/-3 dB is applied. Also, when the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB.

### 5.17.3	REFSENS requirements

According to the PC3 CA_n30A-n77A study captured in TR 38.717-02-01, the following co-existence issues were identified:

- 4th order IMD products may fall into own Rx frequencies of band n30.

Therefore, additional MSD values for IMD4 need to be defined for PC2 CA_n30A-n77A.

#### 5.17.3.1	Power class 2 Case A

The additional MSD due to intermodulation for PC2 CA_n30A-n77A are defined in Table 5.17.3.1-1.

Table 5.17.3.1-1: MSD test points for PCell due to dual uplink operation for PC2 NR CA in NR FR1 (two bands)

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n30-n77 | n30 | 2310 | 5 | 25 | 2355 | 17.6 | FDD | IMD4 |
|  | n77 | 3487.5 | 10 | 50 | 3487.5 | N/A | TDD | N/A |

#### 5.17.3.2	Power class 2 Case B

Using a similar approach as CA_n25-n77 to calculate the PC2 harmonic mixing MSD based on the PC3 case, the MSD was calculated by starting with MSD for PC3 and calculating the PC2 MSD using this equation, where X is set to 3 due to the 3dB increase in interference.

The additional MSD due to receiver harmonic mixing for Case B are defined in Table 5.17.3.2-1.

Table 5.17.3.2-1: Reference sensitivity exceptions due to harmonic mixing from a PC2 aggressor NR UL band for NR DL CA FR1

| NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n77 | n302 | 13.2 | 10.6 |  |  |  |  |  |  |  |  |  |
| NOTE 2: The requirements should be verified for UL NR-ARFCN of the aggressor (high) band (superscript HB) such that in MHz and with carrier frequency in the victim (lower) band in MHz and the channel bandwidth configured in the higher band. |  |  |  |  |  |  |  |  |  |  |  |  |

The additional MSD due to intermodulation for PC2 Case B CA_n30A-n77A are same as the Case A defined in Table 5.17.3.1-1.

Although no cross-band isolation MSD was identified for PC3 n77 into n30, PC2 n77 cross band isolation into n41 can be considered as a baseline and de-rated accordingly.

PC3 n77 into n41: MSD=4.5dB (note that there is 4.5dB for n78 into n40 too) => UL noise ~3dB above REFSENS

PC3 n77 into n30: MSD<1dB = “no MSD” => UL noise <-6dB below REFSENS => 9 dB below noise in n41

PC2 n77 into n41: MSD=6.5dB => UL noise ~5.5dB above REFSENS

PC2 n77 into n30 => UL noise at 5.5-9=-3.5dB => ~1dB MSD

Table 5.17.3.2-2: Reference sensitivity exceptions (MSD) due to cross band isolation from a PC2 aggressor NR UL band for NR CA FR1

| NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n77 | n30 | 1.0 | 1.0 |  |  |  |  |  |  |  |  |  |  |  |

Table 5.17.3.2-3: Uplink configuration for reference sensitivity exceptions due to cross band isolation for NR CA FR1

| NR Band / SCS / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n77 | n30 | 30 | 270 | 270 |  |  |  |  |  |  |  |  |  |  |  |

#### 5.17.3.3	Single uplink n77 Power Class 1.5

Comparing with PC2 CA_n30-n77, additional MSD is expected for PC1.5 n77 single uplink due to higher Tx power in n77. The MSD was calculated by starting with MSD for PC3 and calculating the PC1.5 MSD using this equation, where X is set to 6 due to the 6dB increase in interference.

The additional MSD due to receiver harmonic mixing for single uplink PC1.5 n77 are defined in table 5.17.3.3-1.

Table 5.17.3.3-1: Reference sensitivity exceptions due to harmonic mixing from a PC1.5 NR UL band for NR DL CA FR1

| NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n77 | n302 | 16.1 | 13.5 |  |  |  |  |  |  |  |  |  |
| NOTE 2: The requirements should be verified for UL NR-ARFCN of the aggressor (high) band (superscript HB) such that in MHz and with carrier frequency in the victim (lower) band in MHz and the channel bandwidth configured in the higher band. |  |  |  |  |  |  |  |  |  |  |  |  |

Although no cross-band isolation MSD was identified for PC3 n77 into n30, it was suggested that for PC2 and PC1.5 n77 cross band isolation into n41 be considered and de-rated. The PC1.5 cross-band isolation MSD was calculated by starting with the 1.0 dB cross-band isolation MSD for PC2 from clause 5.17.3.2 and calculating the PC1.5 MSD using this equation, where X is set to 3 due to the 3dB increase in interference.

The result is 1.8 dB.

Therefore, the following is proposed.

Table 5.17.3.3-2: Reference sensitivity exceptions (MSD) due to cross band isolation from a PC1.5 aggressor NR single UL band for DL NR CA FR1

| NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n77 | n30 | 1.8 | 1.8 |  |  |  |  |  |  |  |  |  |  |  |

### 5.17.4		∆TIB and ∆RIB values

For the ∆TIB,c and ∆RIB,c values, same PC3 CA_n30A-n77A requirements are applied for PC2 CA_n30A-n77A.

---End of changes---

# References