3GPP TSG-RAN WG4 Meeting # 103-e	R4-2207796

# Electronic Meeting, May 09 - May 20, 2022

Agenda item:	9.6.4.2

Source:	Apple Inc.

Title:	Discussion on SDR requirements with 1KQAM

Release:	Rel-17

Document for:	Discussion

# 1. Introduction

In RAN4#102-e SDR requirements with 1024QAM in NR DL were discussed and way forward [1] was agreed.

- Set the practical MCS indexes with table 4 (TS38.214 Table 5.1.3.1-4) for SDR test for 2Rx as follows: - Rank 1: MCS23 - Rank 2: Not define SDR test - Set the practical MCS indexes with table 4 (TS38.214 Table 5.1.3.1-4) for SDR test for 4Rx as follows: - Rank 1: MCS24 - Rank 2: MCS23 - Set MCSreqj as shown in Table 2 and Table 3 from the formula: - MCSreqj= min⁡(MCSupperboundj, MCSpracticalj)

# 2. Discussion

In RAN4#101-e it was agreed to add MCS indices based on the methodology agreed in Rel-15 for SDR requirements.

  - MCSreqj= min⁡(MCSupperboundj, MCSpracticalj)

In [1] the MCS indices in Table1 and Table 2 for SDR test were agreed

Table 1: MCS indexes for SDR test with 1024QAM for 2Rx

| vLayersj | Qmj | fj | MCSupperboundj | MCSpracticalj | MCSreqj |
| --- | --- | --- | --- | --- | --- |
| 1 | 10 | 1 | 25 | [23] | [23] |
| 1 | 10 | 0.8 | 21 |  | [21] |
| 1 | 10 | 0.75 | 19 |  | [19] |
| 1 | 10 | 0.4 | 9 |  | [9] |

Table 2: MCS indexes for SDR test with 1024QAM for 4Rx

| vLayersj | Qmj | fj | MCSupperboundj | MCSpracticalj | MCSreqj |
| --- | --- | --- | --- | --- | --- |
| 1 | 10 | 1 | 25 | [24] | [24] |
| 1 | 10 | 0.8 | 21 |  | [21] |
| 1 | 10 | 0.75 | 19 |  | [19] |
| 1 | 10 | 0.4 | 9 |  | [9] |
| 2 | 10 | 1 | 25 | [23] | [23] |
| 2 | 10 | 0.8 | 21 |  | [21] |
| 2 | 10 | 0.75 | 19 |  | [19] |
| 2 | 10 | 0.4 | 9 |  | [9] |

Based on the simulation results from companies it was concluded that the operating SNR for Rank2 with 2RX was not practical to introduce SDR requirements. It was agreed that no SDR requirements will be introduced for 1024QAM with Rank 2 with 2RX. For completeness we should use the existing SDR requirements for 2 MIMO layers with 256QAM for 1024QAM with 2RX.

Proposal #1: For Rank 2 with 2RX re-use the existing SDR requirements for 2 MIMO layers with 256QAM for 1024QAM with 2RX.

# 3. Conclusion

In this paper, we provide we propose SDR requirements with 1024QAM for 2 RX with rank 2. Our proposals are captured below:

Proposal #1: For Rank 2 with 2RX re-use the existing SDR requirements for 2 MIMO layers with 256QAM for 1024QAM with 2RX.

# Reference

- R4-2203044, “Way forward for NR DL1024QAM demodulation and CQI reporting requirements”, Ericsson.