3GPP TSG-RAN WG4 Meeting # 103-e	R4-2207802

# Electronic Meeting, May 09 - May 20, 2022

Agenda item:	9.11.2.2

Source:	Apple Inc.

Title:	Simulation results for PDSCH requirements in MU-MIMO scenarios

Release:	Rel-17

Document for:	Discussion

# 1. Introduction

In RAN4#102-e PDSCH demodulation requirements with MMSE-IRC for intra-cell inter-user interference was discussed and way forward [1] was agreed. In this contribution we present our simulation results for requirements definition in MU-MIMO.

# 2. Simulation Results

For the simulation assumptions agreed in [1], we present simulation results below.

Table 1: Simulation results for PDSCH demod in MU-MIMO

| Duplex | Ant Config | Prop. Channel | MCS | Co-scheduled UE precoder | SNR @ 70% TP |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Alignment results | Impairment results |
| FDD | 2x2 | TDLC300 | MCS13 | Random | 14.5 | 16.5 |
|  | 2x4 | TDLC300 | MCS13 | Random | 9.9 | 11.9 |
|  | 4x4 | TDLA30 | MCS13 | Orthogonal | 14.5 | 16.5 |
| TDD | 2x2 | TDLC300 | MCS13 | Random | 14.7 | 16.7 |
|  | 2x4 | TDLC300 | MCS13 | Random | 10.0 | 12.0 |
|  | 4x4 | TDLA30 | MCS13 | Orthogonal | 14.5 | 16.5 |

# 3. Conclusion

In this paper, we present our views on the open issues, simulation results for requirements definition for requirements in MU-MIMO.

# Reference

- R4-2207244, “Way Forward on MMSE-IRC for intra-cell inter-user interference”, Huawei, HiSilicon.