3GPP TSG-RAN WG4 Meeting # 103-e	R4-2207923

Electronic Meeting, 9 – 20 May 2022

Agenda Item:	9.15.4.1

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Proposals on BS transmitter requirements for extending current NR operation to 71 GHz

Document for:	Approval

# 1.	Introduction

The work item for extending current NR operation to 71 GHz was extended at TSG RAN#95-e [1]. One of the incomplete tasks listed in the approved exception sheet is requirement in 38.104 clause 9.6.2 (EVM window length within pending on conformance feasibility issues).

This contribution provides further proposals on this requirement based on the agreed WF in RAN4#102-e [2].

# 2.	Discussion

2.1	EVM

The following points were agreed in [2]:

-	Add new tables with information only agreed values and remaining FFS 480 kHz and 960 kHz as described in Table 2.4-3 and Table 2.4-4.

-	Put 50% EVM window length within [] until conformance test feasibility issues are resolved.

-	Note: The value on EVM window length can be further discussed and updated if needed.

-	FFT sizes to be also double checked by companies.

The EVM window length for BS type 2-O are defined as 50% of the normal CP length in TS 38-104 [3] for both 60 kHz and 120 kHz SCS for channel bandwidths between 50 to 400 MHz. For BS type 1-O, the same ratios are defined for channel bandwidths between 25 to 50 MHz, while a larger ratio (60% of the normal CP length) are defined for larger channel bandwidths. It is expected that larger channel bandwidths will commonly be used for NR operation in 52.6 – 71 GHz range, hence a larger ratio would be considered if the same logic for BS type 1-O is applied here. However, considering the implementation feasibility (e.g., filtering to meet out-of-band emissions requirements with the defined spectral utilization) in this higher frequency range (compared to current frequency range of BS type 2-O), we propose to define the EVM window length as 50% of the normal CP length for NR operation in 52.6 – 71 GHz range.

Proposal 1: Remove the [] around the 50% EVM window length for NR operation in 52.6 – 71 GHz range.

The FFT sizes for the EVM window length should be calculated according to the transmission bandwidth configuration for NR operation in 52.6 – 71 GHz range. The working assumptions in the agreed WF [4] are shown in Table 1 below.

Table 1: Transmission bandwidth configuration NRB for FR2-2

| SCS (kHz) | 100 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB |
| 120 | 66 | 264 | N/A | N/A | N/A |
| 480 | N/A | 66 | 132 | 264 | N/A |
| 960 | N/A | 33 | 66 | 132 | [156] |

The corresponding FFT sizes for the EVM window length are calculated and shown in Table 2 below.

Table 2: FFT sizes for the EVM window length for FR2-2

| SCS (kHz) | 100 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB |
| 120 | 1024 | 4096 | N/A | N/A | N/A |
| 480 | N/A | 1024 | 2048 | 4096 | N/A |
| 960 | N/A | 512 | 1024 | 2048 | [2048] |

Proposal 2: Use the FFT sizes in Table 2 for the EVM window length for NR operation in 52.6 – 71 GHz range.

# 3.	Conclusion

This contribution has provided further proposals on BS transmitter requirements for extending current NR operation to 71 GHz based on the agreed WF. They are summarized as follows.

Proposal 1: Remove the [] around the 50% EVM window length for NR operation in 52.6 – 71 GHz range.

Proposal 2: Use the FFT sizes in Table 2 for the EVM window length for NR operation in 52.6 – 71 GHz range.

Table 2: FFT sizes for the EVM window length for FR2-2

| SCS (kHz) | 100 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB |
| 120 | 1024 | 4096 | N/A | N/A | N/A |
| 480 | N/A | 1024 | 2048 | 4096 | N/A |
| 960 | N/A | 512 | 1024 | 2048 | [2048] |

# References

[1]	RP-220991, “Exception sheet on Extending current NR operation to 71GHz”, Qualcomm CDMA

Technologies, Intel.

[2]	R4-2207216, “WF on >52.6 GHz BS RF Tx Requirements”, Nokia, Nokia Shanghai Bell.

[3]	3GPP TS 38.104 v17.5.0: “NR; Base Station (BS) radio transmission and reception”.

[4]	R4-2206536, “WF on 60 GHz UE RF”, Qualcomm Incorporated.