3GPP TSG-RAN WG4 Meeting # 103-e	R4-2207988

# Electronic Meeting, May 9 - 20, 2022

Source: 	Skyworks Solutions, Inc.

Title: 	Additional criteria for triple beat issue detection

Agenda Item:	8.3.1 UE RF requirements [WI code]

Document for:	Approval

# Introduction

In the R4#102e meeting additional progress on triple beat MSD evaluation for inter-band combinations with two band UL configuration containing contiguous ULCA was captured in way forwards [1]. For this meeting, a formal check of all potential triple beat cases identified in the latest revision of 31.101-3 is presented in [2]. Based on this premise, we propose that an additional criterion related to band separation be added to detect the need for triple beat MSD analysis.

# Discussion

## Potential triple beat cases in latest 38.101-3

Table 1 provides the list of triple beat cases that have been analysed in [2]. The first order triple beat cases that require attention are highlighted in red in the “TB order” column.

Table 1: Two band inter-band ENDC with two UL band and LTE intra-band ULCA MSD analysis

| Band Combination | UL configuration | 1UL | Duplex [MHz] | max DLBW [MHz] | max ULBW [MHz] | 2UL | max aggBW [MHz] | TB order | comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1C_n3A | DC_1C_n3A | n3A | 95 | 50 | 50 | 1C | 40 | 1.113 | 2nd order TB OK |
| DC_2C_n71A | DC_2C_n71A | n71A | -46 | 35 | 20 | 2C | 40 | -1.85 | 2nd order TB OK |
| DC_3C_n1A | DC_3C_n1A | n1A | 190 | 50 | 50 | 3C | 40 | 3.488 | High order TB OK |
| DC_3C_n5A | DC_3C_n5A | n5A | 45 | 25 | 20 | 3C | 40 | 0.55 | Needs further analysis MB to LB -> R18 |
| DC_3C_n7A | DC_3C_n7A | n7A | 120 | 50 | 50 | 3C | 40 | 1.738 | 2nd order TB OK |
| DC_3C_n28A | DC_3C_n28A | n28A | 55 | 30 | 30 | 3C | 40 | 0.613 | Needs further analysis MB to LB -> R18 |
| DC_7C_n1A | DC_7C_n1A | n1A | 190 | 50 | 50 | 7C | 40 | 3.488 | High order TB OK |
| DC_7C_n3A | DC_7C_n3A | n3A | 95 | 50 | 50 | 7C | 40 | 1.113 | 2nd order TB OK |
| DC_7C_n5A | DC_7C_n5A | n5A | 45 | 25 | 20 | 7C | 40 | 0.55 | TB OK due to band distance? HB to LB |
| DC_7C_n28A | DC_7C_n28A | n28A | 55 | 30 | 30 | 7C | 40 | 0.613 | TB OK due to band distance? HB to LB |
| DC_41C_n1A | DC_41C_n1A | n1A | 190 | 50 | 50 | 41C | 40 | 3.488 | High order TB OK |
| DC_41C_n3A | DC_41C_n3A | n3A | 95 | 50 | 50 | 41C | 40 | 1.113 | 2nd order TB OK |
| DC_41C_n28A | DC_41C_n28A | n28A | 55 | 30 | 30 | 41C | 40 | 0.613 | TB OK due to band distance? HB to LB |
| DC_42C_n1A | DC_42C_n1A | n1A | 190 | 50 | 50 | 42C | 40 | 3.488 | High order TB OK |
| DC_42C_n3A | DC_42C_n3A | n3A | 95 | 50 | 50 | 42C | 40 | 1.113 | 2nd order TB OK |
| DC_42C_n28A | DC_42C_n28A | n28A | 55 | 30 | 30 | 42C | 40 | 0.625 | TB OK due to band distance? HB to LB |
| DC_3A_n7B | DC_3A_n7B | 3A | 95 | 20 | 20 | n7B | 50 | 1.5 | 2nd order TB OK |
| DC_8A_n79C | DC_8A_n79C | 8A | 45 | 10 | 10 | n79C | 200 | 0.175 | OK due to band distance? VHB to LB |
| DC_28A_n7B | DC_28A_n7B | 28A | 55 | 20 | 20 | n7B | 50 | 0.7 | OK due to band distance? HB to LB |
| DC_3A_n41C | DC_3A_n41C | 3A | 95 | 20 | 20 | 41C | 160 | 0.469 | TB example case for [1] |

Observation on triple beat cases:

- DC_3C_n5A and DC_3C_n28A requires further analysis for triple beat MSD and are proposed to be postponed to Release 18 in [2] (orange highlight in comment column)

- DC_3A_n41C is used as the example triple beat analysis for [1]

- DC_7C_n5A, DC_7C_n28A, DC_41C_n28A, DC_42C_n28A DC_8A_n79C and DC_28A_n7B are proposed to not require TB MSD thanks to band separation (yellow highlight in comment column)

- Note that in the last bullet although a first order triple beat issue exists, it is proposed that no MSD requirement is needed based on a band distance criterion. In the following chapter we formalise such criterion.

## Inter-band ENDC with two UL band and intra-band ULCA missing fallbacks

In way forward [1] the following criteria have been agreed in order to decide if a triple beat MSD may be required:

•	ULCA band (TDD or FDD) Parameters

•	Must have non-contiguous RB allocation in UL LCRB (ie. 1RB + 1RB)

•	Duplex mode (i.e.. FDD or TDD)

•	Spacing between RB allocations in each CC must equal the FDD Victim band duplex offset

•	UL CC1 BW and UL CC1 Fc

•	UL CC2 BW and UL CC2 Fc

•	Non-ULCA FDD band Parameters

•	DL Fc and UL Fc

•	TX LCRB

•	RX BW

•	TX BW

•	Triple Beat Order

•	1st order triple beat is for 3rd order non-linearity

•	In Release-17, capture only MSD test point due to 1st order triple beat interference

First it should be noted that although not specifically stated here some additional aspects should be considered:

- The ULCA band can be both bands (for example 3C_n7B exists in the specification as an UL configuration)

  - In this case two triple beat analysis may be required if both bands are FDD bands with two UL CCs

- In Release 17 when there is two band UL configuration, only contiguous ULCA is allowed in one or both UL bands

- Regarding “Spacing between RB allocations in each CC must equal the FDD Victim band duplex offset” this is to ensure that the triple beat product is optimally centred in the FDD DL channel when feasible. However, the triple beat issue is present for cases described in [3] where the ULCA allocation distance is equal to the distance between the FDD UL allocation and the closest edge of the DL channel:

  - where:

  - And where ULCAMBW is equivalent to ULCA Aggregated BW – CC1 GBLow – CC2 GBHigh since only contiguous ULCA is considered.

- The triple beat issue can arise to a third DL band if simultaneous Tx/Rx with both UL bands.

Beyond these already agreed to criteria, it is recognized by experts that when there is a significant distance between the two UL bands, there is typically sufficient combined rejection from the band group diplexing and the band filters as it has already been concluded in [1] for DC_8A_n79C.

This conclusion can be formalized in additional criteria

Proposal on band groups distance criteria:

- For the two-band case, triple beat MSD must be analysed only when the two simultaneous Tx/Rx UL bands are part of the same or adjacent band group as defined in Table 2

- For the three-band case, triple beat MSD must be analysed only when the third DL bands is simultaneous Tx/Rx with both UL bands and is part of the same or adjacent band group of one of the UL band as defined in Table 2

Table 2: Band group definition for adjacent band-group criterion

| FR1 band group range |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name | FR1-1 (LB) | FR1-2 (MB) | FR1-3 (HB) | FR1-4 (VHB) | FR1-5 (UHB) |
| Range (MHz) | 600-1000 | 1400-2200 | 2300-2700 | 3300-5000 | 5250-7125 |
| Duplex mode | Mostly FDD | Mostly FDD | FDD and TDD | TDD only | TDD only |

It should be noted that since the last two ranges are TDD only, there is no possible triple beat issue for:

- If UL band in FR1-4 ot FR1-5 is the non-ULCA band

- Between bands within FR1-4 and FR1-5

# Conclusions

In this contribution, provide further understanding on applicable criteria to detect potential triple beat issues in inter-band combinations with 2 band UL including intra-band contiguous ULCA and make the following proposal.

Proposal on band groups distance criteria:

- For the two-band case, triple beat MSD must be analysed only when the two simultaneous Tx/Rx UL bands are part of the same or adjacent band group as defined in Table 2

- For the three-band case, triple beat MSD must be analysed only when the third DL bands is simultaneous Tx/Rx with both UL bands and is part of the same or adjacent band group of one of the UL band as defined in Table 2

Table 2: Band group definition for adjacent band-group criterion

| FR1 band group range |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name | FR1-1 (LB) | FR1-2 (MB) | FR1-3 (HB) | FR1-4 (VHB) | FR1-5 (UHB) |
| Range (MHz) | 600-1000 | 1400-2200 | 2300-2700 | 3300-5000 | 5250-7125 |
| Duplex mode | Mostly FDD | Mostly FDD | FDD and TDD | TDD only | TDD only |

# References

- R4-2206394	WF on capturing triple beat MSD in 38.101-1 and 38.101-3,	Skyworks, Qualcomm, RAN4#102-e

- R4-2207989 Completion of inter-band band combinations with intra-band ULCA in UL configuration, Skyworks Solution Inc., RAN4#103-e

- R4-2202279 WF on triple beat evaluation and specification framework, Qualcomm Incorporated, Skyworks Solutions Inc., RAN4#101b-e