3GPP TSG-RAN WG4 Meeting # 103-e                                                            R4-2208136

Electronic Meeting, May 09 – May 20, 2022

Title: 	Discussion of test configuration for conducted and radiated

Source: 	CATT

Agenda item:	9.5.5.1.2

Document for:	Approval

- Introduction

This contribution provides our views on repeater test configuration. The analysis is based on the E-UTRA repeater test spec and BS test spec considering the NR RF repeater requirements refer both of them.

- Discussion

2.1 FR1 test configuration

For the test configuration part in test spec, E-UTRA repeater and BS have different contents. For E-UTRA repeater, there’re two clauses, i.e. Power supply options and Combining of repeaters, to describe repeater specific configurations. Our understanding is that the two clauses should be kept for NR repeater.

Proposal 1: The following two clauses from E-UTRA repeater test specification are kept for NR FR1 repeater.

- Power supply options

- Combining of repeaters

On the other hand, NR repeater reuses many NR BS requirements such as ACLR, non-contiguous spectrum requirements, multi-band requirements, therefore the test configurations similar with BS needs to be clarified. There’re totally 6 NRTC for BS, the NRTC2: Contiguous CA occupied bandwidth is not needed for repeater. So 5 NRTC for BS can be reused by NR repeater.

Proposal 2: FR1 BS NRTC1, NRTC3 - NRTC6 are reused by NR FR1 repeater.

If the proposal 2 can be agreed by the companies, some modifications for the NRTC are still needed. First is the name, the name like “NRREPTCx” or “NR_Repeater_TCx”, etc can be discussed and used. The other issue is to adapt some description for NR repeater.

The following is some preliminary thinking for TC1.

------------------------------------------------------------------

4.7.3	NRTC1: Contiguous spectrum operation

The purpose of test configuration NRTC1 is to test the repeater requirements according to the applicability in 4.8.3 and 4.8.4.

4.7.3.1	NRTC1 generation

NRTC1 shall be constructed on a per band basis using the following method:

-	Declared maximum repeater passband Bandwidth supported for contiguous spectrum operation (D.11) shall be used;

-	Select the carrier to be tested according to 4.7.2 and place it adjacent to the lower repeater passband edge. Place same signal adjacent to the upper repeater passband edge.

-

-	For transmitter tests, select as many carriers (according to 4.7.2) that the repeater supports within an operating band and fit in the rest of the declared maximum repeater passband RF Bandwidth (D.11). Place the carriers adjacent to each other starting from the upper repeater RF Bandwidth edge. The nominal channel spacing defined in TS 38.104 [2], clause 5.4.1 shall apply.

The test configuration should be constructed sequentially on a per band basis for all component carriers of the inter-band CA bands declared to be supported by the repeater and are transmitted using the same antenna connector. All configured component carriers are transmitted simultaneously in the tests where the transmitter should be ON.

4.7.3.2	NRTC1 power allocation

Set the power spectral density of each carrier to the same level so that the sum of the carrier powers equals the rated total output power (Prated,t,AC, or Prated,t,TABC, D.22) according to the manufacturer's declaration in clause 4.6.

--------------------------------------------------------------------------

The following is another example for NRTC3 modification.

--------------------------------------------------------------------------

4.7.5	NRTC3: Non-contiguous spectrum operation

The purpose of test configuration NRTC3 is to test the repeater requirements according to the applicability in 4.8.3 and 4.8.4.

4.7.5.1	NRTC3 generation

NRTC3 is constructed on a per band basis using the following method:

-	The NR repeater passband Bandwidth shall be the maximum NR repeater passband Bandwidth supported for non-contiguous spectrum operation (D.11). The NR repeater passband RF Bandwidth consists of one sub-block gap and two sub-blocks located at the edges of the declared maximum supported NR repeater passband RF Bandwidth (D.11).

-	Select the carrier to be tested according to 4.7.2. Place it adjacent to the upper NR repeater passband Bandwidth edge and another carrier (as described in 4.7.2) adjacent to the lower NR repeater passband Bandwidth edge.

-

-	The sub-block edges adjacent to the sub-block gap shall be determined using the specified Foffset_high and Foffset_low for the carriers adjacent to the sub-block gap.

4.7.5.2	NRTC3 power allocation

Set the power of each carrier to the same level so that the sum of the carrier powers equals the rated total output power (Prated,t,AC, or Prated,t,TABC, D.22) according to the manufacturer's declaration in clause 4.6.

2.2 FR2 test configuration

For FR2 test configuration, there’s no E-UTRA repeater test spec can be the reference, but aligning FR1 decisions can be the starting point. Taking BS FR2 test configuration as reference and considering the analysis in 2.1. It can be founded that the similar proposals can be made.

Proposal 3: The following clauses similar with FR1 repeater is created for FR2 repeater.

- Power supply options

- Combining of repeaters

Proposal 4: FR2 BS NRTC1, NRTC3 – NRTC5 are reused by NR FR2 repeater.

- Conclusion

This contribution provides our proposal for NR repeater. We have the following proposals.

Proposal 1: The following two clauses from E-UTRA repeater test specification are kept for NR FR1 repeater.

- Power supply options

- Combining of repeaters

Proposal 2: FR1 BS NRTC1, NRTC3 - NRTC6 are reused by NR FR1 repeater.

Proposal 3: The following clauses similar with FR1 repeater is created for FR2 repeater.

- Power supply options

- Combining of repeaters

Proposal 4: FR2 BS NRTC1, NRTC3 – NRTC5 are reused by NR FR2 repeater.

Some modifications for NRTC are needed. We provided some preliminary thinking for FR1 NRTC1 and NRTC3.

Reference

[1] TS 38.106 v1.0.0

[2] TS 38.141-1

[3] TS 38.143

[4] TS 38.141-2