3GPP TSG-RAN WG4 Meeting # 103-e                                                            R4-2208139

Electronic Meeting, May 09 – May 20, 2022

Title: 	Discussion of FR1 repeater declaration

Source: 	CATT

Agenda item:	9.5.5.2.2

Document for:	Approval

- Introduction

When repeater is tested, the declarations should be provided. This contribution provides the analysis for the repeater declarations.

- Discussion

Taking BS declaration as the starting point, the following are the repeater modifications compared with BS type 1-C declarations. The declaration numbers are renamed for repeater.

Table 4.6-1 Manufacturer declarations for repeater type 1-C requirements

| Declaration identifier | Declaration | Description |  |  |
| --- | --- | --- | --- | --- |
| D.1 | Repeater class | Repeater class of the repeater, declared as Wide Area repeater, Medium Range repeater, or Local Area repeater. The class for downlink and uplink can be declared separately. |  |  |
| D.2 | Operating bands and frequency ranges | List of NR operating band(s) supported by single-band connector(s) and/or multi-band connector(s) of the repeater and if applicable, frequency range(s) within the operating band(s) that the repeater can operate in. |  |  |
| D.3 | Spurious emission category | Declare the repeater spurious emission category as either category A or B with respect to the limits for spurious emissions, as defined in Recommendation ITU-R SM.329 [5]. |  |  |
| D.4 | Additional operating band unwanted emissions | The manufacturer shall declare whether the BS under test is intended to operate in geographic areas where the additional operating band unwanted emission limits defined in clause 6.6.4.5.6 apply. (Note 3, Note 6). |  |  |
| D.5 | Co-existence with other systems | The manufacturer shall declare whether the repeater under test is intended to operate in geographic areas where one or more of the systems GSM850, GSM900, DCS1800, PCS1900, UTRA FDD, UTRA TDD, E-UTRA, PHS and/or NR operating in another band are deployed. |  |  |
| D.6 | Co-location with other base stations, repeaters and IABs | The manufacturer shall declare whether the repeater under test is intended to operate co-located with Base Stations of one or more of the systems GSM850, GSM900, DCS1800, PCS1900, UTRA FDD, UTRA TDD, E-UTRA and/or NR operating in another band. |  |  |
| D.7 | Single band connector or multi-band connector | Declaration of the single band or multi-band capability of single band connector(s) or multi-band connector(s), declared for every connector. |  |  |
| D.8 | Contiguous or non-contiguous spectrum operation support | Ability to support contiguous or non-contiguous (or both) frequency distribution of carriers when operating multi-carrier. Declared per single band connector or multi-band connector, per operating band. |  |  |
| D.9 | Maximum repeater RF Bandwidth | Maximum repeater RF Bandwidth in the operating band for single-band operation. Declared per supported operating band, per antenna connector for repeater type 1-C, or TAB connector for repeater type 1-H. (Note 2) |  |  |
| D.10 | Maximum repeater RF Bandwidth for multi-band operation | Maximum Repeater RF Bandwidth for multi-band operation. Declared per supported operating band, per antenna connector. |  |  |
| D.11 | Passband RF bandwidth (BWpassband) | Total RF bandwidth BWpassband of transmitter and receiver, declared per the band combinations (D.27). |  |  |
| D.12 | Rated total output power (Prated,AC) | Conducted total rated output power. Declared per supported operating band, per antenna connector for repeater type 1-C. For multi-band connectors declared for each supported operating band This can be declared separately for downlink and uplink. |  |  |
| D.13 | Rated multi-band total output power, Prated,MB,TABC | Conducted multi-band rated total output power. Declared per supported operating band combinations, per multi-band connector. (Note 1) |  |  |
| D.14 | Operating band combination support | List of operating bands combinations supported by single-band connector(s) and/or multi-band connector(s) of the repeater. Declared per antenna connector for repeater type 1-C |  |  |
| NOTE 1: If a repeater is capable of 256QAM DL operation then two rated output power declarations may be made. One declaration is applicable when configured for 256QAM transmissions and the other declaration is applicable when not configured for 256QAM transmissions. NOTE 2: Parameters for contiguous or non-contiguous spectrum operation in the operating band are assumed to be the same unless they are separately declared. When separately declared, they shall still use the same declaration identifier. |  |  |  |  |

- Conclusion

This contribution provides our consideration of the repeater declarations. The modification compared with BS is provided in the contribution.

Reference

[1] RP-212129, “Revised WID on NR Repeaters”, Qualcomm, RAN#93e

[2] TS 38.106 v1.0.0

[3] TS 38.141-1

[4] TS 38.143