3GPP TSG-RAN4 Meeting #103-e	                                                        R4-2208209

Electronic meeting, May 9 – May 20, 2022

Title:	Test cases for NCSG

Source:	CATT

Agenda Item:	9.10.2.3

Document for:	Discussion

1 Introduction

In RAN4#102-e meeting, the core part of R17 measurement gap enhancement WI has been completed. According to the timeline, the performance part will start in this meeting and the test cases are needed for the defined requirements.

In this paper, we have some initial discussions on the test cases for NCSG and give our proposals.

2 Discussion

Based on the agreements in core part [1], the following requirements are defined for Pre-configured MG pattern:

- NCSG pattern and applicability

- Gap interruption (VIL related requirements)

- CSSF

- Intra-frequency measurement with NCSG (including the measurement on deactivated Scell)

- Inter-frequency measurement with NCSG

- Inter-RAT measurement with NCSG

- Scheduling restriction

- Impact on the L1 measurement

- Interruption for de-activated SCell measurement

Among the requirements above, we think only the measurement requirements and the interruption requirements need to be tested. The interruption requirements for NCSG are reflected in the VIL and the test can be included in the test cases of measurement requirements, i.e. the same configurations can be used.

And for the test cases of NCSG, the test configurations in existing test cases for the measurement with gap can be reused except that the gap configurations should be replaced by NCSG configurations.

Proposal 1: The following test cases for NCSG are expected:

| No. | Test case |
| --- | --- |
| 1 | TC for intra-frequency measurement with NCSG in FR1 |
| 2 | TC for inter-frequency measurement with NCSG in FR1 |
| 3 | TC for inter-RAT measurement with NCSG in FR1 |
| 4 | TC for intra-frequency measurement with NCSG in FR2 |
| 5 | TC for inter-frequency measurement with NCSG in FR2 |
| 6 | TC for inter-RAT measurement with NCSG in FR2 |
| Note: VIL requirements test are also included in each test case. |  |

Proposal 2: For the test cases of NCSG, the test configurations in existing test cases for the measurement with gap can be reused except that the gap configurations should be replaced by NCSG configurations.

3 Summary

In this paper, we have some initial discussions on the test cases for NCSG and the following proposals are given：

Proposal 1: The following test cases for NCSG are expected:

| No. | Test case |
| --- | --- |
| 1 | TC for intra-frequency measurement with NCSG in FR1 |
| 2 | TC for inter-frequency measurement with NCSG in FR1 |
| 3 | TC for inter-RAT measurement with NCSG in FR1 |
| 4 | TC for intra-frequency measurement with NCSG in FR2 |
| 5 | TC for inter-frequency measurement with NCSG in FR2 |
| 6 | TC for inter-RAT measurement with NCSG in FR2 |
| Note: VIL requirements test are also included in each test case. |  |

Proposal 2: For the test cases of NCSG, the test configurations in existing test cases for the measurement with gap can be reused except that the gap configurations should be replaced by NCSG configurations.

4 References

- R4-2120305 WF on R17 NR MG enhancement - NCSG, Apple, RAN4#101-e