3GPP TSG-RAN4 Meeting #103-e	                                                           R4-2208219

Electronic meeting, May 9 – May 20, 2022

Title:	Discussion on the performance requirements for the PRS measurement in RRC_INACTIVE state

Source:	CATT

Agenda Item:	9.20.2.3

Document for:	Discussion

1 Introduction

In RAN4#102-e meeting, the core part of R17 ePOS WI has been completed. And according to the timeline, the performance part will start from RAN4#103e meeting. In this contribution, we provide the general discussion on the performance part related to the PRS measurement in RRC_INACTIVE state and give our proposals.

2 Discussion

2.1 Accuracy requirements

In last meeting, the measurement period requirements for the PRS measurement in RRC_INACTIVE state were specified and the accuracy requirements need to be defined in this meeting. For the UL measurement, it was agreed that the existing accuracy requirements of gNB Rx-Tx time difference and SRS-RSRP measurement can be reused in RRC_INACTIVE state. Besides we think the the report mapping can also be reused. In R17 ePOS, the SRS-RSRPP measurement is introduced and based on our companion discussion paper [2], we think the SRS-RSRPP accuracy requirements need to be defined. And if defined, the SRS-RSRPP measurement accuracy requirements can be applied for both RRC_INACTIVE and RRC_CONNECTED state.

For the DL measurement, similar as the UL measurement, the accuracy requirements for RSTD/PRS-RSRP/UE Rx-Tx time difference/PRS-RSRPP measurement defined in RRC_CONNECTED state can also be used in RRC_INACTIVE state.

Proposal 1: The existing report mapping of gNB Rx-Tx time difference and SRS-RSRP measurement can be reused in RRC_INACTIVE state.

Proposal 2: The SRS-RSRPP measurement accuracy requirements and report mapping if defined can be applied for both RRC_CONNECTED state and RRC_INACTIVE state.

Proposal 3: The accuracy requirements and report mapping for RSTD/PRS-RSRP/UE Rx-Tx time difference/PRS-RSRPP measurement can be reused in RRC_INACTIVE state.

2.2 Test cases

After defining the measurement period requirements and accuracy requirements for the PRS measurement in RRC_INACTIVE state, the corresponding test cases need to be specified. Based on the agreements in last meeting [1] and the discussion in section 2.1 above, the following test cases are needed and the test configurations for the PRS measurement with gaps in RRC_CONNECTED state can be reused separately for each measurement except that the gap configuration should be replaced by DRX configuration.

Proposal 4: The following test cases for the PRS measurement in RRC_INACTIVE are needed:

- TC for RSTD measurement period requirements in RRC_INACTIVE state

- TC for RSTD measurement accuracy requirements in RRC_INACTIVE state

- TC for PRS-RSRP measurement period requirements in RRC_INACTIVE state

- TC for PRS-RSRP measurement accuracy requirements in RRC_INACTIVE state

- TC for UE Rx-Tx time difference measurement period requirements in RRC_INACTIVE state

- TC for UE Rx-Tx time difference measurement accuracy requirements in RRC_INACTIVE state

- TC for PRS-RSRPP measurement requirements in RRC_INACTIVE state

- TC for PRS-RSRPP measurement accuracy requirements in RRC_INACTIVE state

Proposal 5: The test configurations for the PRS measurement with gaps in RRC_CONNECTED state can be reused except that the gap configuration should be replaced by DRX configuration.

3 Summary

In this paper, we discuss the performance requirements related to the PRS measurement in RRC_INACTIVE state and the following proposals are given:

Proposal 1: The existing report mapping of gNB Rx-Tx time difference and SRS-RSRP measurement can be reused in RRC_INACTIVE state.

Proposal 2: The SRS-RSRPP measurement accuracy requirements and report mapping if defined can be applied for both RRC_CONNECTED state and RRC_INACTIVE state.

Proposal 3: The accuracy requirements and report mapping for RSTD/PRS-RSRP/UE Rx-Tx time difference/PRS-RSRPP measurement can be reused in RRC_INACTIVE state.

Proposal 4: The following test cases for the PRS measurement in RRC_INACTIVE are needed:

- TC for RSTD measurement period requirements in RRC_INACTIVE state

- TC for RSTD measurement accuracy requirements in RRC_INACTIVE state

- TC for PRS-RSRP measurement period requirements in RRC_INACTIVE state

- TC for PRS-RSRP measurement accuracy requirements in RRC_INACTIVE state

- TC for UE Rx-Tx time difference measurement period requirements in RRC_INACTIVE state

- TC for UE Rx-Tx time difference measurement accuracy requirements in RRC_INACTIVE state

- TC for PRS-RSRPP measurement requirements in RRC_INACTIVE state

- TC for PRS-RSRPP measurement accuracy requirements in RRC_INACTIVE state

Proposal 5: The test configurations for the PRS measurement with gaps in RRC_CONNECTED state can be reused except that the gap configuration should be replaced by DRX configuration.

- References

- R4-2206997, WF on NR Positioning Enhancements (Part 2), CATT, RAN4#102-e

- R4-2208218, Discussion on performance requirements for latency reduction of positioning measurement, CATT, RAN4#103-e