3GPP TSG-RAN WG4 Meeting #103-e	R4-2208329

Electronic Meeting, 9th – 20th May 2022

Agenda Item:	9.15.10.2.3

Source:	Ericsson

Title:	Simulation results for SDR requirements in 52.6 – 71 GHz band

Document for:	Information

# 1	Introduction

In this paper, we present some initial simulation results to support our view on the SDR requirements in the 52.6 – 71 GHz band. It is worth mentioning that no prior agreements have been achieved in this item [1], and that the proposed results can present a good material for further discussions when defining the SDR requirements.

# 2	Simulation results and discussions

In this section, we present the simulation results following the SDR test steps as per TS 38.101-4 recommendations, where the purpose is to verify that the Layer 1 and Layer 2 correctly process in a sustained manner the received packets corresponding to the maximum data rate indicated by UE capabilities.

## 2.1	Simulation setup

According to TS38.101-4, the existing MCS index table has already covered the configuration applicable for UEs such as rank 1/2 and QPSK/16QAM/64QAM. RAN4 only need to evaluate the applicability rule for FR2-2 where rank 1 is supported as shown in Table 1 (based on MCS index Table in the Appendix, in clause 5.1.3.1 of TS 38.214).

| Received antenna | Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- | --- |
| 2Rx UE | 1 | 6 | 1 | 27 |
|  |  |  | 0.8 | 23 |
|  |  |  | 0.75 | 22 |
|  |  |  | 0.4 | 14 |
|  |  | 4 | 1 | 16 |
|  |  |  | 0.8 | 16 |
|  |  |  | 0.75 | 16 |
|  |  |  | 0.4 | 10 |
|  |  | 2 | 1 | 9 |
|  |  |  | 0.8 | 9 |
|  |  |  | 0.75 | 9 |
|  |  |  | 0.4 | 4 |

## 2.2	Simulation results

We present the SDR performance in terms of normalized throughput as function of SNR (dB).

Fig. 1 PDSCH simulation results depicting the normalized throughput as function of SNR to determine the practical MCS for SDR requirements

Fig. 1 depicts the PDSCH normalized throughput as function of the SNR according to parameters in Table 1, where the SNR values required to achieve 85% of PDSCH peak throughput under AWGN conditions are summarized in the following table.

| Received antenna | Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS | SNR (dB) @ 85% max Thpt |
| --- | --- | --- | --- | --- | --- |
| 2Rx UE | 1 | 6 | 1 | 27 | 13.21 |
|  |  |  | 0.8 | 23 | 8.57 |
|  |  |  | 0.75 | 22 | 7.67 |
|  |  |  | 0.4 | 14 | 1.25 |
|  |  | 4 | 1 | 16 | 2.94 |
|  |  |  | 0.4 | 10 | -2.05 |
|  |  | 2 | 1 | 9 | -2.64 |
|  |  |  | 0.4 | 4 | -7.64 |

Considering the simulations results, we can note that an SNR of 7.67 dB is needed to achieve 85% of the peak throughput in AWGN conditions when using MCS 22.

Observation 1: An SNR of 7.67 dB is needed to achieve 85% of the peak throughput in AWGN conditions when using MCS 22.

# 6	Summary

In this paper, we provide some initial simulation results to assess the SDR performance.

Observation 1: An SNR of 7.67 dB is needed to achieve 85% of the peak throughput in AWGN conditions when using MCS 22.

# References

- R4-2207223, “WF on demodulation performance requirements definition for 52.6 - 71 GHz”, Intel.

# Appendix

Table 5.1.3.1-1: MCS index table 1 for PDSCH

| MCS Index IMCS | Modulation Order Qm | Target code Rate R x [1024] | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | 2 | 120 | 0.2344 |
| 1 | 2 | 157 | 0.3066 |
| 2 | 2 | 193 | 0.3770 |
| 3 | 2 | 251 | 0.4902 |
| 4 | 2 | 308 | 0.6016 |
| 5 | 2 | 379 | 0.7402 |
| 6 | 2 | 449 | 0.8770 |
| 7 | 2 | 526 | 1.0273 |
| 8 | 2 | 602 | 1.1758 |
| 9 | 2 | 679 | 1.3262 |
| 10 | 4 | 340 | 1.3281 |
| 11 | 4 | 378 | 1.4766 |
| 12 | 4 | 434 | 1.6953 |
| 13 | 4 | 490 | 1.9141 |
| 14 | 4 | 553 | 2.1602 |
| 15 | 4 | 616 | 2.4063 |
| 16 | 4 | 658 | 2.5703 |
| 17 | 6 | 438 | 2.5664 |
| 18 | 6 | 466 | 2.7305 |
| 19 | 6 | 517 | 3.0293 |
| 20 | 6 | 567 | 3.3223 |
| 21 | 6 | 616 | 3.6094 |
| 22 | 6 | 666 | 3.9023 |
| 23 | 6 | 719 | 4.2129 |
| 24 | 6 | 772 | 4.5234 |
| 25 | 6 | 822 | 4.8164 |
| 26 | 6 | 873 | 5.1152 |
| 27 | 6 | 910 | 5.3320 |
| 28 | 6 | 948 | 5.5547 |
| 29 | 2 | reserved |  |
| 30 | 4 | reserved |  |
| 31 | 6 | reserved |  |