3GPP TSG-RAN WG4 Meeting #103-e	R4-2208330

Electronic Meeting, 9th – 20th May 2022

Agenda Item:	9.15.10.2.4

Source:	Ericsson

Title:	CSI reporting requirements in 52.6 – 71 GHz band

Document for:	Discussion

# 1	Introduction

In this paper, we present our view on the CSI reporting requirements in 52.5 GHz – 71 GHz band, where the related aspects have been gathered in the WF [1].

It is worth reminding that the channel model and the corresponding delay spread have been agreed in RAN4 meeting #102-e,

The channel models have been decided to be TDLA and TDLD, where both using the following delay spread RMS: - SCS = 120 KHz, 10 ns. - SCS = 480 KHz, 10 ns / 5 ns. - SCS = 960 KHz, 10 ns / 5 ns.

while several CSI reporting aspects still need to be discussed, such that

Issue 1-3-3: Max Doppler frequency Consider 3 km/h UE speed (200 Hz). FFS on higher UE speed. - Option 1: 10 km/h (650 Hz) - Option 2: 30 km/h (2000 Hz) Issue 3-1-1: SCS for DL requirements definition Consider the following SCS for DL requirements definition: - 120, 480 kHz - FFS 960 kHz Issue 3-1-2: CBW for DL requirements definition 120 kHz: - 100, 400 MHz 480 MHz: - 400 MHz - FFS on 1600 MHz 960 kHz: - 400 MHz - FFS on 2000 MHz Issue 3-3-7: MCS, modulation order for PDSCH requirements Define PDSCH requirements with MCS 4, MCS 13 and MCS [22] - Other MCS values are not precluded Issue 3-3-8: Rank Define PDSCH requirements with: - Rank 1 - FFS FD-OCC is not applied to all the antenna ports for DMRS - FFS Rank 2

# 2	CSI Reporting Requirements

RAN4 has defined CQI, PMI and RI reporting requirements in Rel-15. For Rel-17 and FR2-2 band, we will follow RAN1 where PDSCH MIMO rank has been agreed to be equal to 1. Consequently, we propose to define channel quality indicator (CQI) reporting requirements only, since we do not need to verify the rank indicator (RI). Furthermore, the precoding matrix indicator (PMI) reporting’s ports could be FFS.

Regarding the CQI reporting, we propose to define both the static condition and fading condition requirements for UE with 2Rx. Since the existing CQI reporting tests verifies the CQI using 256 QAM for PDSCH FR2-1 (see 38.101-4 Table 8.1.1.3-1), we need to define the CQI reporting requirements using 64QAM for 2Rx UE.

## 2.1 CQI reporting under AWGN conditions

The reporting accuracy of the CQI under static conditions is determined by the reporting variance and the BLER performance using the transport format indicated by the reported CQI median.

Proposal 1: Define the CQI reporting definition test for 2Rx UE with CQI table 1 (64QAM) by reusing the existing test setup and metrics for SCS 120 KHz.

Proposal 2: For SCS 480 KHz in static conditions, it is worth investigating whether the CSI-RS periodicity/offset of 8/1 slot(s) should be revisited or not.

- Option 1: Keep periodicity/offset = 8/1 slot and follow TDD UL-DL pattern FR2.120-2 as:

FR2-2 TDD 480 KHz: 11D S 4U, S: 2D:12G:0U

- Option 2: Define new CSI-RS periodicity/offset and eventually a new TDD UL-DL pattern

Observation 1: For SCS 480 KHz in static conditions, it is worth investigating whether the CQI delay should be maintained at 8.375 or not.

## 2.2 CQI reporting under fading conditions

For the CQI reporting under fading condition, we should evaluate whether the maximum CBW of 100 MHz for FR2-2 is enough to define a wideband CQI reporting test.

Proposal 3: Define the wideband CQI reporting under fading condition for 2Rx UE with CQI table 1 (64QAM) by reusing the existing test setup and metrics for SCS 120 KHz.

Proposal 4: For SCS 480 KHz in fading conditions, it is worth investigating whether the CSI-RS periodicity/offset of 8/1 slot(s) should be revisited or not.

- Option 1: Keep periodicity/offset = 8/1 slot and follow TDD UL-DL pattern FR2.120-2 as:

FR2-2 TDD 480 KHz: 11D S 4U, S: 2D:12G:0U

- Option 2: Define new CSI-RS periodicity/offset and eventually a new TDD UL-DL pattern

Observation 2: For SCS 480 KHz in static conditions, it is worth investigating whether the CQI delay should be maintained at 1.375 or not.

# 3	Specification structure

Table 1	Specification structure for UE CSI reporting requirements in FR2-2 (Radiated requirements).

| Section number | Section name | Note |
| --- | --- | --- |
| 8.2 | Reporting of Channel Quality Indicator (CQI) |  |
| 8.2.2 | 2RX requirements |  |
| 8.2.2.2 | TDD |  |
| 8.2.2.2.1 | CQI reporting definition under AWGN | Updates on FR2-2 |
| 8.2.2.2.2 | CQI reporting under fading conditions | Updates on FR2-2 |

# 4	Summary

In this paper, we tried to contribute with the following proposals for further discussions:

Proposal 1: Define the CQI reporting definition test for 2Rx UE with CQI table 1 (64QAM) by reusing the existing test setup and metrics for SCS 120 KHz.

Proposal 2: For SCS 480 KHz in static conditions, it is worth investigating whether the CSI-RS periodicity/offset of 8/1 slot(s) should be revisited or not.

- Option 1: Keep periodicity/offset = 8/1 slot and follow TDD UL-DL pattern FR2.120-2 as:

FR2-2 TDD 480 KHz: 11D S 4U, S: 2D:12G:0U

- Option 2: Define new CSI-RS periodicity/offset and eventually a new TDD UL-DL pattern

Observation 1: For SCS 480 KHz in static conditions, it is worth investigating whether the CQI delay should be maintained at 8.375 or not.

Proposal 3: Define the wideband CQI reporting under fading condition for 2Rx UE with CQI table 1 (64QAM) by reusing the existing test setup and metrics for SCS 120 KHz.

Proposal 4: For SCS 480 KHz in fading conditions, it is worth investigating whether the CSI-RS periodicity/offset of 8/1 slot(s) should be revisited or not.

- Option 1: Keep periodicity/offset = 8/1 slot and follow TDD UL-DL pattern FR2.120-2 as:

FR2-2 TDD 480 KHz: 11D S 4U, S: 2D:12G:0U

- Option 2: Define new CSI-RS periodicity/offset and eventually a new TDD UL-DL pattern

Observation 2: For SCS 480 KHz in static conditions, it is worth investigating whether the CQI delay should be maintained at 1.375 or not.

# References

- R4-2207223, “WF on demodulation performance requirements definition for 52.6 - 71 GHz”, Intel.

# Appendix

## A.1	CQI Tables

We consider Table A.4-1 (TS 38.101-4) mapping CQI index to the information bit payload, given by

Table A.4-1: Mapping of CQI Index to Information Bit payload (CQI table 1)

| TBS Scheme |  |  |  | TBS.1-1 | TBS.1-2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MCS table |  |  |  | 64QAM |  |  |  |  |  |
| Number of allocated PDSCH resource blocks |  |  |  | 66 | 66 | 264 (New) |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  | 12 | 12 | 12 |  |  |  |
| Number of PDSCH MIMO layers |  |  |  | 1 | 2 | 1 |  |  |  |
| Number of DMRS REs (Note 1) |  |  |  | 24 | 24 | TBD |  |  |  |
| Overhead for TBS determination |  |  |  | 6 | 6 | TBD |  |  |  |
| Available RE-s |  |  |  | 7590 | 7590 | TBD |  |  |  |
| CQI index | Spectral efficiency | MCS index | Modulation | Information Bit Payload per Slot |  |  |  |  |  |
| 0 | OOR | OOR | OOR | N/A | N/A |  |  |  |  |
| 1 | 0.2344 | 0 | QPSK | 1800 | 3624 |  |  |  |  |
| 2 | 0.2344 | 0 |  | 1800 | 3624 |  |  |  |  |
| 3 | 0.3770 | 2 |  | 2856 | 5640 |  |  |  |  |
| 4 | 0.6016 | 4 |  | 4480 | 8968 |  |  |  |  |
| 5 | 0.8770 | 6 |  | 6528 | 13064 |  |  |  |  |
| 6 | 1.1758 | 8 |  | 8712 | 17928 |  |  |  |  |
| 7 | 1.4766 | 11 | 16QAM | 11016 | 22032 |  |  |  |  |
| 8 | 1.9141 | 13 |  | 14343 | 28680 |  |  |  |  |
| 9 | 2.4063 | 15 |  | 17928 | 35856 |  |  |  |  |
| 10 | 2.7305 | 18 | 64QAM | 20496 | 40976 |  |  |  |  |
| 11 | 3.3223 | 20 |  | 25104 | 50184 |  |  |  |  |
| 12 | 3.9023 | 22 |  | 29192 | 58384 |  |  |  |  |
| 13 | 4.5234 | 24 |  | 33816 | 67584 |  |  |  |  |
| 14 | 5.1152 | 26 |  | 38936 | 77896 |  |  |  |  |
| 15 | 5.5547 | 28 |  | 42016 | 83976 |  |  |  |  |

where the CQI indexes can be mapped to the corresponding MCS as given by Table 5.1.3.1-1 (TS 38.214)

| CQI Index | MCS Index IMCS | Modulation Order Qm | Target code Rate R x [1024] | Spectral efficiency |
| --- | --- | --- | --- | --- |
| 1 - 2 | 0 | 2 | 120 | 0.2344 |
| 3 | 2 | 2 | 193 | 0.3770 |
| 4 | 4 | 2 | 308 | 0.6016 |
| 5 | 6 | 2 | 449 | 0.8770 |
| 6 | 8 | 2 | 602 | 1.1758 |
| 7 | 11 | 4 | 378 | 1.4766 |
| 8 | 13 | 4 | 490 | 1.9141 |
| 9 | 15 | 4 | 616 | 2.4063 |
| 10 | 18 | 6 | 466 | 2.7305 |
| 11 | 20 | 6 | 567 | 3.3223 |
| 12 | 22 | 6 | 666 | 3.9023 |
| 13 | 24 | 6 | 772 | 4.5234 |
| 14 | 26 | 6 | 873 | 5.1152 |
| 15 | 28 | 6 | 948 | 5.5547 |

## A.2	CQI reporting in static condition

### A.2.1	Test metric

- The reported CQI value according to the reference channel shall be in the range of ±1 of the reported median more than 90% of the time.

- If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 0.1, then the BLER using the transport format indicated by the (median CQI+1) shall be greater than 0.1. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 0.1, then the BLER using transport format indicated by (median CQI-1) shall be less than or equal to 0.1.

| For FR2-2 Parameter |  |  | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 100 | 400 | 400 | 1600 |
| Subcarrier spacing |  |  | kHz | 120 | 120 | 480 | 480 |
| Duplex Mode |  |  |  | TDD |  |  |  |
| TDD Slot Configuration |  |  |  | FR2.120-2 Annex A.1.3 |  |  |  |
| SNRBB |  |  | dB | TBD |  |  |  |
| Propagation channel |  |  |  | AWGN |  |  |  |
| Antenna configuration |  |  |  | 2x2 with static channel specified in Annex B.1 |  |  |  |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |  |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | 8 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  | 13 |  |  |  |
|  | CSI-RS periodicity and offset |  | slot | 8/1 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 2 |  |  |  |
|  | CDM Type |  |  | fd-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | 6 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 8/1 |  |  |  |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic |  |  |  |
|  | CSI-IM RE pattern |  |  | 1 |  |  |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (8, 13) |  |  |  |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 8/1 |  |  |  |
| ReportConfigType |  |  |  | Periodic |  |  |  |
| CQI-table |  |  |  | Table 1 |  |  |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  | RB | 8 |  |  |  |
| csi-ReportingBand |  |  |  | 111111111 |  |  |  |
| CSI-Report periodicity and offset |  |  | slot | 8/3 |  |  |  |
| aperiodicTriggeringOffset |  |  |  | Not configured |  |  |  |
| Codebook configuration |  | Codebook Type | typeI-SinglePanel | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured |  |  |  |
|  |  | CodebookSubsetRestriction |  | 010000 |  |  |  |
|  |  | RI Restriction |  | N/A |  |  |  |
| Physical channel for CSI report |  |  |  | PUCCH |  |  |  |
| CQI/RI/PMI delay |  |  | ms | 8.375 |  |  |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |  |  |
| Measurement channel |  |  |  | TBD |  |  |  |

## A.3	CQI reporting in fading condition

### A.3.1	Test metric

- A CQI index not in the set {median CQI -1, median CQI, median CQI +1} shall be reported at least α% of the time.

- The ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index and that obtained when transmitting a fixed transport format configured according to the wideband CQI median shall be ≥ γ.

- When transmitting the transport format indicated by each reported wideband CQI index, the average BLER for the indicated transport formats shall be greater than or equal to [0.02].

| Parameter |  |  | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 100 | 400 | 400 | 1600 |
| Subcarrier spacing |  |  | kHz | 120 | 120 | 480 | 480 |
| Duplex Mode |  |  |  | TDD |  |  |  |
| TDD Slot Configuration |  |  |  | FR2.120-2 Annex A.1.3 |  |  |  |
| SNRBB |  |  | dB | TBD |  |  |  |
| Propagation channel |  |  |  | TDL-A/D (10,10/5) ns |  |  |  |
| Antenna configuration |  |  |  | 2x2 ULA Low |  |  |  |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |  |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | 8 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  | 13 |  |  |  |
|  | CSI-RS periodicity and offset |  | slot | 8/1 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Aperiodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 2 |  |  |  |
|  | CDM Type |  |  | fd-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | 6 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | Not configured |  |  |  |
|  | aperiodicTriggeringOffset |  |  | 0 |  |  |  |
| CSI-IM configuration | CSI-IM resource Type |  |  | Aperiodic |  |  |  |
|  | CSI-IM RE pattern |  |  | 1 |  |  |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (8, 13) |  |  |  |
|  | CSI-IM timeConfig periodicity and offset |  | slot | Not configured |  |  |  |
| ReportConfigType |  |  |  | Aperiodic |  |  |  |
| CQI-table |  |  |  | Table 1 |  |  |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  | RB | 8 |  |  |  |
| csi-ReportingBand |  |  |  | 111111111 |  |  |  |
| CSI-Report periodicity and offset |  |  | slot | Not configured |  |  |  |
| Aperiodic Report Slot Offset |  |  |  | 6 |  |  |  |
| CSI request |  |  |  | 1 in slots i, where mod(i, 8) = 1, otherwise it is equal to 0 |  |  |  |
| reportTriggerSize |  |  |  | 1 |  |  |  |
| CSI-AperiodicTriggerStateList |  |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |  |  |  |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured |  |  |  |
|  |  | CodebookSubsetRestriction |  | 000001 |  |  |  |
|  |  | RI Restriction |  | N/A |  |  |  |
| Physical channel for CSI report |  |  |  | PUSCH |  |  |  |
| CQI/RI/PMI delay |  |  | ms | 1.375 |  |  |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |  |  |
| Measurement channel |  |  |  | TBD |  |  |  |