3GPP TSG-RAN WG4 Meeting # 103-e	R4-2208340

# Electronic Meeting, May 9 – May 20, 2022

Source: 	MediaTek Inc.

Title: 	Discussion on UE REFSENS for 6GHz licensed band

Agenda Item:	8.1.3

Document for:	Approval

Introduction

Regarding WF on UE RF requirements for the 6GHz licensed band [1] from RAN4#102-e, there is open discussion on REFSENS requirements. In the contribution, the evaluation of UE REFSENS is provided.

Discussion

In RAN4#102-e meeting, WF on UE REFSENS for the 6GHz licensed band [1] is shown below.

- Reference sensitivity Way forward: Decide among 5 options - Option 1: 11.5 dB NF - Option 2: 10.5 dB NF - Option 3: 10 dB NF - Option 4: 12 dB~13.5 dB NF - Option 5: FFS

The 6GHz licensed band is tentatively identified as n104. In the approved draft CR[2], the configuraion of SCS and CBW is shown as follows.

NR operating band; Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high; Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high; Duplex Mode. [n104]; 6425 MHz – 7125 MHz; 6425 MHz – 7125 MHz; TDD. NR Band; SCS (kHz); UE Channel bandwidth (MHz). 20; 25; 30; 35; 40; 45; 50; 60; 70; 80; 90; 100. [n104]; 15; 20; 30; 40; 50. 30; 20; 30; 40; 50; 60; 70; 80; 90; 100. 60; 20; 30; 40; 50; 60; 70; 80; 90; 100. Operating band / SCS / Channel bandwidth / REFSENS. Operating band; SCS kHz; Channel bandwidth (MHz); REFSENS (dBm)8; Duplex Mode. [n104]; 15; 20, 30, 40, 50; TBD; TDD. 30; 20, 30, 40, 50, 60, 70, 80, 90, 100; TBD. 60; 20, 30, 40, 50, 60, 70, 80, 90, 100; TBD.

UE REFSENS:

Based on TS 38.101-1, REFSENS requirements of n1 and n77 are shown in Table 1. The UE REFSENS level increases from middle band to high band.

| NR band | SCS (kHz) | REFSENS of 20MHz CBW (dBm) | Band’s Highest RX Frequency (MHz) |
| --- | --- | --- | --- |
| n1 | 15 | -93.8 | 2170 |
| n77 | 15 | -92.2 | 4200 |
| [n104] | 15 | FFS | 7125 |

Table 2-1 UE REFSENS level from low to high band

Proposal 1: From UE implementation perspective, UE NF level increases from 4200MHz to 7125MHz. NR band’s highest-channel frequency should be considered for defining REFSENS. It is better not to reuse the same REFSENS as n77 for the NR licensed band 6425 – 7125 MHz due to increased NF.

Regarding NR licensed band 6425 – 7125 MHz, the FE architecture is shown below.

Figure 2-1 FE architecture

Proposal 2: Based on FE architecture in Figure 2-1 and 6GHz band’s higher operation frequency, the 6GHz band NF is higher than band n79’s NF. It is better to combine options 1 and 4 in WF[1] to give the NF of 11.5~13.5dB as a starting point for REFSENS in NR licensed band 6425 – 7125 MHz.

Conclusion

In the contribution, the evaluated UE REFSENS requirement for NR licensed band 6425 – 7125 MHz is provided.

UE REFSENS:

Proposal 1: From UE implementation perspective, UE NF level increases from 4200MHz to 7125MHz. NR band’s highest-channel frequency should be considered for defining REFSENS. It is better not to reuse the same REFSENS as n77 for the NR licensed band 6425 – 7125 MHz due to increased NF.

Proposal 2: Based on FE architecture in Figure 2-1 and 6GHz band’s higher operation frequency, the 6GHz band NF is higher than band n79’s NF. It is better to combine options 1 and 4 in WF[1] to give the NF of 11.5~13.5dB as a starting point for REFSENS in NR licensed band 6425 – 7125 MHz.

- References

[1] R4-2206373, “WF on UE RF requirements for the 6 GHz licensed band”, Qualcomm Inc.

[2] R4-2206576, “Introduction of NR licensed band 6425 – 7125 MHz”, Qualcomm Inc.