3GPP TSG-RAN WG4 Meeting # 103-e	R4-2208378

# Electronic Meeting, May 9 – May 20, 2022

Source: 	MediaTek Inc.

Title: 	Discussion on UE RX REFSENS and OOBB for band n256

Agenda Item:	9.12.5.2

Document for:	Approval

Introduction

Regarding WF on NTN UE RF [1] from RAN4#102-e, there are still band n256 REFSENS and OOBB requirements unfinished. In the contribution, requirements of UE band n256 REFSENS and OOBB are provided separately.

Discussion

In RAN4#102-e meeting, regarding NR satellite band n256, decision about n256 REFSENS [1][2] is in square brackets. The status is shown below.

NTN UE Rx requirement: Duplexer assumption for n256 with below candidate options - Option 1: dedicated 30MHz - Option 2: reusing n65 REFSENS for n256 Agreement: Keep UE implementation freedom for both options and define RAN4 RF requirements compatible with above options. For REFSENS, choosing the minimum requirements among option 1 and option 2. NTN UE Rx requirement for REFSENS with [ ] into TP: Operating band / SCS / Channel bandwidth. Operating Band; SCS kHz; 5 MHz (dBm); 10 MHz (dBm); 15 MHz (dBm); 20 MHz (dBm); 25 MHz (dBm); 30 MHz (dBm); 35 MHz (dBm); 40 MHz (dBm); 45 MHz (dBm); 50 MHz (dBm). n256; 15; -99.5; -96.3; -94.5; -93.3. 30; -96.6; -94.6; -93.5. 60; -97.0; -94.9; -93.7. OOBB requirements FFS for OOBB requirements for band n256

NTN UE n256 REFSENS:

In [1][2], there are two duplexer assumption for n256. To design dedicated 30MHz filter or reuse n65 filter is discussed. During GTW, it is agreed to keep UE implementation freedom for both options.

Observation 1: In [1][2], there are two duplexers assumption for n256. It is agreed to keep UE implementation freedom for both options and choose the minimum requirements among option 1 and option 2 for REFSENS.

Regarding option 2, band n256 REFSENS is shown Table 2-1.

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
| n256 | 15 | -99.5 | -96.3 | -94.5 | -93.3 |  |  |  |  |  |  |
|  | 30 |  | -96.6 | -94.6 | -93.5 |  |  |  |  |  |  |
|  | 60 |  | -97.0 | -94.9 | -93.7 |  |  |  |  |  |  |

Table 2-1

Proposal 1: REFSENS is normally minimum requirement. Regarding two feasible duplexers for band n256, REFSENS in Table 2-1 shall not preclude better UE implementation performance. To remove square brackets of TP REFSENS specification.

OOBB requirements for band n256:

In RAN4#102-e, regarding OOBB requirements, to put [] into TP was agreed.

|  | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| n255, n256 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: For band n256 in Range 2 requirement, the applicable lower frequency range should be modified as -[145] < f – FDL_low ≤ -60. NOTE 2: For band n256 in Range 3 requirement, the applicable lower frequency range should be modified as 1 ≤ f ≤ FDL_low – [145] NOTE 3: For band n256 in Range 2 requirement, the Pinterferer should be modified as [-30] NOTE 4: For band n256 in Range 3 requirement, the Pinterferer should be modified as [-15] |  |  |  |  |  |

Table 2-2

When n65 hardware is reused for band n256, to modify frequency range of OOBB range 3 is needed. For reference, a similar scenario using n96 hardware for unlicensed band n102 is shown in Table 2-3.

| Operating band | Parameter | Unit | Range1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| n46, n96, n102 | Finterferer (CW) | MHz | N/A | -200 < f – FDL_low ≤ -3*CBW or 3*CBW ≤ f – FDL_high < 200 | 1 ≤ f ≤ FDL_low – MAX(200,3*CBW) or FDL_high + MAX(200,3*CBW) ≤ f ≤ 12750 |
| NOTE 1: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 4200 MHz. For band n102 the power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 4200 MHz except for the range FDL_high + MAX(200,3*CBW) < f ≤ MAX(375,3*CBW) MHz in which the said level shall be modified to -33 dBm. For band n102 the power level of the interferer (PInterferer) for Range 2 shall be modified to -33 dBm for FInterferer in the range 3*CBW ≤ f – FDL_high < 200 MHz. NOTE 2: CBW denotes the channel bandwidth of the wanted signal |  |  |  |  |  |

Table 2-3

Observation 2: Regarding n65 hardware for band n256, similar requirements about reusing n96 hardware for band n102 can be studied. As shown in Table 2-3, Pinterferer for n102 OOBB range2 is modified to be -33dBm. Regarding modified Pinterferer of -33dBm in n102 OOBB range 3, the modified and extended frequency range is FDL_high + MAX(200,3*CBW) < f ≤ MAX(375,3*CBW) MHz.

Proposal 2: Based on observation 2 and evaluation, regarding n256 OOBB range 2 and 3 requirements, it is feasible to remove Note 3 and 4, and the square brackets of Note 1 and 2 as shown Table 2-4.

|  | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| n255, n256 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: For band n256 in Range 2 requirement, the applicable lower frequency range should be modified as -145 < f – FDL_low ≤ -60 NOTE 2: For band n256 in Range 3 requirement, the applicable lower frequency range should be modified as 1 ≤ f ≤ FDL_low – 145 NOTE 3: For band n256 in Range 2 requirement, the Pinterferer should be the same as -30 NOTE 4: For band n256 in Range 3 requirement, the Pinterferer should be the same as -15 |  |  |  |  |  |

Table 2-4

Conclusion

In the contribution, the evaluated NTN UE RX requirements are provided.

UE n256 REFSENS:

Observation 1: In [1][2], there are two duplexers assumption for n256. It is agreed to keep UE implementation freedom for both options and choose the minimum requirements among option 1 and option 2 for REFSENS.

Regarding option 2, band n256 REFSENS is shown Table 2-1.

Proposal 1: REFSENS is normally minimum requirement. Regarding two feasible duplexers for band n256, REFSENS in Table 2-1 shall not preclude better UE implementation performance. To remove square brackets of TP REFSENS specification.

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
| n256 | 15 | -99.5 | -96.3 | -94.5 | -93.3 |  |  |  |  |  |  |
|  | 30 |  | -96.6 | -94.6 | -93.5 |  |  |  |  |  |  |
|  | 60 |  | -97.0 | -94.9 | -93.7 |  |  |  |  |  |  |

OOBB requirements for band n256:

Observation 2: Regarding n65 hardware for band n256, similar requirements about reusing n96 hardware for band n102 can be studied. As shown in Table 2-3, Pinterferer for n102 OOBB range2 is modified to be -33dBm. Regarding modified Pinterferer of -33dBm in n102 OOBB range 3, the modified and extended frequency range is FDL_high + MAX(200,3*CBW) < f ≤ MAX(375,3*CBW) MHz.

Proposal 2: Based on observation 2 and evaluation, regarding n256 OOBB range 2 and 3 requirements, it is feasible to remove Note 3 and 4, and the square brackets of Note 1 and 2 as shown Table 2-4.

|  | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| n255, n256 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: For band n256 in Range 2 requirement, the applicable lower frequency range should be modified as -145 < f – FDL_low ≤ -60 NOTE 2: For band n256 in Range 3 requirement, the applicable lower frequency range should be modified as 1 ≤ f ≤ FDL_low – 145 NOTE 3: For band n256 in Range 2 requirement, the Pinterferer should be the same as -30 NOTE 4: For band n256 in Range 3 requirement, the Pinterferer should be the same as -15 |  |  |  |  |  |

- References

[1] R4-2207416, “WF on NTN UE RF requirements”, ZTE

[2] R4-2207441, “Email discussion summary for [102-e][311] NTN_solutions-Part4”, ZTE

[3] R4-2203113, “Email discussion summary for [101-bis-e][308] NTN_Solutions_Part3”, CATT

[4] R4-2119861, “draftCR to 38.101-1 introduction of lower 6GHz NR unlicensed operation for Europe”, Ericsson, Apple, Nokia