3GPP TSG-RAN4 Meeting # 103e	R4-2208576

# Online, 9th – 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | - | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction CA configuration for PDSCH demodulation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_perf_enh-Perf |  |  |  |  |  | Date: |  |  | 2022-05-19 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The CA requirements defined in Rel-16 required the configuration of PUCCH ressource groups. This is however a Rel-16 feature and since the CA tests also do apply in a release independent manner to Rel-15 devices, it is not possible to configure PUCCH ressource groups for those devices. Therefor this element needs to be removed from the configuration tables. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove Number of PUCCH RessourceGroups element. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel-15 devices may fail the Test Cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 38.521-4 CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

## 5.2A	PDSCH demodulation requirements for CA

The parameters specified in Table 5.2-1 for PDSCH single carrier tests are reused for PDSCH CA tests unless otherwise stated.

Table 5.2A-1: Common test parameters for CA

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD and TDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | FDD: 12TDD: Specific to each Reference channel |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | As defined in Table 5.2A-2 |
| TDD UL-DL pattern |  |  | 15kHz SCS: FR1.15-1 30kHz SCS: FR1.30-1 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | As defined in Table 5.2A-3 |
| PUCCH format for HARQ-ACK feedback |  |  | PUCCH format 1 for cases with no more chan 2 DL CCs PUCCH format 3 for cases with more than 2 DL CCs |

Table 5.2A-2: Test parameters for number of HARQ processes

| HARQ process number |  | CCs with the same duplex mode & SCS with Pcell | CCs with different duplex mode / SCS with Pcell |
| --- | --- | --- | --- |
| FDD 15 kHz + TDD 30 kHz CA | FDD PCell | 4 | 8 |
|  | TDD PCell | 8 | 8 |
| FDD 15 kHz + TDD 15 kHz CA | FDD PCell | 4 | 4 |
|  | TDD PCell | 8 | 8 |
| TDD 15 kHz + TDD 30 kHz CA | 15kHz PCell | 8 | 12 |
|  | 30kHz PCell | 8 | 8 |
| FDD 15 kHz + FDD 15 kHz CA | FDD PCell | 4 | N/A |
| TDD 30 kHz + TDD 30 kHz CA | TDD PCell | 8 | N/A |

Table 5.2A-3: Test parameters for K1 values

| The number of slots between PDSCH and corresponding HARQ-ACK information |  | CCs with the same duplex mode and SCS with Pcell | CCs with different duplex mode and/or SCS with Pcell |
| --- | --- | --- | --- |
| FDD 15 kHz + TDD 30 kHz CA | FDD PCell | {2} | {2} |
|  | TDD PCell | {8,7,6,5,5,4,3,2} | {7,5,4,11,9} |
| FDD 15 kHz + TDD 15 kHz CA | FDD PCell | {2} | {2} |
|  | TDD PCell | {4,3,2,6} | {4,3,2,6,5} |
| TDD 15 kHz + TDD 30 kHz CA | 15kHz PCell | {4,3,2,6} | {4,4,3,3,2,2,6,6} |
|  | 30kHz PCell | {8,7,6,5,5,4,3,2} | {7,5,4,11} |
| FDD 15 kHz + FDD 15 kHz CA | FDD PCell | {2} | N/A |
| TDD 30 kHz + TDD 30 kHz CA | TDD PCell | {8,7,6,5,5,4,3,2} | N/A |

< End of changes >