3GPP TSG-RAN WG4 Meeting #103-e		R4-2208610

Electronic Meeting, May 09 – May 20, 2022

Title:	[draft] LS on DC location for intra-band CA

Response to:	-

Release:	Release 17

Work Item:	NR_RF_FR2_req_enh2

Source:	RAN4

To:	RAN2

Cc:	-

Contact person:	Hao Du

duhao.txyjy@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

1	Overall description

RAN4 concluded that the R17 DC location reporting scheme is also applicable for 2CCs and single carrier within CA. The new reporting scheme is optional and for cases where multiple reporting scheme are applicable and UE supports these capabilities, it is up to the network to decide which scheme will be used. For a UL CA configuration in one band combination, there is only one scheme will be used for DC location(s) reporting.

In addition, for FR2, if FreqSeparationClassDL-Only-r16 is configured, the component carrier(s) or BWP(s) located in DL-only spectrum will not be used to calculate the default DC location.

As for the offset range, it should be ±75 MHz for FR1 while ±675 MHz for FR2. It is suggested to use 14 bit to cover FR1 and 15 bit to cover FR2 based on the minimum SCS value, but the detail of signalling bit depends on RAN2’s decision.

2	Actions

To RAN WG2 group

ACTION: 	RAN4 respectfully asks RAN2 to take RAN4 conclusions into consideration.

3	Dates of next TSG RAN WG4 meetings

RAN4#104			22 Aug 2022- 26 Aug 2022 	Toulouse, FR

RAN4#104-bis-e			10 Oct 2022 - 19 Oct 2022 	Electronic