# 3GPP TSG-RAN WG4 Meeting # 103-e                                                        R4-2208700

# Electronic Meeting, May. 9-May. 20, 2022

Source:	ZTE Corporation

Title:	TP for TR38.717-03-01_CA_n3A-n8A-n79A

Agenda Item:	8.6.2

Document for:	Approval

- Introduction

This contribution provides a text proposal to introduce 3DL/1UL CA_n3A-n8A-n79A in TR38.717-03-01.

- Reference

- TR38.717-03-01,Rel-17 NR inter-band Carrier Aggregation for 3 bands DL with 1 band UL,v0.7.0

- Text Proposal

----- Start of TP -----

# 6.X	CA_n3-n8-n79

### 6.X.1	Operating bands for CA

Table 6.X.1-1: 3DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n3-n8-n79 | n3 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

### 6.X.2	Channel bandwidths per operating band for CA

Table 6.X.2-1: Supported channel bandwidths per CA configuration for 3DL inter-band CA

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n8A-n79A | - | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |

### 6.X.3	Co-existence studies

The coexistence studies of harmonic interference have been captured in the constituent fallback modes, where:

- 2nd harmonic produce of band n8 may fall into band n3, and there is no harmonics mixing issue for the band combination of n3 and n8.

- 5th harmonic produce of band n8 may fall into band n79, and there is no harmonics mixing issue for the band combination of n8 and n79.

- There is no harmonics issue and harmonics mixing issue for the band combination of n3 and n79.

    - 6.X.4	∆TIB and ∆RIB values

For CA_n3A-n8A-n79A,  TIB,c and RIB,c values of DC_3-8_n79 are resued, which are given in table 6.x.4.1 and 6.x.4.2

# 6.x.4.1 ΔTIB,c

| Inter-band CA combination | NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| CA_n3-n8-n79 | n3 | 0.3 |
|  | n8 | 0.3 |
|  | n79 | 0.5 |

# 6.x.4.2 ΔRIB,c

| Inter-band CA combination | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| CA_n3-n8-n79 | n3 | 0 |
|  | n8 | 0 |
|  | n79 | 0 |

### 6.X.5	MSD

No additional MSD is needed for 1UL/3DL NR CA CA_n3A-n8A-n79A.

----- End of TP -----