3GPP TSG-RAN WG4 Meeting#103-e	R4-2208750

# Electronic meeting, 9 – 20 May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | CRNum | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of requirements for DL inter-band CA for CA_n258-n261 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Sony |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR2_req_enh2-Core |  |  |  |  |  | Date: |  |  | 2022-05-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduce requirement for DL inter-band CA within the same frequency group for IBM-capable UEs (CA_n258-n261). There should be one minimum requirement per BC and the network should be able to configure a supported band combination based on this. The requirement shall at least ensure the UE is functional for a supported BC (all frequency separations for this) and for the beam management configured according to the declared capability/BC. If the UE does not support all carrier separations the UE does not include this BC amongst supported combinations (no frequency-separation indication) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 3.1: defintion of IBM beam management type. Clause 5.2A.2: the band combiantion with their associated BM capabilty type (IBM only in this release) are introduced, n258 + n261 an example band combination within a single frequency group. Clause 7.1A: the UE shall meet the requirement according to its supported beam management type. Define in-gap receiver measurement between the two bands like the corresponding gap requirement for FR1 (this would also be applicable for overlapping bands) The requirements could be specified as follows: for IBM the UE meets the requirement with different input level PSD. IBM support is beneficial for non-colocation (then the different PSD also makes sense) that may also be used for bands in the same frequency group. Clause 7.3A.2.3: the input levels in the test case for peak EIS: For IBM-only for an inter-band BC, then - Measured carrier: peak REFSENS + relaxation - Other carrier: spherical coverage EIS + relaxation (about 10 dB higher than the measured carrier) The relaxation is 1.7 dB (MBR of 0.7 dB and 1 dB for concurrent operation with unbalanced PSD condition). Clause 7.3A.3.3: the input levels in the test case for EIS sperical coverage: For IBM: - Measured carrier: measure the CCDF as per the existing EIS test procedure - Other carrier: spherical coverage EIS + relaxation, a fixed level A further 1.5 dB relaxation is added to the above peak requirement for compliance with 50% common spherical coverage (3.2 dB total). The peak- and spherical coverage requirements are differentiated since the bands are in the same frequency group. The TP should be > 95% for both carriers for the measured carrier to pass. Swapping the roles, the result should be within the spherical coverage area. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requriements for DL inter-band CA within the same frequency group for IBM-capable UEs are not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 5.2A.2, 7.1A (new), 7.3A.2.3, 7.3A.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  | . |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< start of changes >

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Aggregated Channel Bandwidth: The RF bandwidth in which a UE is configured to transmit and receive multiple contiguously aggregated carriers.

Bidirectional spectrum: UL/DL common spectrum in which the UE supports the configuration of uplink or downlink CCs.

Beam correspondence: the ability of the UE to select a suitable beam for UL transmission based on DL measurements with or without relying on UL beam sweeping.

Beam management reference signal: The DL signal designated by the network for the UE to make measurements on, for the purpose of selecting its DL Rx beam(s).

Carrier aggregation: Aggregation of two or more component carriers in order to support wider transmission bandwidths.

Carrier aggregation band: A set of one or more operating bands across which multiple carriers are aggregated with a specific set of technical requirements.

Carrier aggregation bandwidth class: A class defined by the aggregated transmission bandwidth configuration and maximum number of component carriers supported by a UE.

Carrier aggregation configuration: A combination of CA operating band(s) and CA bandwidth class(es) supported by a UE.

NOTE:	Carriers aggregated in each band can be contiguous or non-contiguous.

Cumulative aggregated channel bandwidth: The cumulative aggregated channel bandwidth is defined as the frequency band from the lowest edge of the lowest CC to the upper edge of the highest CC of all UL and DL configured CCs inside the bidirectional spectrum of the UE.

EIRP(Link=Link angle, Meas=Link angle): measurement of the UE such that the link angle is aligned with the measurement angle. EIRP (indicator to be measured) can be replaced by EIS, Frequency, EVM, carrier Leakage, In-band eission and OBW.

EIRP(Link=TX beam peak direction, Meas=Link angle): measurement of the EIRP of the UE such that the measurement angle is aligned with the beam peak direction within an acceptable measurement error uncertainty. EIRP (indicator to be measured) can be replaced by Frequency, EVM, carrier Leakage, In-band eission and OBW

EIRP(Link=Spherical coverage grid, Meas=Link  angle): measurement of the EIRP spherical coverage of the UE such that the EIRP link and measurement angles are aligned with the directions along the spherical coverage grid within an acceptable measurement error uncertainty. Alternatively, the spherical coverage grid can be replaced by the beam peak search grid as the results from the beam peak search can be re-used for spherical coverage.

EIS (effective isotropic sensitivity): sensitivity for an isotropic directivity device equivalent to the sensitivity of the discussed device exposed to an incoming wave from a defined AoA

EIS(Link=RX beam peak direction, Meas=Link angle): measurement of the EIS of the UE such that the measurement angle is aligned with the RX beam peak direction within an acceptable measurement error uncertainty.

NOTE 1:	The sensitivity is the minimum received power level at which specific requirement is met.

NOTE 2:	Isotropic directivity is equal in all directions (i.e. 0 dBi).

Fallback group: Group of carrier aggregation bandwidth classes for which it is mandatory for a UE to be able to fallback to lower order CA bandwidth class configuration. It is not mandatory for a UE to be able to fallback to lower order CA bandwidth class configuration that belong to a different fallback group

IBM(Independent Beam Management): A UE that supports an inter-band CA configuration with IBM selects its DL Rx beam(s) for all CCs in each configured band based on DL reference signals measurements made in that band.

Inter-band carrier aggregation: Carrier aggregation of component carriers in different operating bands.

NOTE:	Carriers aggregated in each band can be contiguous or non-contiguous.

Intra-band contiguous carrier aggregation: Contiguous carriers aggregated in the same operating band.

Intra-band non-contiguous carrier aggregation: Non-contiguous carriers aggregated in the same operating band.

Link angle: a DL-signal AoA from the view point of the UE, as described in Annex J. If the beam lock function is used to lock the UE beam(s), the link angle can become any arbitrary AoA once the beam lock has been activated.

Measurement angle: the angle of measurement of the desired metric from the view point of the UE, as described in Annex J

radiated interface boundary: operating band specific radiated requirements reference point where the radiated requirements apply

radiated requirements reference point: for the RF measurement setup, the radiated requirements reference point is located at the centre of the quiet zone. From the UE perspective the reference point is the input of the UE antenna array

RX beam peak direction: direction where the maximum total component of RSRP and thus best total component of EIS is found

Sub-block: This is one contiguous allocated block of spectrum for transmission and reception by the same UE. There may be multiple instances of sub-blocks within an RF bandwidth.

TX beam peak direction: direction where the maximum total component of  EIRP is found

TRP(Link=TX beam peak direction, Meas=TRP grid): measurement of the TRP of the UE such that the measurement angles are aligned with the directions of the TRP grid points within an acceptable measurement uncertainty while the link angle is aligned with the TX beam peak direction

NOTE:	For requirements based on EIRP/EIS, the radiated interface boundary is associated to the far-field region

UE transmission bandwidth configuration: Set of resource blocks located within the UE channel bandwidth which may be used for transmitting or receiving by the UE.

Vehicular UE: A UE embedded in a vehicle

## 3.2	Symbols

< text omitted >

### 5.2A.2	Inter-band CA

NR inter-band carrier aggregation is designed to operate in the operating bands and the beam management types defined in Table 5.2A.2-1, where all operating bands are within FR2.

Beam management type is according to the UE capability beamManagementType-r16.

Table 5.2A.2-1: Inter-band CA operating bands in FR2

| NR CA Band | NR Band (Table 5.2-1) | Beam management type |
| --- | --- | --- |
| CA_n257-n259 | n257, n259 | IBM |
| CA_n258-n260 | n258, n260 | IBM |
| CA_n258-n261 | n258, n261 | IBM |
| CA_n260-n261 | n260, n261 | IBM |

## 5.2D	Operating bands for UL MIMO

< text omitted >

## 7.1	General

Unless otherwise stated, the receiver characteristics are specified over the air (OTA). The reference receive sensitivity (REFSENS) is defined assuming a 0 dBi reference antenna located at the center of the quiet zone.

## 7.1A	General

UEs shall meet the requirements for a supported inter-band CA configuration according to the indicated beamManagementType-r16.

For inter-band carrier aggregation of two sub-blocks in different operating bands, overlapping or non-overlapping in frequency, the adjacent channel selectivity requirements and in-band blocking requirements (for each case) apply for in-gap tests only if the corresponding interferer frequency offsets with respect to the two measured sub-blocks satisfy the following condition in relation to the sub-block gap size Wgap for at least one of these sub-blocks j = 1,2, so that the interferer frequency position does not change the nature of the core requirement tested:

Wgap ≥ 2∙|FInterferer (offset),j|  – BWSub-block(j)

where FInterferer (offset),j for a sub-block is the interferer frequency offset with respect to sub-block j of bandwidth BWSub-block(j) as specified in clause 7.5 and clause 7.6.2 for a single component carrier per sub-block and in 7.5A and 7.6A.2 for sub-blocks with two or more contiguous component carriers. The interferer frequency offsets for adjacent channel selectivity and each of the in-band blocking cases shall be tested separately with a single in-gap interferer at a time.

## 7.2	Diversity characteristics

The minimum requirements on effective isotropic sensitivity (EIS) apply to two measurements, corresponding to DL signals in orthogonal polarizations.

< text omitted >

#### 7.3A.2.3	Inter-band CA

The inter-band requirement applies for all active component carriers. The throughput for each component carrier shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.3.2 and A.3.3.2 (with one sided dynamic OCNG Pattern OP.1 TDD for the DL-signal as described in Annex A.5.2.1) at the specified input power level. The throughput requirement shall be met for all carriers at the specified input levels.

For band combinations for which the UE supports IBM beam management type, the requirement for each component carrier shall be met at the peak reference sensitivity level specified in sub-clause 7.3.2 for the respective band increased by the relaxation DRIB,P,n specified in Table 7.3A.2.3-1 with the power of the component carrier in the other band set according to its EIS spherical coverage requirement for inter-band CA as specified in sub-clause 7.3A.3.3.

For the combination of intra-band and inter-band carrier aggregation, the intra-band CA relaxation, ΔRIB, is also applied according to the clause 7.3A.2.1 and 7.3A.2.2.

Table 7.3A.2.3-1:  ΔRIB,P,n reference sensitivity relaxation for inter-band CA with IBM capability for power class 3

| NR CA band combinations | NR band | ΔRIB,P,n (dB) |
| --- | --- | --- |
| CA_n257-n259 | n257 | 4.0 |
|  | n259 | 4.0 |
| CA_n258-n260 | n258 | 3.5 |
|  | n260 | 3.5 |
| CA_n258-n261 | n258 | [1.7] |
|  | n261 | [1.7] |
| CA_n260-n261 | n260 | 3.5 |
|  | n261 | 3.5 |

### 7.3A.3	EIS spherical coverage for DL CA

#### 7.3A.3.1	Void

#### 7.3A.3.2	Void

#### 7.3A.3.3	EIS spherical coverage for inter-band CA

The inter-band CA requirement applies per operating band, for all active component carriers with UL assigned to one band and one DL component carrier per band. The reference measurement channels and throughput criteria shall be as specified in clause 7.3A.2.3. The requirement shall be met for an uplink transmission using QPSK DFT-s-OFDM waveforms and for uplink transmission bandwidth less than or equal to that specified in clause 7.3.2. .

For band combinations for which the UE supports IBM beam management type, the spherical coverage requirement for each component carrier in the respective band shall be met as specified in sub-clause 7.3.4 increased by the relaxation DRIB,S,n specified in Table 7.3A.3.3-1 with the power of the component carrier in the other band fixed at its EIS spherical coverage requirement power level specified in Table 7.3A.3.3-1.

The inter-band CA spherical coverage requirement for each power class will be satisfied if the intersection set of spherical coverage areas exceeds the common coverage requirement. Intersection set of spherical coverage areas is defined as a fraction of area of full sphere measured around the UE where both bands meet their defined individual EIS spherical coverage requirements for inter-band CA operation. The common coverage requirement is determined as <100-percentile rank> %, where ‘percentile rank’ is the percentile value in the specification of spherical coverage for that power class from clause 7.3.4.The requirement is verified with the test metric of EIS (Link=Beam peak search grids, Meas=Link angle).

Unless otherwise specified, the minimum requirements for reference sensitivity shall be verified with the network signalling value NS_200 (Table 6.2.3.1-1) configured.

The required spherical coverage EIS for each band in inter-band CA operation is given in clause 7.3.4 and modified by ΔRIB,S,n. The value of ∆RIB,S,n is defined in Table 7.3A.3.3-1.

Table 7.3A.3.3-1: ΔRIB,S,n EIS spherical coverage requirement relaxation for inter-band CA with IBM capability for power class 3

| NR CA band combination | NR band | ΔRIB,S,n (dB) |
| --- | --- | --- |
| CA_n257-n259 | n257 | 3.5 |
|  | n259 | 3.5 |
| CA_n258-n260 | n258 | 3.5 |
|  | n260 | 3.5 |
| CA_n258-n261 | n258 | [3.2] |
|  | n261 | [3.2] |
| CA_n260-n261 | n260 | 3.5 |
|  | n261 | 3.5 |

## 7.3D	Reference sensitivity for UL MIMO

< end of changes >