3GPP TSG-RAN WG4 # 103-e	R4-2208840

Electronic Meeting, May 09 – May 20, 2022

Agenda item:	9.18.4.2.2

Source:	MediaTek Inc.

Title:	Discussion on demodulation performance requirements definition for Rel17 multi-TRP

Document for:	Discussion

# Introduction

In RAN4#102-e, the detail for the simulation assumption for PDCCH with FDM repetition scheme was agreed [1]. We provide discussion with simulation results in this contribution.

# Discussion

The simulation settings are based on the agreed assumptions from 102-e WF document [1] and shown in Table 1. Multi-transmission point (TRP) PDCCH repetition is simulated with agreed Aggregation Levels (AL) 2 and 4. In addition for fair comparison Single-TRP PDCCH performance is simulated with AL 4 and 8.

Table 1: Simulation settings.

| Parameter | Value |
| --- | --- |
|  | FDD 15 kHz SCS |
| Repetition transmission schemes | FDM |
| CBW | 10 MHz |
| CORESET RB | 24 |
| CORESET duration | 2 |
| Aggregation level for repetition | 2/4 |
| Aggregation level for single TRP | 4/8 |
| Propagation Condition | TDLA30-10 |
| Antenna configuration | 2x2, 2x4 |
| CCE to REG mapping type | nonInterleaved |
| REG bundle size | 6 |
| Payload bits(without CRC) | 39 |
| Test metric | 1% of Pm-dsg (%) |

PDCCH
AL = {2,4}

PDCCH
AL = {2,4}

TRP2

TRP1

In the following simulation results we want to do fair performance comparison with the same number of resources elements (RE) from either 2 TRPs using AL (see Figure 1) or 1 TRP using AL*2 (see Figure 2). Also, we want to check performance impact of SNR imbalance between 2 TRPs where the secondary TRP (TRP2) SNR is decreased by 0, 3, 6 or 9 dBs compared to TRP1. Single-TRP in these simulations is always TRP1.

Figure 1: Multi-TRP PDCCH repetition with AL = {2, 4}.

PDCCH
AL = {4,8}

TRP1

TRP2

Figure 2: Single PDCCH with AL = {4, 8}.

The simulations results are collected in 2 Tables where Table 2 has results with 4 receiver antennas and Table 3 has results with 2 receiver antennas. Positive gain means PDCCH repetition scheme is better than single TRP with double AL and vice versa. Performance is measured in 1% Pm-dsg. We can do the following observations from these results.

Observation #1: When SNR of 2 TRPs is balanced we can see little gain on average.

Observation #2: When SNR of 2 TRPs is balanced we can see loss in some cases.

Observation #3: When SNR of 2 TRPs is imbalanced the performance of PDCCH repetition decreases as expected.

Our conclusion of these 3 observations is that multi-TRP PDCCH repetition transmission scheme offers only minor benefits and therefore it is not seen beneficial feature.

Table 2: Simulation results of 2 transmit and 4 receiver antennas

| TRP1 and TRP2 SNR difference | PDCCH repetition AL2 vs PDCCH single TRP AL4 gain | PDCCH repetition AL4 vs PDCCH single TRP AL8 gain |
| --- | --- | --- |
| 0 dB | -0.88dB | 0.88dB |
| 3 dB | -2.20dB | -0.62dB |
| 6 dB | -3.36dB | -1.65dB |
| 9 dB | -4.37dB | -2.52dB |

Table 3: Simulation results of 2 transmit and 2 receiver antennas

| TRP1 and TRP2 SNR difference | PDCCH repetition AL2 vs PDCCH single TRP AL4 gain | PDCCH repetition AL4 vs PDCCH single TRP AL8 gain |
| --- | --- | --- |
| 0 dB | 0.37dB | 1.65dB |
| 3 dB | -1.02dB | 0.39dB |
| 6 dB | -2.52dB | -0.84dB |
| 9 dB | -3.68dB | -2.02dB |

The following options were agreed for further discussion previous meeting

Issue 1-1-1: Whether to define PDCCH requirement for multi-TRP repetition transmission schemes

Candidate options:

- Option 1: Yes

- Option 2: No

[1]:

Issue 1-1-3: Whether to define PDSCH requirement for Multi-TRP inter-cell operation

Candidate options:

- Option 1: Yes

  - Option 1a: Introduce test applicable rule between existing Multi-DCI intra-cell M-TRP test case and new test case for inter-cell Multi-DCI PDSCH

  - Option 1b: Define performance requirement for enhancements on multi-TRP inter-cell operation with full-overlapping resource allocation.

- Option 2: No

  - Option 2a: Define applicability for UE that supports “IntCell-Mtrp” feature that if such UE satisfied Rel-16 minimum requirements for PDSCH multi-DCI based transmission scheme, inter-cell operation can be also guaranteed

Proposal #1: We support Option 2 to not define PDCCH requirement for multi-TRP repetition transmission scheme as we see only minor benefits.

Proposal #2: We support Option 2/2a to not define PDSCH requirement for multi-TRP inter-cell operation as we see the existing Rel-16 minimum requirements for PDSCH multi-DCI based transmission scheme is enough to guarantee performance.

# Conclusion

In this paper we provided the view on the requirements definition for multi-TRP Rel-17 enhancements. The following observations and proposals are made:

Observation #1: When SNR of 2 TRPs is balanced we can see little gain on average.

Observation #2: When SNR of 2 TRPs is balanced we can see loss in some cases.

Observation #3: When SNR of 2 TRPs is imbalanced the performance of PDCCH repetition decreases as expected.

Proposal #1: We support Option 2 to not define PDCCH requirement for multi-TRP repetition transmission scheme as we see only minor benefits.

Proposal #2: We support Option 2/2a to not define PDSCH requirement for multi-TRP inter-cell operation as we see the existing Rel-16 minimum requirements for PDSCH multi-DCI based transmission scheme is enough to guarantee performance.

# Reference

- R4-2207207, “WF on demodulation requirement for Enhancement on Multi-TRP”, Huawei, HiSilicon