3GPP TSG-RAN WG4 Meeting #103-e 	       R4-2208878

# Electronic Meeting, 9 - 20 May 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on PUSCH demodulation requirements for NTN

Agenda item: 	9.12.8.2.1

Document for:	Discussion

# Introduction

The NTN WI is presented in [1], where the following RAN4 objectives are defined:

### 4.1.4 RAN4 Study the framework how NTN core requirements are defined. Specify the following requirements [RAN4] (Note 1) - UE RRM core requirements - Study and identify which bands may be potentially relevant to NTN including: - Analysis of regulations in the spectrum considered - Adjacent channel co-existence - Considering the potential bands to be used as example for the WID: - Specify needed generic RF core requirements for the network and the UE such that adjacent channel co-existence scenarios are met and performance of other RF parameters (RX performance, TX signal quality etc.) are subject to acceptable minimum requirements - Investigate and specify UE timing & frequency pre compensation accuracy requirements as needed [RAN4]. Note 1: It is assumed that this work item will be frequency agnostic and therefore we can consider that NTN can operate in FR1 or FR2 ranges. Defining NR bands for NTN should be included as part of dedicated Rel-17 RAN4 led work items including an analysis of regulations in spectrum considered, which bands 3GPP should specify, as well as potential co-existence between NR terrestrial and satellite Note 2: The spectrum usage on the service link for HAPS might be a different spectrum allocation than for Satellite.

In this contribution we present discussion of some open issues, proposal for simulation parameters and preliminary simulation results of PUSCH demodulation requirements for NTN.

# Discussion

## PUSCH demodulation requirements

Issue 2-2-3: SCS/CBW set for PUSCH requirements

Candidate options

- Option 1: 15kHz SCS: 5MHz/10MHz/20MHz, 30kHz SCS: 10MHz/20MHz

- Option 2: a few of PRBs for all SCS

Issue 2-2-5: Antenna configuration for PUSCH requirements

Agreements

- UE 1Tx

- SAN 2Rx

Candidate options

- FFS: SAN 1Rx, SAN 4Rx and SAN 8Rx

Issue 2-2-6: Test parameters for NTN PUSCH

For information, not agreement

- Interested companies are encouraged to provide initial simulation results in next meeting.

Agreed way forward [2]:

Issue 2-2-3 : SCS/CBW set for PUSCH requirements

NTN targets to provide the most critical services where few PRBs configuration is most commonly used for all SCS. A few PRBs allocation is also usually used in coverage limited scenario. Hence, a wide/full PRB allocation is not expected for PUSCH in NTN (e.g. voice, IoT, ...).

Observation 1 : A few PRBs allocation is the most commonly used in the considered NTN applications.

Our preliminary results with 4 PRB shows quite similar performance for different CBW.

Observation 2 : Considering different CBW with the same PRB allocation have similar results.

Hence, we support few PRBs allocation.

Proposal 1 : Consider 4 PRBs allocation for all SCS.

Issue 2-2-5 : Antenna configuration for PUSCH requirements

The most common SAN antenna arrays is assumed to be 2 Rx. Doubling Rx antennas at SAN Rx provide a predictable receive diversity gain. Thus, it is enough to consider SAN 2 Rx maximum even if SAN will support larger antenna array.

Observation 3 : The most common SAN antenna arrays is 2 Rx.

Observation 4 : Doubling Rx antennas at SAN Rx provide a predictable receive diversity gain.

Proposal 2 : Consider UE 1 Tx and SAN 1Rx, SAN 2 Rx.

## Simulation parameters

Proposed simulation parameters for demodulation requirements are shown in Table 1. The parameters are assuming NTN TDL-A, NTN TDL-C channel profiles, MCS4, and 0.1 ppm doppler error in service link according to agreement in [2]. Delay spread is 100ns according to agreement made in RAN4 #102-e.

We have chosen to use k-factor of 21.6 for TDL-C channel, corresponding to suburban scenario with 50 degrees elevation in TR 38.811 [4]. This k-factor seems to be a good compromise providing adequate difference between the LOS and NLOS conditions.

In these simulations we have assumed UE velocity to be 3km/h as it is also in TR 38.821 [3] link simulation assumptions for S-band.

We propose the parameters provided to be taken into account when parameters for NTN PUSCH demodulation requirements are decided.

Proposal 3:  Use parameters in Table 1 for NTN PUSCH demodulation requirements.

We have discussed the residual frequency error (doppler shift,) in our accompanying contribution [3].

Table 1 Proposed simulation parameters for PUSCH demodulation requirements

| Parameter |  | Value |
| --- | --- | --- |
| Number of UE antennas/SAN antennas |  | 1x1 1x2 |
| Channel model |  | NTN-TDL- A 100 ns (NLOS) NTN-TDL-C 100ns (LOS) |
| Channel model k-factor |  | 21.6 (corresponds to suburban scenario with 50 degrees elevation) |
| UE speed |  | 3km/h |
| SCS/CBW |  | 15kHz SCS: 10MHz/20MHz, 30kHz SCS: 10MHz/20MHz |
| Fc |  | 2GHz |
| Residual frequency error |  | 0.1ppm (200Hz) |
| MCS |  | MCS4 |
| Transform precoding |  | Enabled |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | Pos1 |
|  | Number of DM-RS CDM group(s) without data | 1 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | {0} |
|  | DM-RS sequence generation | NID0=49, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type | A |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
| Frequency domain resource assignment | RB assignment | 4PRBs |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |

## Simulation results

Preliminary simulation results for PUSCH demodulation requirements are shown in the following Figures 1-4.

The BLER and throughput percentage is provided for all considered configurations shown in Table 1. The results consider 0.1ppm (200Hz) residual frequency error and the perfect case with no error, as well as both proposed SAN configuration (1 Rx and 2 Rx).

Observation 5: The degradation with 0.1 ppm residual frequency error is negligible with MCS4 for all configuration even with SAN 1 Rx.

Observation 6: Results with SAN 1 Rx, especially with NTN TDL-A, require a high SNR even with MCS4 that could not be available for some cases. Important gain is achieved with SAN 2 Rx receive diversity especially in NLOS NTN-TDL-A.

Observation 7: For each channel profile, the 1% BLER and 70% throughput with MCS4 and same PRB allocation do not differ significantly between the different SCS/CBW.

| NTN-TDL-A SCS=15 KHz with CBW=10MHz/20MHz |  |
| --- | --- |

Figure 1 Results for NTN-TDL-A SCS=15 KHz with CBW=10MHz/20MHz

| NTN-TDL-C SCS=15 KHz with CBW=10MHz/20MHz |  |
| --- | --- |

Figure 2 Results for NTN-TDL-C SCS=15 KHz with CBW=10MHz/20MHz

| NTN-TDL-A SCS=30 KHz with CBW=10MHz/20MHz |  |
| --- | --- |

Figure 3 Results for NTN-TDL-A SCS=30 KHz with CBW=10MHz/20MHz

| NTN-TDL-C SCS=30 KHz with CBW=10MHz/20MHz |  |
| --- | --- |

Figure 4 Results for NTN-TDL-C SCS=30 KHz with CBW=10MHz/20MHz

# Conclusion

This contribution discusses aspects related to PUSCH demodulation requirements for NTN and has the following proposal and observations:

Observation 1 : A few PRBs allocation is the most commonly used in the considered NTN applications.

Observation 2 : Considering different CBW with the same PRB allocation have similar results.

Proposal 1 : Consider 4 PRBs allocation for all SCS.

Observation 3 : The most common SAN antenna arrays is 2 Rx.

Observation 4 : Doubling Rx antennas at SAN Rx provide a predictable receive diversity gain.

Proposal 2 : Consider UE 1 Tx and SAN 1Rx, SAN 2 Rx.

Proposal 3:  Use parameters in Table 1 for NTN PUSCH demodulation requirements.

Observation 5: The degradation with 0.1 ppm residual frequency error is negligible with MCS4 for all configuration even with SAN 1 Rx.

Observation 6: Results with SAN 1 Rx, especially with NTN TDL-A, require a high SNR even with MCS4 that could not be available for some cases. Important gain is achieved with SAN 2 Rx receive diversity especially in NLOS NTN-TDL-A.

Observation 7: For each channel profile, the 1% BLER and 70% throughput with MCS4 and same PRB allocation do not differ significantly between the different SCS/CBW.

# References

- RP-220208, Revised WID: Solutions for NR to support non-terrestrial networks (NTN), Thales

- R4-2207198, WF on NTN SAN demodulation requirements, Huawei

- R4-2208874, Discussion on general issues for NTN demodulation requirements, Nokia, NSB