3GPP TSG-RAN4 Meeting #103-e	R4-2208902

# Electronic meeting, May 3 - 20, 2022

Source:		Ericsson, UIC

Title:			RMR1900 MHz band – TP to TR 38.852

Agenda item:		5.1.3

Document for:		Approval

# Introduction

The 1900MHz NR band for European RMR WI ([1]) has been finalzied in last RAN4#102-e meeting introducing a new 1900MHz band for Railway in Europe.

This contribution is proposing a TP to TR 38.852 clarifying BS maximum output power level for 5 MHz channel bandwidth.

# Proposal

It is proposed that the proposed updates to TR 38.852 [2].

# References

- RP-211495, Introduction of 1900MHz NR band for Europe for Rail Mobile Radio (RMR), UIC

- TR 38.852, Introduction of 1900MHz NR band for Europe for Rail Mobile Radio (RMR)

# Text proposal

<Start of the change>

#### 7.1.2.1	BS maximum output power

Based on ECC Decision (20)02 [1], the BS maximum output power for BS operating in band n101 applicable for [uncoordinated deployment], shall not exceed the Prated,c,AC derived based on table 4.3.1-1, with the assumption of fixed value of the BS antenna gain GantRMR1900 as:

(65 – GantRMR1900) dBm / 10 MHz = 47 dBm / 10 MHz,

(62 – GantRMR1900) dBm / 5 MHz = 44 dBm / 5 MHz,

where GantRMR1900 is the antenna gain of the RMR BS.

<End of the change>