3GPP TSG-RAN WG4 Meeting #103-e	R4-2208987

Electronic Meeting, May 09 – 20, 2022

Title: 	Discussion on maintaining PL-RS switching delay requirements in R16

Source: 	Huawei, HiSilicon

Agenda item:	4.1.5.1

Document for:	Discussion

Introduction

In this contribution, we provide the discussion on maintaining issues on PL-RS switching delay requirements for NR eMIMO.

Discussion

In R16, the PL-RS switching delay requirements are only applied when the target PL-RS is known. During PL-RS switching delay, at most 5 samples without beam sweeping is allowed for PL-RS RSRP calculation.

Observation 1: The existing PL-RS switching delay requirements are applied when UE is not required to perform beam sweeping on the target PL-RS.

However, there is no TCI state configuration for SSB resource. The current known conditions for PL-RS signal are defined as:

The pathloss reference signal is known if the following conditions are met during the period between the last transmission of the RS resource used for L1-RSRP measurement reporting and the completion of pathloss reference signal switch, where the RS resource is the target pathloss reference signal or QCLed (with Type D) to the target pathloss reference signal. - Pathloss reference signal switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement - The UE has sent at least 1 L1-RSRP report for the target pathloss reference signal before the pathloss reference signal switch command - The target pathloss reference signal remains detectable during the pathloss reference signal switching period - SNR of the target pathloss reference signal≥-3dB - The associated SSBs with the target pathloss reference signal remain detectable during the pathloss reference signal switching period - SNR of the associated SSB ≥-3dB Otherwise, the pathloss reference signal is unknown.

Both SSB and periodic CSI-RS can be configured as PL-RS. There is no TCI state configuration for SSB resource. According to the current known condition requirements, a SSB indicated as PL-RS is considered to be known only when this SSB is also used for L1-RSRP measurement reporting. Since R15, beam sweeping is always assumed for L1-RSRP measurements measured on SSB resource in FR2. Therefore, beam sweeping shall also be assumed for PL-RS measurements measured on SSB resource in FR2.

Observation 2: When a SSB resource indicated as PL-RS is also configured for L1-RSRP measurements, UE needs to perform beam sweeping on the SSB resource for both PL-RS measurements and L1-RSRP measurements.

When the target PL-RS is a SSB, regardless of whether PL-RS is known or not, UE still need to perform beam sweeping for PL-RS measurements. However, at most 5 measurement samples are allowed for PL-RS RSRP calculation during PL-RS switching delay, which does not allow enough time for UE performing beam sweeping on the SSB resource. Longer PL-RS switching delay is expected. Two options can be considered. Option 1 is only to clarify that longer switching delay is expected for SSB based PL-RS in FR2, which can be captured as a note. Option 2 is to define the PL-RS switching delay for SSB based PL-RS in FR2, and the switching delay can defined as 5 L1-RSRP measurement intervals. We prefer option 1 to use the same pro

Proposal 1: For PL-RS switching in FR2, the following two options can be considered to define the PL-RS switching delay when the target PL-RS is SSB and used for L1-RSRP measurements.

- Option 1: (Preferred)

  - To clarify that longer PL-RS switching delay is expected, which can be captured in the note.

- Option 2:

  - To define the PL-RS switching delay as 5*TL1-RSRP_SSB, where TL1-RSRP_SSB is SSB based L1-RSRP measurement period with the assumption of M=1.

Conclusions

This contribution provides our analysis on maintaining PL-RS switching delay requirements for R16 NR eMIMO. The followings are provided:

Observation 1: The existing PL-RS switching delay requirements are applied when UE is not required to perform beam sweeping on the target PL-RS.

Observation 2: When a SSB resource indicated as PL-RS is also configured for L1-RSRP measurements, UE needs to perform beam sweeping on the SSB resource for both PL-RS measurements and L1-RSRP measurements.

Proposal 1: For PL-RS switching in FR2, the following two options can be considered to define the PL-RS switching delay when the target PL-RS is SSB and used for L1-RSRP measurements.

- Option 1: (Preffered)

  - To clarify that longer PL-RS switching delay is expected, which can be captured in the note.

- Option 2:

  - To define the PL-RS switching delay as 5*TL1-RSRP_SSB, where TL1-RSRP_SSB is SSB based L1-RSRP measurement period with the assumption of M=1.

A draft CR [1] is submitted accordingly.

Reference

- R4-2208988,  “DraftCR on maintaining PL-RS switching delay requirements”, Huawei, HiSilicon