3GPP TSG-RAN WG4 Meeting #103e	R4-2209030

Electronic Meeting, 9th – 20th May, 2022

Source:	ZTE Corporation

Title:	Discussion on RRM test cases for NR SDT

Agenda Item:	9.24.2

Document for:	Discussion

# Introduction

For the Rel-17 WI on NR small data transmissions in INACTIVE state, the core part was completed with a big CR for RRM core requirements [3]. Now it is time to discuss the performance objective [1] [2]:

### 4.2 Objective of Performance part WI NOTE: Leave empty if the WI proposal does not contain a RAN performance part. - Specify RRM test cases.

As agreed in RAN4, a similar approach to LTE PUR is applied for NR SDT. However, since there was no RRM test cases defined in particular for LTE PUR [4]-[16], we propose to remove the above performance objective from the WID and close the whole WID.

# Discussion

For RA-SDT, since RAN4 agreed that RAN4 does not introduce any new UE RRM requirements for RA-SDT[17], no new RRM test cases need to be defined.

Observation 1: No new RRM test cases to be defined for RA-SDT.

In the agreed big CR on RRM core requirements for NR CG-SDT [3], there are mainly three requirements involved:

- Requirement 1: UE synchronization requirement for CG-SDT: reuse the existing requirements using the latest available NTA value as specified in subclause 7.1.2. - Requirement 2: TA validation requirement for CG-SDT: - Two time windows are specified for measuring RSRP1 and RSRP2 - The threshold for the two RSRP values difference is defined in RAN2 specs - Requirement 3: UE initial timing requirement for CG-SDT: Reusing the existing requirements defined in subclause 7.1.2.

Obviously, for Requirement 1 and 3 which are reusing the existing requirements, there is no need to introduce additional RRM test cases.

Observation 2: No need to introduce additional RRM test cases for UE synchronization requirement and initial timing requirement for NR SDT which are reusing the existing requirements.

For the TA validation requirement for SDT, RAN4 specifies two windows for measuring RSRP1 and RSRP2, and the difference threshold is defined in RAN2 specs. From RRM perspective, the two windows are clearly defined, and UE has a flexibility and freedom to measure RSRP values at any moment within the two windows, thus there is no need to introduce an RRM test case for such purpose.

Observation 3: No need to introduce an RRM test case for TA validation for NR SDT.

Actually, this is also aligned with LTE PUR. In the WID “Additional MTC enhancement for LTE” where LTE PUR was specified, the final RRM test cases were specified for relaxed serving cell measurements, RSS-based measurements, RLM with enhanced MPDCCH, and DL channel quality reporting, and there was no RRM test cases defined in particular for LTE PUR [10].

Observation 4: There was no RRM test cases defined in particular for LTE PUR in the WID “Additional MTC enhancement for LTE”.

With such considerations, we propose to remove the performance objective from the WID on NR SDT.

Proposal: Remove the performance objective specifying RRM test cases for NR SDT.

# Conclusion

In this paper, we have the following observations and proposal for the RRM test cases for NR SDT:

Observation 1: No new RRM test cases to be defined for RA-SDT.

Observation 2: No need to introduce additional RRM test cases for UE synchronization requirement and initial timing requirement for NR SDT which are reusing the existing requirements.

Observation 3: No need to introduce an RRM test case for TA validation for NR SDT.

Observation 4: There was no RRM test cases defined in particular for LTE PUR in the WID “Additional MTC enhancement for LTE”.

Proposal: Remove the performance objective specifying RRM test cases for NR SDT.

# Reference

RP-212594 Work Item on NR small data transmissions in INACTIVE state, ZTE

RP-220153 SR on NR small data transmissions in INACTIVE state, RAN2

R4-2207128 Big CR on RRM requirements for Rel-17 NR SDT in INACTIVE state, ZTE

R4-2017071 Big CR: Introduction of Rel-16 eMTC RRM performance requirements (TS 36.133), Ericsson

RP-202377, Big CR: Introduction of Rel-16 eMTC RRM performance requirements (TS 36.133)

RP-192001, SR on Additional MTC enhancements for LTE(Core 65%, Perf. 0%)

RP-192874, SR on Additional MTC enhancements for LTE(Core 90%, Perf. 10%)

RP-200819, SR on Additional MTC enhancements for LTE (Core 100%, Perf. 40%) with the following open issues:

For the RRM performance part, there are the following open issues in RAN4: - Agree on the scope of RRM tests - Agree 36.133 CR for test cases corresponding to the test scope

RP-201520, SR on Additional MTC enhancements for LTE (Core 100%, Perf. 75%)

#### 2.4.2 Remaining Open issues For the RRM performance part, there are the following open issues in RAN4: - Agree TS36.133 CR for test cases corresponding to the test scope

RP-202377, SR on Additional MTC enhancements for LTE (Core 100%, Perf. 100%)

- RAN4#97-e - To this meeting, 11 contributions were submitted (for detailed see agenda items 6.1.3 and 6.1.4 in Tdoc list). - RAN4 endorsed all the necessary draft CRs of RRM test cases for relaxed serving cell measurements, RSS-based measurements, RLM with enhanced MPDCCH, and DL channel quality reporting. All the endorsed CRs were combined into a single big CR in R4-2017071.

RP-192040   Cat B   CR for Tx timing requirements for PUR transmission

RP-193023	Cat B	NB-IoT PUR transmit timing requirements

RP-200962	Cat B	CR to add additional timing requirements for PUR

RP-201509	Cat F	CR on PUR requirements for NB-IoT

RP-201508	Cat F	CR on PUR related requirements

RP-202483	Cat F	CR on PUR requirements for NB-IoT

R4-2120338  WF on RRM requirements for NR SDT in INACTIVE state, ZTE