3GPP TSG-RAN WG4 Meeting # 103-e                	R4-2209052

Electronic Meeting, May 9 – 20, 2022

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Discussion on RRM performance requirements for extension to 71 GHz

Agenda item: 	9.15.9

Document for: 	Discussion

Introduction

This paper presents Nokia’s view on RRM performance requirements for the operation between 52.6 GHz and 71 GHz.

RRM performance requirements without CCA

From the WID for extension to 71 GHz we have the following RRM performance objectives [1]:

### 4.2 Objective of Performance part WI NOTE: Leave empty if the WI proposal does not contain a RAN performance part. Specify the following requirements [RAN4] - Base station demodulation performance requirements - UE demodulation performance requirements - Radio Resource Management performance requirements, including RRM/RLM test cases - Base station conformance testing - Study and define NR 52.6-71 GHz BS OTA methods

As part of the RRM core requirements, the reason for new requirements were mostly due to:

- Impact of different subcarrier spacings

- Impact due to changing RX beam sweeping factor

Regarding the new subcarrier spacings, it is important that there are enough test cases that guarantee test coverage. Furthermore, it cannot be assumed that a UE that meets the RRM performance requirements for one SCS will be able to meet similar requirements for another SCS. Therefore, we think it is important to define RRM performance for 480 and 960 kHz SCS.

Most of the existing RRM performance requirements cannot be assumed to cover the UE behaviour with 480 and 960 kHz SCS.

Define RRM performance requirements for 480 kHz and 960 kHz SCS.

For RRM performance requirements using 120 kHz SCS, it would be desirable to reuse as many test cases as possible in order to avoid workload. Many test cases would be in principle reusable for 120 kHz SCS if configurations that are allowed for FR2-2 are used and if the RRM core requirements related to that test are not affected by FR2-2.

The Rx beam sweeping factor is still under discussion for the RRM core requirements, and as analyzed in our RRM core discussion paper, it has impact in many core requirements. There is still no definition on the Rx beam sweeping

Additionally, many test cases for FR2-1 use 240 kHz SCS for the SSB, which is not supported at FR2-2. For these test cases, it is clear that new RRM performance requirements need to be defined for FR2-2.

No definition on RRM core requirements was reached so far regarding Rx beam sweeping.

Open options for Rx beam sweeping are N=8 (unchanged) or in the range from N=12 to N=16.

If beam sweeping scaling factor is changed, most of the RRM performance requirements from FR2-1 with 120 kHz cannot be reused for FR2-2.

Some FR2-1 test cases may be reused when the configuration does not include SSB SCS which is not supported by FR2-2.

Attempt to reuse FR2-1 RRM performance requirements for 120 kHz SCS that do not depend on the Rx Beam sweeping scaling factor.

If Rx beam sweeping scaling factor is changed in RRM core requirements, define new RRM performance requirements for 120 khz SCS considering the decision on the number of Rx Beam for RRM requirements.

RAN4 not to reuse FR2-1 RRM performance requirements for 120 kHz SCS with SSB using 240 kHz SCS.

RAN4 not to reuse FR2-1 RRM performance requirements for 120 kHz SCS for test cases whose RRM core requirements changed, for example due to new Rx beam sweeping scaling factor.

Deployment scenarios for RRM performance requirements:

RAN4 #101-e Deployment scenarios - To clarify the scope of RRM, RRM requirements for Ext_to_71GHz should be defined for the scenarios below in Rel-17. - FR2-2 single carrier and CA in SA - FR1+FR2-2 CA (FR1 is PCell) - FR1+FR2-2 DC (FR1 is PCell) - Other scenarios are precluded in Rel-17. RAN4 #101-bis-e Deployment scenarios - Following the agreement from RAN4#101e, no RRM requirements for EN-DC/NE-DC with FR2-2 will be defined in Rel-17

Considering the agreed deployment scenarios, we propose to use the same deployment scenarios as previously agreed.

RAN4 to define RRM performance requirements for the following scenarios:
                      -FR2-2 only standalone
                      -FR2-2 non-contiguous CA 
                      -FR1+FR2-2 CA (FR1 is PCell)
                      -FR1+FR2-2 DC (FR1 is PCell)

Since there is a limited number of meetings to finalise the RRM performance work. We think it is important to limit the scope of the RRM performance requirements in order to keep a manageable workload. Ideally, we would define test cases that are enough for guaranteeing a good test coverage and avoid redundancy in the tests. One option is to prioritize FR2-2 test cases taking as a baseline the Rel 15 test cases. Additionally, in order to reduce the total number of test cases, another approach is for requirements with multiple configurations/scenarios, attempt to downscope the test cases in such a way that common configuration/scenarios are not repeated. One example of such procedure can be shown in Table 1. In this table, the sub-tests that are down-scoped for SSB-based test cases are prioritized for CSI-RS based test cases. For example, out-of-sync with DRX configuration would used for the CSI-RS based configuration, but not for the SSB-based configuration. A list of test cases with their priority using this approach is defined in Table 2.

Table 1 Example of down-scoping strategy with wide testing coverage

| Group of requirements | Test cases | Sub-test | Define requirements/down-scope |
| --- | --- | --- | --- |
| Radio link monitoring | FR2-2 PCell, SSB-based | Out-of-sync, in non-DRX | Yes |
|  |  | In-sync, in non-DRX mode | Down scope |
|  |  | Out-of-sync, in DRX mode | Down scope |
|  |  | In-sync, in DRX mode | Yes |
|  | FR2-2 PCell, CSI-RS-based | Out-of-sync, in non-DRX | Down scope |
|  |  | In-sync, in non-DRX mode | Yes |
|  |  | Out-of-sync, in DRX mode | Yes |
|  |  | In-sync, in DRX mode | Down scope |

RAN4 to use as a baseline for the RRM performance without CCA the Rel 15 test cases.

RAN4 to discuss the list of test cases for RRM performance without CCA based on Table 2.

Table 2 Proposed RRM test case list for the operation above 52.6 GHz.

| Group of requirements | Test cases | Sub-test | 120 kHz SCS | 480/960 kHz SCS |
| --- | --- | --- | --- | --- |
| RRC_IDLE, cell re-selection | FR2-2 -> FR2-2 | intra-frequency | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
|  |  | inter-frequency | Can be reused from FR2-1 if RX beam sweeping is not changed | yes |
| HO (delay and interruptions) | FR2-2->FR2-2 | intra-frequency, unknown target cell | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
|  |  | inter-frequency, unknown target cell | Can be reused from FR2-1 if RX beam sweeping is not changed | Low priority |
|  | FR1->FR2-2 | unknown target cell | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
| RRC Re-establishment | FR2-2->FR2-2 | intra frequency | Can be reused from FR2-1 if RX beam sweeping is not changed | Low priority |
|  |  | inter frequency | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
|  |  | intra frequency without serving cell timing | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
| Random access | 4-step RACH | Contention based | Reuse FR2-1 | Yes |
|  |  | Non-contention based | Reuse FR2-1 | Yes |
| RRC Connection Release with Redirection | FR2-2->FR2-2 |  | Can be reused from FR2-1 if RX beam sweeping is not changed | Low priority |
| Timing | UE Transmit Timing |  | Yes FR2 configuration uses 240 kHz SSB, so new test is needed for 120 kHz SCS | Yes |
|  | Timing advance adjustment accuracy |  | Yes FR2 configuration uses 240 kHz SSB, so new test is needed for 120 kHz SCS | Yes |
| Active BWP switch | DCI-based and timer-based active BWP switch | FR2-2->FR2-2 | Reuse FR2-1 | Yes |
|  |  | FR1->FR2-2 | Reuse FR2-1 | Low priority |
|  |  | One FR2-2 cell | Reuse FR2-1 | Low priority |
|  | RRC-based active BWP switch |  | Reuse FR2-1 | Yes |
|  | SCell dormancy switch | FR2-2->FR2-2 | Reuse FR2-1 | Low priority |
|  |  | FR1->FR2-2 | Reuse FR2-1 | Low priority |
|  | RRC-based Active BWP Switch on multiple CCs |  | Reuse FR2-1 | Yes |
| Radio link monitoring | FR2-2 PCell, SSB-based | Out-of-sync, in non-DRX | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
|  |  | In-sync, in non-DRX mode | Can be reused from FR2-1 if RX beam sweeping is not changed | Low priority |
|  |  | Out-of-sync, in DRX mode | Can be reused from FR2-1 if RX beam sweeping is not changed | Low priority |
|  |  | In-sync, in DRX mode | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
|  | FR2-2 PCell, CSI-RS-based | Out-of-sync, in non-DRX | Can be reused from FR2-1 if RX beam sweeping is not changed | Low priority |
|  |  | In-sync, in non-DRX mode | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
|  |  | Out-of-sync, in DRX mode | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
|  |  | In-sync, in DRX mode | Can be reused from FR2-1 if RX beam sweeping is not changed | Low priority |
|  | Scheduling restrictions |  | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
| Beam failure detection and link recovery | FR2-2 PCell | SSB-based in non-DRX mode | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
|  |  | SSB-based in non-DRX mode | Can be reused from FR2-1 if RX beam sweeping is not changed | Low priority |
|  |  | CSI-RS-based in non-DRX mode | Can be reused from FR2-1 if RX beam sweeping is not changed | Low priority |
|  |  | CSI-RS -based in non-DRX mode | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
|  | Scheduling restrictions | SSB-based in non-DRX mode | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
| SCell activation/deactivation delay | SCell in FR2 | intra-band in non-DRX | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
|  |  | inter-band in non-DRX | Can be reused from FR2-1 if RX beam sweeping is not changed | Low priority |
|  | FR1+FR2 inter-band with target SCell in FR2 |  | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
| PSCell addition and release delay | Known PSCell |  | Yes FR2 configuration uses 240 kHz SSB, so new test is needed for 120 kHz SCS | No |
|  | Unknown PSCell |  | Yes FR2 configuration uses 240 kHz SSB, so new test is needed for 120 kHz SCS | Yes |
| Active TCI state switching delay | MAC-CE based | PCell in FR2-2 | Reuse FR2-1 | Yes |
|  | RRC based | PCell in FR2-2 | Reuse FR2-1 | Low priority |
| Interruptions | during measurements on deactivated NR SCC in FR2-2 |  | Reuse FR2-1 | Yes |
| Intra-frequency measurement procedure | SA event triggered reporting test without gap | under non-DRX | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
|  |  | under DRX | Can be reused from FR2-1 if RX beam sweeping is not changed | Low priority |
|  | SA event triggered reporting test with per-UE gap | under non-DRX | Can be reused from FR2-1 if RX beam sweeping is not changed | Low priority |
|  |  | under DRX | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
| Inter-frequency measurement procedure | SA event triggered reporting tests For FR2 without SSB time index detection | PCell in FR2-2, DRX is not used | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
|  |  | PCell in FR2-2, DRX is used | Can be reused from FR2-1 if RX beam sweeping is not changed | Low Priority |
|  |  | PCell in FR1, DRX is not used | Can be reused from FR2-1 if RX beam sweeping is not changed | Low Priority |
|  |  | PCell in FR1, DRX is used | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
|  | SA event triggered reporting tests For FR2 with SSB time index detection | PCell in FR2-2, DRX is not used | Can be reused from FR2-1 if RX beam sweeping is not changed | Low Priority |
|  |  | PCell in FR2-2, DRX is used | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
|  |  | PCell in FR1, DRX is not used | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
|  |  | PCell in FR1, DRX is used | Can be reused from FR2-1 if RX beam sweeping is not changed | Low Priority |
| L1-RSRP measurement for beam reporting | SSB based | DRX not used | Can be reused from FR2-1 if RX beam sweeping is not changed | Low Priority |
|  |  | DRX is used | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
|  | CSI-RS based | DRX not used | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
|  |  | DRX is used | Can be reused from FR2-1 if RX beam sweeping is not changed | Low Priority |
| Accuracy for measurements | SS-RSRP | Intra-frequency FR2-2 serving and target cell | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
|  |  | Inter frequency FR2-2 serving and target cell | Can be reused from FR2-1 if RX beam sweeping is not changed | Low Priority |
|  |  | Inter frequency FR1 serving and FR2-2 target cell | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
|  | SS-RSRQ | Intra-frequency FR2-2 serving and target cell | Can be reused from FR2-1 if RX beam sweeping is not changed | Low Priority |
|  |  | Inter frequency FR2-2 serving and target cell | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
|  | SS-SINR | Intra-frequency FR2-2 serving and target cell | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
|  |  | Inter frequency FR2-2 serving and target cell | Can be reused from FR2-1 if RX beam sweeping is not changed | Low Priority |
|  | L1-RSRP | SSB based | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
|  |  | CSI-RS based | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |

RRM performance requirements with CCA

In the past 2 RAN4 meetings, RAN4 #101-bis and RAN4 #102, the RRM requirements for operation above 52GHz with LBT were discussed. In the last meeting, the Big CR was endorsed.

In general, RAN4 has agreed to extend the RRM requirements based on DL LBT failures for sending SSBs, in a similar manner as what was done in NR-U. The difference between the framework in FR1 and FR2-2 is that in FR2-2, RAN4 agreed on the concept of a SMTC / SSB occasion group, which is a set of N SMTC/SSB occasions, where N is the RX beam sweeping factor. In case one SMTC/SSB occasion fails within a group, the requirements are extended by N SMTC/SSB occasions, up to a limit of extensions which is defined according to the specific measurement.

The definition of group is different from what was discussed in NR-U, and it is possible that RAN4 needs to discuss whether it will impact or not on the definition of the LBT model assumed in the test cases.

RAN4 to discuss whether the LBT model defined in NR-U needs to be adjusted to support the SMTC/SSB/DRX occasion group definition in FR2-2.

Regarding the test cases, in NR-U the structure of test cases was defined as follows:

A.10 EN-DC Tests with NR PSCell under CCA and Other NR Cells in FR1

A.11 NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR1

A.12 E-UTRA Standalone tests with at Least One NR Cell under CCA

A.13 NR Standalone Tests with NR Scell under CCA and All Other NR Cells in FR1

Per agreement in RAN4 #101, requirements for EN-DC and NE-DC scenarios will not be defined in this Release. Therefore, we propose:

Create in TS 38.133 the following new top-level sections for NR-test cases with CCA in FR2-2:

A.14 NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR2-2

A.15 NR Standalone Tests with NR Scell under CCA and All Other Cells in FR2-2

Similar to our discussion in the last Section for the RRM test cases without CCA, we have concerns on the workload posed by the test cases for requirements with CCA. Therefore, we believe it is reasonable that we do not duplicate test cases for requirements that are equal for operation with CCA and without CCA. Since the requirements still need to be tested in case the UE supports CCA, but not operation without CCA, instead of defining a new test case we propose to reuse the correspondent test case defined for operation without CCA.

For requirements that are the same for operation with CCA and without CCA, tests defined for operation without CCA can be reused to test the requirements with CCA.

RAN4 to discuss the list of test cases for RRM performance with CCA based on Table 3

Table 3 - Proposed RRM test case list for the operation above 52.6 GHz with CCA

| Group of requirements | Test cases | Sub-test | Needed / Not needed |
| --- | --- | --- | --- |
| RRC_IDLE, cell re-selection | FR2-2 -> FR2-2 (CCA) | intra-frequency | Yes, at least with 120 kHz and 480 kHz |
|  |  | inter-frequency | Yes, at least with 120 kHz and 480 kHz |
| HO (delay and interruptions) | FR2-2->FR2-2 (CCA) | intra-frequency, unknown target cell | Yes |
|  |  | inter-frequency, unknown target cell | Low priority |
|  | FR1->FR2-2 (CCA) | unknown target cell | Yes |
| RRC Re-establishment | FR2-2->FR2-2 (CCA) | intra frequency | Low priority |
|  |  | inter frequency | Yes |
|  |  | intra frequency without serving cell timing | Yes |
| Random access | 4-step RACH | Contention based | Yes |
|  |  | Non-contention based | Yes |
| RRC Connection Release with Redirection | FR2-2->FR2-2 (CCA) |  | Low priority |
| Timing | UE Transmit Timing |  | Yes |
|  | Timing advance adjustment accuracy |  | Yes |
| Active BWP switch | DCI-based and timer-based active BWP switch | FR2-2 ->FR2-2 (CCA) | Yes, but this test case can be same as the one defined for operation without CCA |
|  |  | FR1->FR2-2 (CCA) | Low priority |
|  |  | One FR2-2 (CCA) cell | Low priority |
|  | RRC-based active BWP switch |  | Yes, but this test case can be same as the one defined for operation without CCA |
|  | UL active BWP switch delay with consistent UL LBT failure on PCell subject to UL CCA | FR2-2 (CCA) | Yes |
|  |  | FR1->FR2-2 | Not needed |
| Radio link monitoring | FR2-2 Pcell (CCA), SSB-based | Out-of-sync, in non-DRX | Yes |
|  |  | In-sync, in non-DRX mode | Low priority |
|  |  | Out-of-sync, in DRX mode | Low priority |
|  |  | In-sync, in DRX mode | Yes |
|  | Scheduling restrictions |  | Yes, but this test case can be same as the one defined for operation without CCA |
| Beam failure detection and link recovery | FR2-2 PCell (CCA) | SSB-based in non-DRX mode | Yes |
|  |  | SSB-based in non-DRX mode | Low priority |
|  |  | CSI-RS-based in non-DRX mode | Low priority |
|  |  | CSI-RS -based in non-DRX mode | Yes |
|  | Scheduling restrictions | SSB-based in non-DRX mode | Yes, but this test case can be same as the one defined for operation without CCA |
| SCell activation/deactivation delay | SCell in FR2 (CCA) | intra-band in non-DRX | Yes |
|  |  | inter-band in non-DRX | Low priority |
|  | FR1+FR2 (CCA) inter-band with target SCell in FR2 |  | Yes |
| PSCell addition and release delay | Known PSCell |  | No |
|  | Unknown PSCell |  | Yes |
| Active TCI state switching delay | MAC-CE based | PCell in FR2-2 (CCA) | Yes |
|  | RRC based | PCell in FR2-2 (CCA) | Low priority |
| Interruptions | during measurements on deactivated NR SCC in FR2-2 |  | Yes |
| Intra-frequency measurement procedure | SA event triggered reporting test without gap | under non-DRX | Yes |
|  |  | under DRX | Low priority |
|  | SA event triggered reporting test with per-UE gap | under non-DRX | Low priority |
|  |  | under DRX | Yes |
| Inter-frequency measurement procedure | SA event triggered reporting tests For FR2 without SSB time index detection | PCell in FR2-2, DRX is not used | Yes |
|  |  | PCell in FR2-2, DRX is used | Low Priority |
|  |  | PCell in FR1, DRX is not used | Low Priority |
|  |  | PCell in FR1, DRX is used | Yes |
|  | SA event triggered reporting tests For FR2 with SSB time index detection | PCell in FR2-2, DRX is not used | Low Priority |
|  |  | PCell in FR2-2, DRX is used | Yes |
|  |  | PCell in FR1, DRX is not used | Yes |
|  |  | PCell in FR1, DRX is used | Low Priority |
| L1-RSRP measurement for beam reporting | SSB based | DRX not used | Low Priority |
|  |  | DRX is used | Yes |
| Accuracy for measurements | SS-RSRP | Intra-frequency FR2-2 serving and target cell | covered by the requirements without CCA |
|  |  | Inter frequency FR2-2 serving and target cell | covered by the requirements without CCA |
|  |  | Inter frequency FR1 serving and FR2-2 target cell | covered by the requirements without CCA |
|  | SS-RSRQ | Intra-frequency FR2-2 serving and target cell | covered by the requirements without CCA |
|  |  | Inter frequency FR2-2 serving and target cell | covered by the requirements without CCA |
|  | SS-SINR | Intra-frequency FR2-2 serving and target cell | covered by the requirements without CCA |
|  |  | Inter frequency FR2-2 serving and target cell | covered by the requirements without CCA |
|  | L1-RSRP | SSB based | covered by the requirements without CCA |
|  |  | CSI-RS based | covered by the requirements without CCA |
|  | RSSI | Intra-frequency | Needed in all SCS |
|  |  | Inter-frequency | Needed in all SCS |

Conclusion

Observation 1: Most of the existing RRM performance requirements cannot be assumed to cover the UE behaviour with 480 and 960 kHz SCS.

Proposal 1: Define RRM performance requirements for 480 kHz and 960 kHz SCS.

Observation 2: No definition on RRM core requirements was reached so far regarding Rx beam sweeping.

Observation 3: Open options for Rx beam sweeping are N=8 (unchanged) or in the range from N=12 to N=16.

Observation 4: If beam sweeping scaling factor is changed, most of the RRM performance requirements from FR2-1 with 120 kHz cannot be reused for FR2-2.

Observation 5: Some FR2-1 test cases may be reused when the configuration does not include SSB SCS which is not supported by FR2-2.

Proposal 2: Attempt to reuse FR2-1 RRM performance requirements for 120 kHz SCS that do not depend on the Rx Beam sweeping scaling factor.

Proposal 3: If Rx beam sweeping scaling factor is changed in RRM core requirements, define new RRM performance requirements for 120 khz SCS considering the decision on the number of Rx Beam for RRM requirements.

Proposal 4: RAN4 not to reuse FR2-1 RRM performance requirements for 120 kHz SCS with SSB using 240 kHz SCS.

Proposal 5: RAN4 not to reuse FR2-1 RRM performance requirements for 120 kHz SCS for test cases whose RRM core requirements changed, for example due to new Rx beam sweeping scaling factor.

Proposal 6: RAN4 to define RRM performance requirements for the following scenarios:

-FR2-2 only standalone

-FR2-2 non-contiguous CA

-FR1+FR2-2 CA (FR1 is PCell)

-FR1+FR2-2 DC (FR1 is PCell)

Proposal 7: RAN4 to use as a baseline for the RRM performance without CCA the Rel 15 test cases.

Proposal 8: RAN4 to discuss the list of test cases for RRM performance without CCA based on Table 2

Proposal 9: RAN4 to discuss whether the LBT model defined in NR-U needs to be adjusted to support the SMTC/SSB/DRX occasion group definition in FR2-2.

Proposal 10: Create in TS 38.133 the following new top-level sections for NR-test cases with CCA in FR2-2:

A.14 NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR2-2

A.15 NR Standalone Tests with NR Scell under CCA and All Other Cells in FR2-2

Proposal 11: For requirements that are the same for operation with CCA and without CCA, tests defined for operation without CCA can be reused to test the requirements with CCA.

Proposal 12: RAN4 to discuss the list of test cases for RRM performance with CCA based on Table 3

References

- RP-213540, Revised WID:  Extending current NR operation to 71 GHz, Quacomm

- R4-2108353, Work plan for NR extension to 71 GHz - RRM, Qualcomm

- R4-2206919 WF on NR extension to 71 GHz RRM requirements (Part 1) Qualcomm

- R4-2202657, WF on NR extension to 71 GHz RRM requirements (Part 1), Qualcomm

- R4-2202659, WF on NR extension to 71 GHz RRM requirements (Part 2), Intel

- R4-2120316, WF on NR extension to 71 GHz RRM requirements (Part 1), Qualcomm

- R4-2108354, WF on NR extension to 71 GHz - RRM, Qualcomm

- R4-2115351, WF on NR extension to 71 GHz – RRM - Part 1, Qualcomm

- R4-2115352, WF on NR extension to 71 GHz – RRM - Part 2, Intel

- R4- 2201787, Discussion on timing requirements for NR 52.6 – 71 GHz, Intel Corporation

- R4-2207122, Big CR: RRM requirements for Rel-17 NR extension to 71GHz, Intel Corporation, Qualcomm

- TS 38.133: “NR; Requirements for support of radio resource management”

- TS 38.533, NR; User Equipment (UE) conformance specification; Radio Resource Management (RRM)

- TS 38.211, NR; Physical channels and modulation

- TR 38.808 v17.0.0, Study on supporting NR from 52.6 GHz to 71 GHz

- R4-2120677, WF on BS RF Tx requirements, Nokia

- R4-2203016, WF on BS RF Tx requirements, Nokia

- R1-2200780, Updated RAN1 UE features list for Rel-17 NR after RAN1 #107bis-e, AT&T, NTT DOCOMO, INC.