3GPP TSG-RAN WG4 Meeting #103-e	R4-2209059

Electronic Meeting, 9 May – 20 May 2022

Agenda Item:	9.19.5.2.1

Source:	Ericsson

Title:	PDSCH demodulation requirements for RedCap

Document for:	Discussion

# 1	Introduction

RAN4#102-e agreed with WF on RedCap demodulation and CQI reporting requirements [1]. This contribution provides our initial simulation results and discusses the open issues on PDSCH demodulation requirements for RedCap.

# 2	Simulation results

Table 1 and Table 2 summarizes our initial PDSCH simulation results for FDD 15kHz and TDD 30kHz, respectively, according to the simulation assumption in [1]. Note we set TDLB100-400 instead of TDLB400-400 for FDD case to reuse the existing test setup.

Table 1	Summary of PDSCH simulation results for FDD 15kHz.

| 1Rx UE or 2Rx UE | CBW / SCS | MCS and rank | Propagation condition | Antenna configuration | Metric | Reference from TS38.101-4 Table | SNR to achieve 70% max Tput |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 10MHz / 15kHz | QPSK 1/3 Rank 1 | TDLB100-400 | 2x1 low | 70% of peak rate | 5.2.2.1.1-3 Test 1-1 | 1.0 |
| 1Rx UE | 10MHz / 15kHz | 16QAM 0.48 Rank 1 | TDLC300-100 | 2x1 low | 70% of peak rate | 5.2.2.1.1-3 Test 1-4 | 9.1 |
| 1Rx UE | 10MHz / 15kHz | 64QAM 0.50 Rank 1 | TDLA30-10 | 2x1 low | 70% of peak rate | 5.2.2.1.1-4 Test 2-1 | 13.6 |
| 2Rx UE | 10MHz / 15kHz | QPSK 1/3 Rank 1 | TDLB100-400 | 2x2 ULA low | 70% of peak rate | 5.2.2.1.1-3 Test 1-1 | Reuse |
| 2Rx UE | 10MHz / 15kHz | 16QAM 0.48 Rank 1 | TDLC300-100 | 2x2 ULA low | 70% of peak rate | 5.2.2.1.1-3 Test 1-4 | 5.1 |
| 2Rx UE | 10MHz / 15kHz | 64QAM 0.5 Rank 2 | TDLA30-10 | 2x2 ULA low | 70% of peak rate | 5.2.2.1.1-4 Test 2-1 | Reuse |

Table 2	Summary of PDSCH simulation results for TDD 30kHz.

| 1Rx UE or 2Rx UE | CBW / SCS | MCS and rank | TDD UL/DL pattern | Propagation condition | Antenna configuration | Metric | Reference from TS38.101-4 Table | SNR to achieve 70% max Tput |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 20MHz / 30kHz | QPSK 1/3 Rank 1 | FR1.30-1A | TDLB100-400 | 2x1 low | 70% of peak rate | 5.2.2.2.1-3 Test 1-1 | 0.7 |
| 1Rx UE | 20MHz / 30kHz | 16QAM 0.48 Rank 1 | FR1.30-1 | TDLC300-100 | 2x1 low | 70% of peak rate | 5.2.2.2.1-3 Test 1-4 | 9.2 |
| 1Rx UE | 20MHz / 30kHz | 64QAM 0.50 Rank 1 | FR1.30-1 | TDLA30-10 | 2x1 low | 70% of peak rate | 5.2.2.2.1-4 Test 2-1 | 13.8 |
| 2Rx UE | 20MHz / 30kHz | QPSK 1/3 Rank 1 | FR1.30-1A | TDLB100-400 | 2x2 ULA low | 70% of peak rate | 5.2.2.2.1-3 Test 1-1 | -2.8 |
| 2Rx UE | 20MHz / 30kHz | 16QAM 0.48 Rank 1 | FR1.30-1 | TDLC300-100 | 2x2 ULA low | 70% of peak rate | 5.2.2.2.1-3 Test 1-4 | 4.9 |
| 2Rx UE | 20MHz / 30kHz | 64QAM 0.50 Rank 2 | FR1.30-1 | TDLA30-10 | 2x2 ULA low | 70% of peak rate | 5.2.2.2.1-4 Test 2-1 | 16.5 |

# 3	256QAM PDSCH requirements

One of the open issues on RedCap UE demodulation requirements is whether to specify 256QAM demodulation requirements or not.

- Option 1: Specify 256QAM demodulation requirements for FR1 only - Candidate test setup: - TS38.101-4 5.2.2.1.1-3 Test 1-3 for FDD 15kHz - TS38.101-4 5.2.2.2.1-3 Test 1-3 for TDD 30kHz - FFS the MCS for 1Rx case - Option 2: Not to specify 256QAM demodulation requirements. - Option 3: Discuss whether to specify the requirements for FR1 once we stabilize the discussion on requirements for mandatory features.

| 1Rx UE or 2Rx UE | CBW / SCS | MCS and rank | Propagation condition | Antenna configuration | Metric | Reference from TS38.101-4 Table | New / Reuse (if specified) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 10MHz / 15kHz | 256QAM MCS24 Rank 1 | TDLA30-10 | 2x1 low | 70% of peak rate | 5.2.2.1.1-3 Test 1-2 | New |
| 2Rx UE | 10MHz / 15kHz | 256QAM MCS24 Rank 1 | TDLA30-10 | 2x2 low | 70% of peak rate | 5.2.2.1.1-3 Test 1-2 | Reuse |
| 1Rx UE | 20MHz / 30kHz | 256QAM MCS24 Rank 1 | TDLA30-10 | 2x1 low | 70% of peak rate | 5.2.2.1.1-3 Test 1-2 | New |
| 2Rx UE | 20MHz / 30kHz | 256QAM MCS24 Rank 1 | TDLA30-10 | 2x2 low | 70% of peak rate | 5.2.2.1.1-3 Test 1-2 | New |

Table 3 shows the 256QAM PDSCH simulation results assuming RedCap UE, where we simulated MCS22, 23 and 24 for 1Rx case according to WF. According to the simulation results, the ideal results with MCS24 exceeds 25dB for 1Rx. If we consider the RF margins, the expected final requirements could be 28dB or more. Although it is feasible from the conformance test point of view, but we think RAN4 should consider lower MCS like MCS22 which gives more reasonable SNR level.

Table 3	Ideal PDSCH simulation results with 256QAM for 1Rx/2Rx RedCap UEs.

|  | MCS22 | MCS23 | MCS24 |
| --- | --- | --- | --- |
| FDD SCS=15kHz, 1Rx | 22.9 | 24.4 | 25.3 |
| FDD SCS=15kHz, 2Rx (reference) |  |  | 20.8 |
| TDD SCS=30kHz, 1Rx | 23.6 | 24.7 | 25.8 |
| TDD SCS=30kHz, 2Rx |  |  | 21.1 |

Proposal 1: If RAN4 defines 256QAM demodulation requirements for RedCap UE in FR1, set MCS22 for 1Rx case.

# 4	Summary

Proposal 1: If RAN4 defines 256QAM demodulation requirements for RedCap UE in FR1, set MCS22 for 1Rx case.

# References

- R4-2207206 “WF on RedCap demodulation and CQI reporting requirements”, Ericsson.