3GPP TSG-RAN WG4 Meeting #103-e	R4-2209073

Electronic Meeting, 9 May – 20 May 2022

Agenda Item:	10.8.6.2.1

Source:	Ericsson

Title:	UE demodulation requirements for Rel-17 NB-IoT

Document for:	Discussion

# 1	Introduction

RAN4#102-e agreed with the way forward on UE demodulation requirements for Rel-17 NB-IoT [1], where RAN4 agreed to define NPDSCH demodulation requirements with 16QAM and CQI reporting requirements. This contribution discusses the open issues on the UE demodulation requirements.

# 2	NPDSCH demodulation requirements

The remaining open issue for NPDSCH demodulation requirements is the TBS for the requirements.

TBS - Option 1: Use (ITBS, ISF) = (16, 5) which corresponds to TBS = 1928bits and effective code rate 0.51 - Option 2: Use (ITBS, ISF) = (21, 7) which corresponds to TBS = 4968bits and effective code rate 0.78

Figure 1 shows our simulation results for NPDSCH with two options and Table 1 summarizes the SNR values to achieve 70% of the maximum throughput. As expected, it is observed the TBS=4968bits requires higher SNR compared with TBS=1928bits (18dB vs 12.9dB).

Figure 1	NPDSCH simulation results.

Table 1	SNR values to achieve 70% of the maximum throughput.

|  | Option 1: (ITBS, ISF) = (16, 5) | Option 2: (ITBS, ISF) = (21, 7) |
| --- | --- | --- |
| SNR to achieve 70% of maximum throughput | 12.9dB | 18.0dB |

Although both the required SNR levels are feasible from the UE demodulation requirement point of view even if we consider the impairment margin, we prefer to set TBS=1928bits. One reason is the required SNR is reasonable compared with TBS=4968bits requiring very high SNR levels compared with the existing NB-IoT and LTE-MTC UE demodulation requirements. Another reason is the TBS=1928 requires less test time because of smaller ISF. Since we believe both the configurations can verify UE demodulation performance for NPDSCH with 16QAM, we propose to define NPDSCH requirements with TBS=1928bits.

Proposal 1: RAN4 define NPDSCH demodulation requirements with 16QAM with (ITBS, ISF) = (16, 5) which corresponds to TBS = 1928 bits.

# 3	CQI reporting requirements

The remaining open issue for CQI reporting test is the test metric.

Option 1: - The reported candidateRep value shall be in the range of +/- 1 of the reported median more than 90% of the time. - If the NPDSCH BLER using the transport format indicated by median candidateRep value is less than or equal to 0.1, the BLER using the transport format indicated by the (median candidateRep value + 1) shall be greater than 0.1. - If the NPDSCH BLER using the transport format indicated by the median candidateRep value is greater than 0.1, the BLER using transport format indicated by (median candidateRep value – 1) shall be less than or equal to 0.1. Option 2: - The reported candidateRep value shall be in the range of +/- X of the reported median more than Y% of the time. - If the NPDSCH BLER using the transport format indicated by median candidateRep value is less than or equal to 0.1, the BLER using the transport format indicated by the (median candidateRep value + 1) shall be greater than 0.1. - If the NPDSCH BLER using the transport format indicated by the median candidateRep value is greater than 0.1, the BLER using transport format indicated by (median candidateRep value – 1) shall be less than or equal to 0.1. - X is FFS and is derived based on the simulation results. Baseline: X=1 - Y is FFS and is derived based on the simulation results. Baseline: Y=90%.

Figure 2 (a) shows the median of reported CQI in the static condition and Figure 2 (b) shows the NPDSCH BLER with fixed CQI index. According to the way forward, RAN4 will define the requirements for SNR corresponding to candidates K to N. From our simulation results the candidate SNR test point range is 9dB to 15dB.

Observation 1: Candidate SNR test points are between 9dB and 15dB for NB-IoT CQI reporting tests with 16QAM.

| (a) Reported median CQI index (CQI10: Candidate-J, CQI11: Candidate-K, CQI12: Candidate-L, CQI13: Candidate-M, CQI14: Candidate-N, CQI15: Candidate-O). | (b) NPDSCH BLER for fixed CQI index. |
| --- | --- |

Figure 2	Reported median CQI index and NPDSCH BLER for fixed CQI index.

Figure 3 shows the CDF of the estimated SINR with 1 NRS ports in static condition, where we choose SNR=12dB for evaluation according to our observation. As we discussed in RAN4#102-e, the available NRS symbols are 8 per subframe in the case of 1 NRS port. We have also studied the measurement improvement with the number of subframes used for CQI estimation. Figure 3 also shows the CDF with 3, 5, 7 and 9 subframes in addition to 1 subframe assumption.

Figure 3	CDF of estimated SINR [dB] with 1 NRS ports at SNR=12dB in AWGN.

It is observed from the simulation results that the measurement accuracy is +/-4dB with 1 subframe measurement if we consider 5%/95%-ile. With the measurement with 3 subframes, the accuracy is improved to +/-2dB. The existing CQI reporting requirements in static condition requires to report {median CQI, median CQI+1 and median CQI-1} with 90% of the time. However if UE estimates CQI only with 1 subframe, we think it is difficult to satisfy the requirements.

Observation 2: UE may not satisfy the requirements to report {median CQI, median CQI-1, median CQI+1} with 90% of the time with the measurement of 1 subframe.

We therefore propose to consider following options:

Option 1: Relax the Y, e.g., 70%.

Option 2: Assume multiple subframes to estimate CQI.

We prefer to combine both options, that is, set slight relaxed Y (e.g., 85%) with the assumption of multiple subframes (e.g., 3 subframes).

Proposal 2: Set X=1, Y=85% for the metric of CQI reporting test for NB-IoT. The test setup ensures UE can measure at least 3 NB-IoT DL subframes from the time UE finishes the decoding of Downlink Channel Quality report MAC CE to the end of NPDCCH period which carries the uplink grant of channel quality report for measurement of DL channel quality of the configured carrier, but it is up to UE how many subframes are used for CQI estimation.

# 4	Summary

Proposal 1: RAN4 define NPDSCH demodulation requirements with 16QAM with (ITBS, ISF) = (16, 5) which corresponds to TBS = 1928 bits.

Observation 1: Candidate SNR test points are between 9dB and 15dB for NB-IoT CQI reporting tests with 16QAM.

Observation 2: UE may not satisfy the requirements to report {median CQI, median CQI-1, median CQI+1} with 90% of the time with the measurement of 1 subframe.

Proposal 2: Set X=1, Y=85% for the metric of CQI reporting test for NB-IoT. The test setup ensures UE can measure at least 3 NB-IoT DL subframes from the time UE finishes the decoding of Downlink Channel Quality report MAC CE to the end of NPDCCH period which carries the uplink grant of channel quality report for measurement of DL channel quality of the configured carrier, but it is up to UE how many subframes are used for CQI estimation.

# References

- R4-2207200, “Way forward for performance requirements of Rel-17 NB-IOT and eMTC”, Huawei, HiSilicon.

# Appendix

## A.1	TBS table for NPDSCH with 16QAM

Table 2	TBS table applicable for NPDSCH with 16QAM.

|  | ISF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ITBS | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | NPSDCH with 16QAM |  |
| 0 | 16 | 32 | 56 | 88 | 120 | 152 | 208 | 256 |  |  |
| 1 | 24 | 56 | 88 | 144 | 176 | 208 | 256 | 344 |  |  |
| 2 | 32 | 72 | 144 | 176 | 208 | 256 | 328 | 424 |  |  |
| 3 | 40 | 104 | 176 | 208 | 256 | 328 | 440 | 568 |  |  |
| 4 | 56 | 120 | 208 | 256 | 328 | 408 | 552 | 680 |  |  |
| 5 | 72 | 144 | 224 | 328 | 424 | 504 | 680 | 872 |  |  |
| 6 | 88 | 176 | 256 | 392 | 504 | 600 | 808 | 1032 |  |  |
| 7 | 104 | 224 | 328 | 472 | 584 | 680 | 968 | 1224 |  |  |
| 8 | 120 | 256 | 392 | 536 | 680 | 808 | 1096 | 1352 |  |  |
| 9 | 136 | 296 | 456 | 616 | 776 | 936 | 1256 | 1544 |  |  |
| 10 | 144 | 328 | 504 | 680 | 872 | 1032 | 1384 | 1736 |  |  |
| 11 | 176 | 376 | 584 | 776 | 1000 | 1192 | 1608 | 2024 | IB |  |
| 12 | 208 | 440 | 680 | 904 | 1128 | 1352 | 1800 | 2280 | IB |  |
| 13 | 224 | 488 | 744 | 1032 | 1256 | 1544 | 2024 | 2536 | IB |  |
| 14 | 256 | 552 | 840 | 1128 | 1416 | 1736 | 2280 | 2856 | IB | GB/SA |
| 15 | 280 | 600 | 904 | 1224 | 1544 | 1800 | 2472 | 3112 | IB | GB/SA |
| 16 | 296 | 632 | 968 | 1288 | 1608 | 1928 | 2600 | 3240 | IB | GB/SA |
| 17 | 336 | 696 | 1064 | 1416 | 1800 | 2152 | 2856 | 3624 | IB | GB/SA |
| 18 | 376 | 776 | 1160 | 1544 | 1992 | 2344 | 3112 | 4008 |  | GB/SA |
| 19 | 408 | 840 | 1288 | 1736 | 2152 | 2600 | 3496 | 4264 |  | GB/SA |
| 20 | 440 | 904 | 1384 | 1864 | 2344 | 2792 | 3752 | 4584 |  | GB/SA |
| 21 | 488 | 1000 | 1480 | 1992 | 2472 | 2984 | 4008 | 4968 |  | GB/SA |

## A.2	CQI table for NB-IoT supporting 16QAM

Table 3	CQI table for NB-IoT supporting 16QAM (TS36.133 Table 9.1.22.17)

|  |  | NPDCCH repetition level | NPDSCH transport block |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Reported value |  | Modulation | Code rate (x 1024) | Repetition | Efficiency |
| 0 | noMeasurement | No measurement reporting |  |  |  |  |
| 1 | candidateRep-A | 1 | QPSK | 221 | 1 | 0.4316 |
| 2 | candidateRep-B | 2 | QPSK | 140 | 1 | 0.2737 |
| 3 | candidateRep-C | 4 | QPSK | 81 | 1 | 0.1579 |
| 4 | candidateRep-D | 8 | QPSK | 81 | 2 | 0.0789 |
| 5 | candidateRep-E | 16 | QPSK | 81 | 4 | 0.0395 |
| 6 | candidateRep-F | 32 | QPSK | 81 | 8 | 0.0198 |
| 7 | candidateRep-G | 1 | QPSK | 336.8 | 1 | 0.6579 |
| 8 | candidateRep-H | 1 | QPSK | 453.6 | 1 | 0.886 |
| 9 | candidateRep-I | 1 | QPSK | 579.4 | 1 | 1.1316 |
| 10 | candidateRep-J | 1 | QPSK | 759 | 1 | 1.4825 |
| 11 | candidateRep-K | 1 | 16QAM | 487.3 | 1 | 1.9035 |
| 12 | candidateRep-L | 1 | 16QAM | 541.2 | 1 | 2.114 |
| 13 | candidateRep-M | 1 | 16QAM | 658 | 1 | 2.5702 |
| 14 | candidateRep-N | 1 | 16QAM | 783.7 | 1 | 3.0614 |
| 15 | candidateRep-O | 1 | 16QAM | 837.6 | 1 | 3.2719 |