3GPP TSG-RAN WG4 Meeting#103-e                	R4-2209086

Electronic Meeting, 9 – 20 May 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	On RRM Tests for Rel-17 FR1 HST scenarios

Agenda item:	9.7.2

Document for:	Discussion

Introduction

In this contribution, we discuss the necessary test cases for RRM requirements which are enhanced Rel-17 FR1 HST scenarios with CA for a UE speed up to 500 km/h.

Discussion

In Table 1, we list relevant test cases along with configuration parameters.

Table 1: Proposed list of test cases and associated parameter settings

| Core requirement | Test case | Parameter |
| --- | --- | --- |
| Interfrequency cell reselection in idle mode | Interfrequency cell reselection | DRX cycle = 640 ms SMTC period = 20 ms Propagation condition: AWGN (serving cell) and AWGN with 3334 Hz frequency offset (neighbour cell) |
| Interfrequency measurements in connected mode: - interfrequency measurements without gaps - interfrequency measurements with gaps | - Interfrequency event triggered reporting tests without SSB time index under DRX (with and without gaps) - Interfrequency event triggered reporting test with SSB time index under DRX (with and without gaps) | DRX cycle = FFS SMTC period = 20 ms Propagation condition: AWGN (serving cell) and AWGN with 3334 Hz frequency offset (neighbour cell) |
| Intrafrequency measurements in connected mode SCells: - intrafrequency measurements without gaps - intrafrequency measurements with gaps | - Intrafrequency event triggered reporting tests under deactivated SCell without SSB time index under DRX without gaps - Intrafrequency event triggered reporting tests under deactivated SCells with SSB time index under DRX without gaps | DRX = FFS SMTC period = 20 ms Propagation condition: AWGN (serving cell) and AWGN with 3334 Hz frequency offset (neighbour cell) |
| L1-RSRP Reporting | SSB and CSI-RS based L1-RSRP measurement when DRX is used | DRX cycle = 40 ms SMTC period = 20 ms Propagation condition: AWGN with 3334 Hz frequency offset |

Conclusion

In this contribution, we have discussed and presented our views on the necessary tests for RRM enhancements for FR1 HST scenarios with CA. Our proposal is as follows:

RAN4 discusses and agrees with the proposed list of RRM tests in the following for the RRM enhancements for FR1 HST with CA.

| Core requirement | Test case | Parameter |
| --- | --- | --- |
| Interfrequency cell reselection in idle mode | Interfrequency cell reselection | DRX cycle = 640 ms SMTC period = 20 ms Propagation condition: AWGN (serving cell) and AWGN with 3334 Hz frequency offset (neighbour cell) |
| Interfrequency measurements in connected mode: - interfrequency measurements without gaps - interfrequency measurements with gaps | - Interfrequency event triggered reporting tests without SSB time index under DRX (with and without gaps) - Interfrequency event triggered reporting test with time index under DRX (with and without gaps) | DRX cycle = FFS SMTC period = 20 ms Propagation condition: AWGN (serving cell) and AWGN with 3334 Hz frequency offset (neighbour cell) |
| Intrafrequency measurements in connected mode SCells: - intrafrequency measurements without gaps - intrafrequency measurements with gaps | - Intrafrequency event triggered reporting tests without SSB time index under DRX without gaps (deactivated SCells) - Intrafrequency event triggered reporting tests with SSB time index under DRX without gaps (deactivated SCells) | DRX = FFS SMTC period = 20 ms Propagation condition: AWGN (serving cell) and AWGN with 3334 Hz frequency offset (neighbour cell) |
| L1-RSRP Reporting | SSB and CSI-RS based L1-RSRP measurement when DRX is used | DRX cycle = 40 ms SMTC period = 20 ms Propagation condition: AWGN with 3334 Hz frequency offset |

References