3GPP TSG-RAN4 Meeting #103-e	R4-2209236

# Electronic Meeting, 9 – 20 May, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 2368 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on requirements for UE Rx-Tx measurement for PDC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IIOT_URLLC_enh-Core |  |  |  |  |  | Date: |  |  | 2022-04-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There are some issues with PDC measurement requirements: - Sample number for TRS PDC is missing. - Impacts of MG is not considered. - Impacts of Rx beam for FR2 is not considered - DRX requirements are missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the PDC measurement requirements: - Add the sample number for TRS PDC. - Add a scaling factor to account resource occasions punctured by MG. - Add scheduling and measuremetn restriction for FR2 PDC measurement. - Add DRX requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The PDC measurement requirements are incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.12.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

9.12.4	Measurement period requirements

9.12.4.1 PRS Measurement Period

The UE shall be able to measure UE Rx-Tx time difference on PCell after receiving [TBD, command from network that triggers the UE Rx-Tx measurement] within TUERx-Tx_PRS, where

TUERxTx, PRS=Kgap*NPRSslotN'*Lavailable_PRSN*(Nsample-1)*Teffect+Tlast

where

Lavailable_PRS is the time duration of available PRS resources to be measured during TPRS, and is calculated in the same way as PRS duration K defined in [TBD].

NPRSslot is the maximum number of DL PRS resources configured in a slot,

{N,T} is UE capability combination per band where N is a duration of DL PRS symbols in ms corresponding to [durationOfPRS-ProcessingSysmbols] in [TBD] processed every T ms corresponding to [durationOfPRS-ProcessingSymbolsInEveryTms] in [TBD] for a given maximum bandwidth supported by UE corresponding to [supportedBandwidthPRS ] in [TBD],

N’ is UE capability for number of DL PRS resources that it can process in a slot corresponding to [maxNumOfDL-PRS-ResProcessedPerSlot] as specified in [TBD],

Nsample is the number of UE Rx-Tx time difference measurement samples and Nsample= 4,

Kp is the scaling factor for an SSB frequency layer to be measured without measurement gaps.

Kp = Ntotal / Navailable, where Navailable and Ntotal are calculated as follows:

-	For a window W of duration max(PRS period,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gap and/or per-FR measurement gap within the same FR as the PRS frequency layer, and starting from the beginning of any PRS occasion:

-	Ntotal is the total number of PRS occasions within the window, including those overlapped with measurement gap occasions within the window, and

-	Navailable is the number of PRS occasions that are not overlapped with any non-dropped MG occasion within the window W, after accounting for measurement gap collisions by applying the measurement gap collision rule in section 9.1.2B.3.

Kp = 1 when Navailable = 0.

- Tlast is the measurement duration for the last UE Rx-Tx time difference measurement, including the sampling time and processing time, Tlast = T + TPRS ,

Teffect is periodicity of UE Rx-Tx time difference measurement:

When no DRX is in use:

Teffect= TTPRS*TPRS

When DRX is in use:

Teffect= TmaxTPRS,TDRX*maxTPRS,TDRX

where

T	corresponds to [durationOfPRS-ProcessingSymbolsInEveryTms] defined in [TBD],

TPRS is the PRS resource periodicity specific for PDC RTT UE Rx-Tx time difference measurement,

TDRX is the DRX cycle if DRX is in use, TDRX is zero otherwise.

Note: the PRS measurement period TUERxTx, PRS can be revisited based on further agreement from RAN1/2 and further RAN4 discussion.

UE is only required to perform UE Rx-Tx time difference on PRS within the active DL BWP.

When UE is configured to perform UE Rx-Tx time difference measurement based on PRS, the requirements apply provided that the SCS of the PRS is same as that of the active BWP on PCell.

For FR2, the requirements apply if QCL information is provided for all the PRS resources in the resource set and for each resource one RS has QCL-TypeD.

9.12.4.2 TRS Measurement Period

The UE shall be able to measure UE Rx-Tx time difference on PCell after receving [TBD, command from network that triggers the UE Rx-Tx measurement] within TUERx-Tx_TRS, where

TUERxTx,TRS=Kgap*Nsample *maxTTRS,TDRXTTRS

Where

-	Nsample is the number of UE Rx-Tx time difference measurement samples and Nsample is 4,

-	TTRS is the TRS resource periodicity specific for PDC RTT UE Rx-Tx time difference measurement,

-	TDRX is the DRX cycle.

-	Kp is the scaling factor for an SSB frequency layer to be measured without measurement gaps.

Kp = Ntotal / Navailable, where Navailable and Ntotal are calculated as follows:

-	For a window W of duration max(PRS period,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gap and/or per-FR measurement gap within the same FR as the PRS frequency layer, and starting from the beginning of any PRS occasion:

-	Ntotal is the total number of PRS occasions within the window, including those overlapped with measurement gap occasions within the window, and

-	Navailable is the number of PRS occasions that are not overlapped with any non-dropped MG occasion within the window W, after accounting for measurement gap collisions by applying the measurement gap collision rule in section 9.1.2B.3.

Kp = 1 when Navailable = 0.

Note: the TRS measurement period TUERx-Tx_TRS can be revisited based on further agreement from RAN1/2 and further RAN4 discussion.

UE is only required to perform UE Rx-Tx time difference on TRS within the active DL BWP.

When UE is configured to perform UE Rx-Tx time difference measurement based on TRS, the requirements apply provided that the SCS of the TRS is same as that of the active BWP on PCell.

For FR2, the requirements apply if QCL information is provided for all the PRS resources in the resource set and for each resource one RS has QCL-TypeD.

9.12.4.3 Schedulign availability during measurement for Propagation Delay Compensation

During measurement for Propagation Delay Compensation in an FR2 cell,

-	For the case where the PRS or TRS for PDC measurement is QCLed with active TCI state for PDCCH/PDSCH, there are no scheduling restrictions due to PDC measurement performed based on PRS or TRS.

-	Otherwise, the UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on symbols corresponding to the PRS or TRS resource configured for PDC measurement.

9.12.4.4 Measurement restriction for measurement for Propagation Delay Compensation

During measurement for Propagation Delay Compensation in an FR2 cell, when the PRS or TRS for PDC measurement on one CC is in the same OFDM symbol as SSB or CSI-RS for RLM, BFD, CBD or L1-RSRP measurement on the same CC or different CCs in the same band, UE is required to measure one of but not both PRS or TRS for PDC measurement and the other RS if they are not QCL-ed w.r.t. QCL-TypeD, or the QCL information is not known to UE. Longer measurement period for PRS or TRS for PDC measurement is expected, and no requirements are defined.

<End of Change 1>