3GPP TSG-RAN WG4 Meeting #103-e	R4-2209304

# Electronic Meeting, 9 - 20 May, 2022

Source:	Huawei, HiSilicon

Title:	TP for 37.717-00-00 CA_n79A_SUL_n41A-n97A

Agenda item:	8.19.2

Document for:	Approval

1 Background

This contribution provides updated text proposal on the NR SUL band combination CA_n79A_SUL_n41A-n97A.

2 Text Proposal

---Start of changes---

## 5.X	CA_n79_SUL_n41-n97

### 5.X.1	Operating bands

Table 5.X.1-1: SUL band combination

| NR Band combination for SUL | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n79_SUL_n41-n97 | n41, n79, n97 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory. |  |

### 5.X.2	Channel bandwidths per operating band

Table 5.X.2-1: Supported bandwidths per SUL band combination

| SUL band combination with CA | UL configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n79A_SUL_n41A-n97A | SUL_n41A-n97A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n97 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |

### 5.X.3	Maximum output power

There is only single UL in uplink so this requirement has been specified in spec.

### 5.X.4	Spurious emission band UE co-existence

There is only single UL in uplink so this requirement has been specified in spec. Harmonic interference analysis was covered by fallback combinations.

### 5.X.5	REFSENS requirements

For SUL operation with downlink CA, the reference receive sensitivity (REFSENS) requirement for downlink bands specified in clause 7.3A.2 from TS 38.101-1 shall be met when supplementary uplink configuration for reference sensitivity are specified in Table 7.3C.2-1 from TS 38.101-1.

Since there are two downlink bands, MSD studies are covered by fallback band combinations.

### 5.X.6	∆TIB and ∆RIB values

For CA_n79_SUL_n41-n97, the TIB,c and RIB,c values are given in the tables below.

Table 5.X.6-1: ΔTIB,c

| SUL Band combination | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n79_SUL_n41-n97 | n41 | 0.5 |
|  | n79 | 0.5 |
|  | n97 | 0.5 |

Table 5.X.6-2: ΔRIB,c

| SUL Band combination | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n79_SUL_n41-n97 | n41 | 0 |
|  | n79 | 0.8 |

---End of changes---

Reference

[1]	RP-220695, “Revised WID: Band combinations for SA NR supplementary uplink (SUL), NSA NR SUL, NSA NR SUL with UL sharing from the UE perspective (ULSUP)”, Huawei, HiSilicon