3GPP TSG-RAN WG4 Meeting # 103-e                	R4-2209391

Electronic Meeting, February 21 – March 3, 2022

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Discussion on PUCCH demodulation requirements for the extension to 71 GHz

Agenda item: 	9.15.10.3.2

Document for: 	Discussion

Introduction

This paper presents Nokia’s view on PUCCH demodulation requirements related to the operation between 52.6 GHz and 71 GHz.

Discussion

PUCCH format 0

On the last RAN4 meeting we reached agreement on PUCCH requirements were as follow [1]:

Issue 2-4-1: Scope of PUCCH performance requirements Define performance requirements for the enhanced PUCCH formats 0, 1, and 4. Define new requirements for PUCCH formats 2 and 3. Issue 2-4-2a: Detailed PUCCH test setup for PUCCH format 0 Number of PRBs: - Option 1: 16 (enhanced format) - Option 2: 10 (enhanced format) Number of OFDM symbols: - Option 1: 1 - Option 2: 1 and 2 Intra-slot frequency hopping: enabled if number of OFDM symbols higher than 1 Test metric: ACK missed detection rate < 1%

It is our view that only one test should suffice for the performance requirements of PUCCH format 0. From our simulation results 10 PRBs with 2 OFDM symbols provide better performance in comparison to 16 PRBs with 1 OFDM symbol. Therefore we would suggest to choose

1 test case is enough for covering PUCCH format 0 performance requirements.

RAN4 to define enhanced PUCCH format 0 with 10 PRBs and 2 OFDM symbols for 120, 480, and 960 kHz SCS.

Proposal 2: RAN4 to adopt the following parameters for enhanced PUCCH format 0 simulations:

| Parameter | Value |
| --- | --- |
| TR/RX | 1T2R |
| Number of UCI information bits | 1 |
| SCS | 120 kHz 480 kHz 960 kHz |
| Number of PRBs | 10 |
| Number of OFDM symbols | 2 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | Enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs - 1) |
| Group and sequence hopping | Neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 13 for 1 symbol |
|  | 12 for 2 symbols |
| Propagation condition | TDL-A with delay spread 5ns ;10 ns |
| SCS and BW | SCS = 120kHz; CBW = 100 MHz SCS = 480 kHz; CBW = 400 MHz SCS = 960 kHz; CBW = 400 MHz |
| Testing metric | Test metric: ACK missed detection rate < 1% |
| Phase noise model | TR 38.808 set 2 |

PUCCH format 1

The latest agreements in relation to PUCCH format 1 are [1]:

Issue 2-4-2b: Detailed PUCCH test setup for PUCCH format 1 Number of PRBs: - Option 1: 1 - Option 2: 16 (enhanced format) - Option 3: 10 (enhanced format) Number of OFDM symbols: - Option 1: 14 Intra-slot frequency hopping: enabled

RAN4 to define enhanced PUCCH format 1 requirements with 10 PRBs, and 2 UCI information bits.

RAN4 to adopt the following table of parameters for PUCCH format 1:

| Parameter | Value |
| --- | --- |
| TR/RX | 1T2R |
| Number of UCI information bits | 2 |
| Number of PRBs and Number of OFDM symbols: | Number of PRBs: 1 and 16 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | Enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs - 1) |
| Group and sequence hopping | Neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Propagation condition | TDL channel model: TDL-A with delay spread 5ns ;10 ns |
| SCS and BW | SCS = 120kHz; CBW = 100 MHz SCS = 480 kHz; CBW = 400 MHz SCS = 960 kHz; CBW = 400 MHz |
| Testing metric | ACK missed detection rate < 1% NACK to ACK (0.1 %) |

Conclusion

This paper has presented Nokia’s views on PUSCH demodulation requirements for the extension to 71 GHz. From this discussion we have derived the following observations and proposals:

- 1 test case is enough for covering PUCCH format 0 performance requirements.

- RAN4 to define enhanced PUCCH format 0 with 10 PRBs and 2 OFDM symbols for 120, 480, and 960 kHz SCS.

Proposal 2: RAN4 to adopt the following parameters for enhanced PUCCH format 0 simulations:

| Parameter | Value |
| --- | --- |
| TR/RX | 1T2R |
| Number of UCI information bits | 1 |
| SCS | 120 kHz 480 kHz 960 kHz |
| Number of PRBs | 10 |
| Number of OFDM symbols | 2 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | Enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs - 1) |
| Group and sequence hopping | Neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 13 for 1 symbol |
|  | 12 for 2 symbols |
| Propagation condition | TDL-A with delay spread 5ns ;10 ns |
| SCS and BW | SCS = 120kHz; CBW = 100 MHz SCS = 480 kHz; CBW = 400 MHz SCS = 960 kHz; CBW = 400 MHz |
| Testing metric | Test metric: ACK missed detection rate < 1% |
| Phase noise model | TR 38.808 set 2 |

RAN4 to define enhanced PUCCH format 1 requirements with 10 PRBs, and 2 UCI information bits.

RAN4 to adopt the following table of parameters for PUCCH format 1:

| Parameter | Value |
| --- | --- |
| TR/RX | 1T2R |
| Number of UCI information bits | 2 |
| Number of PRBs and Number of OFDM symbols: | Number of PRBs: 1 and 16 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | Enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs - 1) |
| Group and sequence hopping | Neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Propagation condition | TDL channel model: TDL-A with delay spread 5ns ;10 ns |
| SCS and BW | SCS = 120kHz; CBW = 100 MHz SCS = 480 kHz; CBW = 400 MHz SCS = 960 kHz; CBW = 400 MHz |
| Testing metric | ACK missed detection rate < 1% NACK to ACK (0.1 %) |

References

- R4-2207223, WF on demodulation performance requirements definition for 52.6 - 71 GHz, Intel Corporation