3GPP TSG-RAN WG4 Meeting #103-e		R4-2209404

Electronic Meeting, 9 - 20 May, 2022

Agenda item:	9.11.2.3.2

Source:	China Telecom

Title:	Simulation assumptions for CRS-IM (for 15kHz FDD and TDD)

Document for:	Approval

Introduction

In this paper we provide detailed simulation assumptions for CRS-IM (for 15kHz FDD and TDD). Based on the agreed WF in [1].

Simulation assumptions for CRS-IM under scenario 1

Scenario 1: Serving and interference cells are both operating in DSS (NR+LTE) mode.

Table 2-1:	15kHz SCS simulation assumptions for CRS-IM under scenario 1 (Serving cell configuration)

| Parameter |  | Unit | FDD 15kHz SCS Case | TDD 15kHz SCS Case |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 | 20 |
| Subcarrier spacing |  | kHz | 15 | 15 |
| TDD UL DL pattern |  |  | N/A | DDDSU S = 10D+2G+2U |
| Rank |  |  | 1 | 1 |
| MCS |  |  | 13 | 13 |
| MCS table |  |  | Table 1 | Table 1 |
| Duplex mode |  |  | FDD | TDD |
| Active DL BWP index |  |  | 1 | 1 |
| NR UL transmission with a 7.5 kHz shift to the LTE raster |  |  | true | true |
| Propagation channel |  |  | TDLA 30-10 | TDLA 30-10 |
| Antenna configuration |  |  | ULA Low 2 x 2 and 2 x 4 4 x 2 and 4 x 4 | ULA Low 2 x 2 and 2 x 4 4 x 2 and 4 x 4 |
| Receiver Type |  |  | MMSE-IRC | MMSE-IRC |
| Overhead for TBS determination |  |  | [12 for 2Tx cases] 18 for 4Tx cases | [12 for 2Tx cases] 18 for 4Tx cases |
| PDCCH configuration | Symbols with PDCCH |  | Symbol# 2 | Symbol# 2 |
| PDSCH configuration | Mapping type |  | Type A | Type A |
|  | k0 |  | 0 | 0 |
|  | Starting symbol (S) |  | 3 | 3 |
|  | Length (L) |  | 9 | 9 |
|  | PDSCH aggregation factor |  | 1 | 1 |
|  | PRB bundling type |  | Static | Static |
|  | PRB bundling size |  | 2 | 2 |
|  | Resource allocation type |  | Type 0 | Type 0 |
|  | RBG size |  | Config2 | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 | Type 1 |
|  | Position of the first DM-RS for downlink |  | 3 | 3 |
|  | Number of additional DMRS |  | 1 | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 | 1 |
| Serving cell CRS rate matching (Note 1) | LTE carrier centre subcarrier location |  | Same as NR carrier centre subcarrier location | Same as NR carrier centre subcarrier location |
|  | LTE carrier BW | MHz | 10 | 20 |
|  | Number of antenna ports |  | 2 for 2Tx cases 4 for 4Tx cases | 2 for 2Tx cases 4 for 4Tx cases |
|  | v-shift |  | 0 | 0 |
| Number of HARQ Processes |  |  | 4 | 8 |
| Max HARQ re-transmission number |  |  | 4 | 4 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |
| PDSCH & PDSCH DMRS Precoding configuration |  |  | Single Panel Type I, Random precoder selection updated per slot with PRB bundling granularity | Single Panel Type I, Random precoder selection updated per slot with PRB bundling granularity |
| LTE interference cell configuration |  |  | Specified in Table 2-2 | Specified in Table 2-3 |
| Reference channel |  |  | ‘FDD Sce1 2Tx’ for 2Tx cases ‘FDD Sce1 4Tx’ for 4Tx cases | ‘TDD Sce1 2Tx’ for 2Tx cases ‘TDD Sce1 4Tx’ for 4Tx cases |
| Note 1: No MBSFN is configured on LTE carrier |  |  |  |  |

Table 2-2:	FDD 15kHz SCS simulation assumptions for CRS-IM under scenario 1 (LTE interference cells configuration)

| Parameter |  | Value | Interference Cell #1 | Interference Cell #2 |
| --- | --- | --- | --- | --- |
| Transmission mode |  |  | TM4 | TM4 |
| Interference power level |  | dB | INR1=10.45dB | INR2=4.6dB |
| CRS pattern | LTE carrier centre subcarrier location |  | Same as NR serving carrier centre subcarrier location | Same as NR serving carrier centre subcarrier location |
|  | LTE carrier BW | MHz | 10 | 10 |
|  | Number of antenna ports |  | 2 for 2Tx cases 4 for 4Tx cases | 2 for 2Tx cases 4 for 4Tx cases |
|  | v-shift |  | 1 | 2 |
| PDSCH loading level |  |  | 20% probability of occurrence of LTE data transmission in time domain, and full bandwidth allocation in frequency domain. | 20% probability of occurrence of LTE data transmission in time domain, and full bandwidth allocation in frequency domain. |
| Modulation order for interference PDSCH when exists |  |  | 16 QAM randomly modulated symbols | 16 QAM randomly modulated symbols |
| Time offset |  | us | 3 | -1 |
| Frequency offset |  | Hz | 300 | -100 |
| Transmission rank |  |  | 80% and 20% probability for rank 1 and rank 2 respectively | 80% and 20% probability for rank 1 and rank 2 respectively |
| Propagation conditions and MIMO configuration (Note 1) |  |  | TDLA30-10 ULA Low | TDLA30-10 ULA Low |
| Network-based CRS interference mitigation (CRS muting) |  |  | Disabled | Disabled |
| MBSFN configuration |  |  | No MBSFN is configured | No MBSFN is configured |
| Note 1: The channel for interference cells and serving cell are independent. |  |  |  |  |

Table 2-3:	TDD 15kHz SCS simulation assumptions for CRS-IM under scenario 1 (LTE interference cells configuration)

| Parameter |  | Value | Interference Cell #1 | Interference Cell #2 |
| --- | --- | --- | --- | --- |
| Transmission mode |  |  | TM4 | TM4 |
| Interference power level |  | dB | INR1=10.45dB | INR2=4.6dB |
| TDD UL-DL pattern (Note 1) |  |  | DSUDDDSUDD S = 10D + 2G + 2U | DSUDDDSUDD S = 10D + 2G + 2U |
| CRS pattern | LTE carrier centre subcarrier location |  | Same as NR serving carrier centre subcarrier location | Same as NR serving carrier centre subcarrier location |
|  | LTE carrier BW | MHz | 20 | 20 |
|  | Number of antenna ports |  | 2 for 2Tx cases 4 for 4Tx cases | 2 for 2Tx cases 4 for 4Tx cases |
|  | v-shift |  | 1 | 2 |
| PDSCH loading level |  |  | 20% probability of occurrence of LTE data transmission in time domain, and full bandwidth allocation in frequency domain. | 20% probability of occurrence of LTE data transmission in time domain, and full bandwidth allocation in frequency domain. |
| Modulation order for interference PDSCH when exists |  |  | 16 QAM randomly modulated symbols | 16 QAM randomly modulated symbols |
| Time offset |  | us | 3 | -1 |
| Frequency offset |  | Hz | 300 | -100 |
| Transmission rank |  |  | 80% and 20% probability for rank 1 and rank 2 respectively | 80% and 20% probability for rank 1 and rank 2 respectively |
| Propagation conditions and MIMO configuration (Note 2) |  |  | TDLA30-10 ULA Low | TDLA30-10 ULA Low |
| Network-based CRS interference mitigation (CRS muting) |  |  | Disabled | Disabled |
| MBSFN configuration |  |  | No MBSFN is configured | No MBSFN is configured |
| Note 1: The start of transmission of LTE frame is delayed by 2 LTE subframes with respect to the start of transmission of NR frame. Note 2: The channel for interference cells and serving cell are independent. |  |  |  |  |

Simulation assumptions for CRS-IM under scenario 2

Scenario 2: Serving cell is operating in NR mode and interference cell is operating in LTE mode.

Table 3-1:	15kHz SCS simulation assumptions for CRS-IM under scenario 2 (Serving cell configuration)

| Parameter |  | Unit | FDD 15kHz SCS Case | TDD 15kHz SCS Case |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 | 20 |
| Subcarrier spacing |  | kHz | 15 | 15 |
| TDD UL-DL pattern |  |  | N/A | DDDSU S = 10D+2G+2U |
| Rank |  |  | 1 | 1 |
| MCS |  |  | 13 | 13 |
| MCS table |  |  | Table 1 | Table 1 |
| Duplex mode |  |  | FDD | TDD |
| Active DL BWP index |  |  | 1 | 1 |
| NR UL transmission with a 7.5 kHz shift to the LTE raster |  |  | false | false |
| Propagation channel |  |  | TDLA 30-10 | TDLA 30-10 |
| Antenna configuration |  |  | ULA Low 4 x 2 and 4 x 4 | ULA Low 4 x 2 and 4 x 4 |
| Receiver Type |  |  | MMSE-IRC | MMSE-IRC |
| Overhead for TBS determination |  |  | 0 | 0 |
| PDCCH configuration | Symbols with PDCCH |  | Symbol# 0 and #1 | Symbol# 0 and #1 |
| PDSCH configuration | Mapping type |  | Type A | Type A |
|  | k0 |  | 0 | 0 |
|  | Starting symbol (S) |  | 2 | 2 |
|  | Length (L) |  | 12 | 12 |
|  | PDSCH aggregation factor |  | 1 | 1 |
|  | PRB bundling type |  | Static | Static |
|  | PRB bundling size |  | 2 | 2 |
|  | Resource allocation type |  | Type 0 | Type 0 |
|  | RBG size |  | Config2 | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 | Type 1 |
|  | Position of the first DM-RS for downlink |  | 2 | 2 |
|  | Number of additional DMRS |  | 1 | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 | 1 |
| Number of HARQ Processes |  |  | 4 | 8 |
| Max HARQ re-transmission number |  |  | 4 | 4 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |
| PDSCH & PDSCH DMRS Precoding configuration |  |  | Single Panel Type I, Random precoder selection updated per slot with PRB bundling granularity | Single Panel Type I, Random precoder selection updated per slot with PRB bundling granularity |
| LTE interference cell configuration |  |  | Specified in Table 3-2 | Specified in Table 3-3 |
| Reference channel |  |  | FDD Sce 2 | TDD Sce 2 |

Table 3-2:	FDD 15kHz SCS simulation assumptions for CRS-IM under scenario 2 (LTE interference cells configuration)

| Parameter |  | Value | Interference Cell #1 | Interference Cell #2 |
| --- | --- | --- | --- | --- |
| Transmission mode |  |  | TM4 | TM4 |
| Interference power level |  | dB | INR1=10.45dB | INR2=4.6dB |
| CRS pattern | LTE carrier centre subcarrier location |  | Same as NR serving carrier centre subcarrier location | Same as NR serving carrier centre subcarrier location |
|  | LTE carrier BW | MHz | 10 | 10 |
|  | Number of antenna ports |  | 4 | 4 |
|  | v-shift |  | 1 | 2 |
| PDSCH loading level |  |  | 20% probability of occurrence of LTE data transmission in time domain, and full bandwidth allocation in frequency domain. | 20% probability of occurrence of LTE data transmission in time domain, and full bandwidth allocation in frequency domain. |
| Modulation order for interference PDSCH when exists |  |  | 16 QAM randomly modulated symbols | 16 QAM randomly modulated symbols |
| Time offset |  | us | 3 | -1 |
| Frequency offset |  | Hz | 300 | -100 |
| Transmission rank |  |  | 80% and 20% probability for rank 1 and rank 2 respectively | 80% and 20% probability for rank 1 and rank 2 respectively |
| Propagation conditions and MIMO configuration (Note 1) |  |  | TDLA30-10 ULA Low | TDLA30-10 ULA Low |
| Network-based CRS interference mitigation (CRS muting) |  |  | Disabled | Disabled |
| MBSFN configuration |  |  | No MBSFN is configured | No MBSFN is configured |
| Note 1: The channel for interference cells and serving cell are independent. |  |  |  |  |

Table 3-3:	TDD 15kHz SCS simulation assumptions for CRS-IM under scenario 2 (LTE interference cells configuration)

| Parameter |  | Value | Interference Cell #1 | Interference Cell #2 |
| --- | --- | --- | --- | --- |
| Transmission mode |  |  | TM4 | TM4 |
| Interference power level |  | dB | INR1=10.45dB | INR2=4.6dB |
| TDD UL-DL pattern (Note 1) |  |  | DSUDDDSUDD S = 10D + 2G + 2U | DSUDDDSUDD S = 10D + 2G + 2U |
| CRS pattern | LTE carrier centre subcarrier location |  | Same as NR serving carrier centre subcarrier location | Same as NR serving carrier centre subcarrier location |
|  | LTE carrier BW | MHz | 20 | 20 |
|  | Number of antenna ports |  | 4 | 4 |
|  | v-shift |  | 1 | 2 |
| PDSCH loading level |  |  | 20% probability of occurrence of LTE data transmission in time domain, and full bandwidth allocation in frequency domain. | 20% probability of occurrence of LTE data transmission in time domain, and full bandwidth allocation in frequency domain. |
| Modulation order for interference PDSCH when exists |  |  | 16 QAM randomly modulated symbols | 16 QAM randomly modulated symbols |
| Time offset |  | us | 3 | -1 |
| Frequency offset |  | Hz | 300 | -100 |
| Transmission rank |  |  | 80% and 20% probability for rank 1 and rank 2 respectively | 80% and 20% probability for rank 1 and rank 2 respectively |
| Propagation conditions and MIMO configuration (Note 2) |  |  | TDLA30-10 ULA Low | TDLA30-10 ULA Low |
| Network-based CRS interference mitigation (CRS muting) |  |  | Disabled | Disabled |
| MBSFN configuration |  |  | No MBSFN is configured | No MBSFN is configured |
| Note 1: The start of transmission of LTE frame is delayed by 2 LTE subframes with respect to the start of transmission of NR frame. Note 2: The channel for interference cells and serving cell are independent. |  |  |  |  |

- References

- R4-2207239 “Way forward general part and 15kHz NR SCS scenario for CRS-IM receiver”, China Telecom, RAN4 #102-e, March 2022.

- Annex

Table A.1: PDSCH Reference Channel for FDD CRS-IM simulation

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | FDD Sce 1 2Tx | FDD Sce 1 4Tx | FDD Sce 2 |  |  |
| Channel bandwidth | MHz | 10 | 10 | 10 |  |  |
| Subcarrier spacing | kHz | 15 | 15 | 15 |  |  |
| Number of allocated resource blocks | PRBs | 52 | 52 | 52 |  |  |
| Number of consecutive PDSCH symbols |  | 9 | 9 | 12 |  |  |
| Allocated slots per 2 frames | Slots | 16 | 16 | 16 |  |  |
| MCS table |  | 64QAM | 64QAM | 64QAM |  |  |
| MCS index |  | 13 | 13 | 13 |  |  |
| Modulation |  | 16QAM | 16QAM | 16QAM |  |  |
| Target Coding Rate |  | 0.48 | 0.48 | 0.48 |  |  |
| Number of MIMO layers |  | 1 | 1 | 1 |  |  |
| Number of DMRS REs |  | 12 | 12 | 12 |  |  |
| Overhead for TBS determination |  | [12] | 18 | 0 |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots i = 0,5,10,15 | Bits | N/A | N/A | N/A |  |  |
| For Slots i, if mod(i, 5) = {1,2,3,4} for i from {0,…,19} | Bits | 8456 | 7680 | 13064 |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots i = 0,5,10,15 | Bits | N/A | N/A | N/A |  |  |
| For Slots i, if mod(i, 5) = {1,2,3,4} for i from {0,…,19} | Bits | 2 | 1 | 2 |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots i = 0,5,10,15 | CBs | N/A | N/A | N/A |  |  |
| For Slots i, if mod(i, 5) = {1,2,3,4} for i from {0,…,19} | CBs | 24 | 24 | 24 |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots i = 0,5,10,15 | Bits | N/A | N/A | N/A |  |  |
| For Slots i = 11 | Bits | 16224 | 15392 | 22880 |  |  |
| For Slots i, if mod(i, 5) = {1,2,3,4} for i from {1,…, 9, 12, …, 19} | Bits | 17472 | 16640 | 24128 |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 6.764 | 6.144 | 10.4512 |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: No user data is scheduled on slots with LTE PBCH/PSS/SSS |  |  |  |  |  |  |

Table A.2: PDSCH Reference Channel for TDD CRS-IM simulation with UL-DL pattern FR1.15-1

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | TDD Sce 1 2Tx | TDD Sce 1 4Tx | TDD Sce 2 |  |  |
| Channel bandwidth | MHz | 20 | 20 | 20 |  |  |
| Subcarrier spacing | kHz | 15 | 15 | 15 |  |  |
| Allocated resource blocks | PRBs | 106 | 106 | 106 |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} |  | N/A | N/A | N/A |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,19} |  | 9 | 9 | 12 |  |  |
| Allocated slots per 2 frames |  | 7 | 7 | 7 |  |  |
| MCS table |  | 64QAM | 64QAM | 64QAM |  |  |
| MCS index |  | 13 | 13 | 13 |  |  |
| Modulation |  | 16QAM | 16QAM | 16QAM |  |  |
| Target Coding Rate |  | 0.48 | 0.48 | 0.48 |  |  |
| Number of MIMO layers |  | 1 | 1 | 1 |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} |  | N/A | N/A | N/A |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,19} |  | 12 | 12 | 12 |  |  |
| Overhead for TBS determination |  | [12] | 18 | 0 |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 5) = {2,3,4} for i from {0,…,19} | Bits | N/A | N/A | N/A |  |  |
| For Slot i, if mod(i, 5) = {0,1} for i from {1,…,19} | Bits | 16896 | 15880 | 26632 |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 5) = {2,3,4} for i from {0,…,19} | Bits | N/A | N/A | N/A |  |  |
| For Slot i, if mod(i, 5) = {0,1} for i from {1,…,19} | Bits | 3 | 2 | 4 |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 5) = {2,3,4} for i from {0,…,19} | CBs | N/A | N/A | N/A |  |  |
| For Slot i, if mod(i, 5) = {0,1} for i from {1,…,19} | CBs | 24 | 24 | 24 |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 5) = {2,3,4} for i from {0,…,19} | Bits | N/A | N/A | N/A |  |  |
| For Slots i = 10, 11 | Bits | 34368 | 32672 | 47936 |  |  |
| For Slot i, if mod(i, 5) = {0,1} for i from {1,…,9,12,…,19} | Bits | 35616 | 33920 | 49184 |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 5.9136 | 5.558 | 9.3212 |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: No user data is scheduled on slots with LTE PBCH/PSS/SSS |  |  |  |  |  |  |