3GPP TSG-RAN WG4 # 103-e	R4-229800

Electronic Meeting, May 09 – May 20, 2022

Agenda item:	9.19.5.3.2

Source:	MediaTek Inc.

Title:	Simulation results and discussion on the PMI requirements for RedCap

Document for:	Discussion

- Introduction

In RAN4#102-e, a way forward on RedCap was agreed [1]. In this contribution, based on the agreed simulation assumptions, we provide simulation results for PMI requirements for RedCap UE.

- Discussion

We provide simulation results in Table 2 and Table 4 based on the assumption in Table 1 and Table 3 respectively.

Table 1. Simulation assumptions for FDD

| 1Rx UE or 2Rx UE | Reference | PMI test | CBW / SCS | Tx ports | Antenna Configuration |
| --- | --- | --- | --- | --- | --- |
| 1Rx | Table 6.3.2.1.1-1 Test 1 | Single PMI, Type I, rank 1 | 10 / 15 | 4 | 4 x 1 ULA high |

Table 2. Simulation results for FDD

| MCS | SNR @ 90% Maximum throughput | Throughput ratio γ |
| --- | --- | --- |
| 13 | 7.6 | 2.3 |

Table 3. Simulation assumptions for TDD

| 1Rx UE or 2Rx UE | Reference | PMI test | CBW / SCS | Tx ports | Antenna Configuration | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- |
| 1Rx | Table 6.3.2.2.1-1 Test 1 | Single PMI, Type I, rank 1 | 20 / 30 | 4 | 4 x 1 ULA high | New |
| 2Rx | Table 6.3.2.2.1-1 Test 1 | Single PMI, Type I, rank 1 | 20 / 30 | 4 | 4 x 2 XP high | New |

Table 4. Simulation results for TDD

| 1Rx |  |  | 2Rx |  |  |
| --- | --- | --- | --- | --- | --- |
| MCS | SNR @ 90% Maximum throughput | Throughput ratio γ | MCS | SNR @ 90% Maximum throughput | Throughput ratio γ |
| 13 | 6.5 | 2.2 | 13 | 2.9 | 2.1 |

According to the simulation results, we suggest using the following MCS and throughput ratio γ to define requirements on PMI for RedCap UE.

- FDD, 1Rx: MCS 13, γ = 1.3

- TDD, 1Rx: MCS 13, γ = 1.3

- TDD, 2Rx: MCS 15, γ = 1.3

- Conclusion

In this contribution, we provide simulation results on PMI requirements for RedCap based on the agreed simulation assumption in [1]. The proposals are summarized as below:

Proposal 1: Consider using the following MCS and throughput ratio γ to define requirements on PMI for RedCap UE.

- FDD, 1Rx: MCS 13, γ = 1.3

- TDD, 1Rx: MCS 13, γ = 1.3

- TDD, 2Rx: MCS 15, γ = 1.3

- Reference

- R4-2207206, “WF on RedCap demodulation and CQI reporting requirements”, Ericsson