3GPP TSG-RAN WG4 Meeting #103-e	R4-2209866

# Electronic Meeting, 9th – 20th May, 2022

Title: 	Simulation results on PUSCH with UL timing adjustment requirements for FR2 HST

Source: 	Huawei, HiSilicon

Agenda item:	9.8.4.2.2

Document for:	Information

Background

During RAN#102-e meeting, WF [2] on FR2 HST BS demodulation was approved. In this contribution, we provide our updated simulation results for the PUSCH with UL timing adjustment demodulation requirements for FR2 HST.

Simulations

Here we provide the updated simulation results for FR2 HST PUSCH with UL timing adjustment for alignment, as shown in Table 2-2, based on simulation assumption as shown in Table 2-1.

Table 2-1 Simulation assumption for FR2 HST PUSCH with UL timing adjustment

| Parameters | Value |
| --- | --- |
| Channel Model | Stationary UE: AWGN, Moving UE: AWGN |
| UE speed | 350km/h |
| Waveform | CP-OFDM |
| CP length | Normal |
| A | AWGN |
| Δω | 1T2R |
| CBW | 50MHz and 200MHz |
| DM-RS type | Type 1 |
| Mapping type | B |
| Length of PUSCH data | 10 |
| RS configuration | 1DMRS+PT-RS(L=1,K=2), 2DMRS+PT-RS(L=1,K=2), 3DMRS+PT-RS(L=1,K=2) |
| Number of DM-RS CDM group(s) without data | 2 |
| Ratio of PUSCH EPRE to DM-RS EPRE | -3dB |
| DM-RS port | {0} |
| DM-RS sequence generation | NID0=0, nSCID =0 for moving UE NID0=1, nSCID =1 for stationary UE |
| PUSCH resource allocation | 50MHz: 0 to 15 RB for moving UE, 16 to 31 for stationary UE (if 100MHz 200MHz: 0 to 65RB for moving UE, 66 to 131 for stationary UE |
| SRS resource allocation | 50MHz CBW (32RBs)~C_SRS =9, B_SRS =0 200MHz CBW (132RBs)~ C_SRS=33, B_SRS=0 |

Table 2-2 Simulation results for FR2 HST PUSCH with UL timing adjustment

| Case number | CBW(MHz) | DMRS | SNR @ 70% maximum throughput [dB] |  |
| --- | --- | --- | --- | --- |
|  |  |  | Ideal | Impairment |
| 1 | 50 | 1 | 6.56 | 9.06 |
| 2 | 50 | 1+1 | 6.41 | 8.91 |
| 3 | 50 | 1+1+1 | 6.43 | 8.93 |
| 4 | 200 | 1 | 6.40 | 8.90 |
| 5 | 200 | 1+1 | 6.40 | 8.90 |
| 6 | 200 | 1+1+1 | 6.40 | 8.90 |

Conclusion

In this contribution, we provide our updated simulation results for PUSCH with UL timing adjustment performance requirements for NR HST FR2.

Reference

R4-2120703, WF on BS demodulation requirement for FR2 HST, RAN4#101-e, Samsung

R4-2207235, WF on BS demodulation requirement for FR2 HST, RAN4#102-e, Nokia, Nokia Shanghai Bell