3GPP TSG-RAN WG4 Meeting #103-e	R4-2209882

# Electronic Meeting, 9th – 20th May, 2022

Title: 	Simulation results on satellite NTN demod PRACH

Source: 	Huawei, HiSilicon

Agenda item:	9.12.8.2.3

Document for:	Information

Background

During RAN4#102-e meeting, WF [1] on NTN SAN demodulation requirements was approved. In this contribution, we provide our initial simulation results about the NTN PRACH demodulation requirements.

Simulation

PRACH

Here we provide the initial simulation results for PRACH for alignment, as shown in Table 2.1-2, based on simulation assumption as shown in Table 2.1-1.

Table 2.1-1 Simulation assumption for PRACH

| Parameter | Value |
| --- | --- |
| Preamble format | A2/B4/C2 |
| Antenna configuration | 2/4/8 Rx for LRA=139, 2Rx for LRA=1151 and 571 |
| SCS | 15kHz for LRA=139 and 1151, 30kHz for LRA=139 and 571 |
| Propagation | NTN-TDLA100-200 |
| Time error tolerance | 0.522μ+Tdelay (Same formula as legacy BS requirements) |
| Frequency offset | 200Hz |

Table 2.1-2 Ideal simulation results for PRACH

| Case Number | SCS (kHz) | Bandwidth (MHz) | LRA | Antenna configuration | SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | A2 | B4 | C2 |
| 1 | 15 | 5 | 139 | 1x2 | -5.72 | -10.03 | -5.57 |
| 2 | 15 | 5 | 139 | 1x4 | -10.52 | -14.09 | -10.85 |
| 3 | 15 | 5 | 139 | 1x8 | -14.34 | -17.37 | -14.46 |
|  | 15 | 5 | 1151 | 1x2 | -16.14 | -19.78 | -16.08 |
| 4 | 30 | 10 | 139 | 1x2 | -6.81 | -10.29 | -6.75 |
| 5 | 30 | 10 | 139 | 1x4 | -11.08 | -14.07 | -11.07 |
| 6 | 30 | 10 | 139 | 1x8 | -14.23 | -17.22 | -14.27 |
|  | 30 | 10 | 571 | 1x2 | -13.00 | -16.38 | -12.80 |

Conclusion

In this contribution, we provide our initial simulation results on NTN PRACH demodulation requirements.

Reference

R4-2207198, WF on NTN SAN demodulation requirements, RAN4#102-e, Huawei, HiSilicon