3GPP TSG-RAN WG4 Meeting # 103-e	R4-2210031

# Electronic Meeting, 9 - 20 May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-2 | CR | <CR#> | rev | <Rev#> | Current version: | 15.13.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.1041-2 on clarifications of interfering signal for the OTA transmitter intermodulation requirement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Huawei, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For the interfering signal for the OTA transmitter intermodulation requirement, it is not clear how the power is split between the supported polarizations, and whether the power is split when the power is 46 dBm but not Prated,t,TRP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify the power is split equally between the supported polarizations - other clarifications related to polarization match and polarization of CLTA |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguities remain and would lead to different interpretations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.12.2, 6.8.4.2, 6.8.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 37.105, 38.104 CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 37.145-2 CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

#### 4.12.2.2	Co-location test antenna characteristics

A co-location test antenna is a practical passive antenna that is used for conformance testing of the co-location requirements and is based on the definition of the co-location reference antenna. A CLTA shall comply with the requirements specified in table 4.12.2.2-1.

Translation of the requirements to other test antennas are not precluded but suitable translations between the co-location reference antenna and test antenna must be provided to demonstrate that the method is within the specified MU.

NOTE:	The currently defined CLTAs are suitable for testing BS type 1-O implemented with a planar antenna array. The method for testing BS with other antenna array implementations is not covered by the present release of this specification.

Table 4.12.2.2-1: CLTA characteristics

| Parameter | In-band CLTA | Out-of-band CLTAs |
| --- | --- | --- |
| Vertical radiating dimension (h) | Test object vertical radiating length ±30% | Test object vertical radiating length ±30% (Note 2) |
| Horizontal beam width | 65° ± 10° | 65° ± 10° |
| Vertical beam width | N/A | The half-power vertical beam width of the CLTA equals the narrowest declared (D.3) vertical beamwidth ±3° (Note 2) |
| Polarization (Note 3) | Match (Note4) | Match to in-band (Note 4) |
| Conducted interface return loss | > 10 dB | > 10 dB |
| NOTE 1: If a multi-column or multi-band antenna is used the column closest to the NR BS shall be selected while other columns are terminated during testing. NOTE 2: The vertical radiating dimension definition shall be used instead of the vertical beam width definition when the test chamber dimensions limit the use of vertical beam width definition. Otherwise the vertical beam width definition shall be used. NOTE 3: For BS type 1-O with dual polarization the CLTA has two conducted interfaces each representing one polarization NOTE 4: Matched to the polarization of EUT antenna |  |  |

<Endof change>erd

<Start of change>

## 6.8	OTA transmitter intermodulation

### 6.8.1	Definition and applicability

The OTA transmitter intermodulation requirement is a measure of the capability of the transmitter unit to inhibit the generation of signals in its non-linear elements caused by presence of the wanted signal and an interfering signal reaching the transmitter unit via the RDN and antenna array from a co-located base station. The requirement applies during the transmitter ON period and the transmitter transient period.

The requirement applies at each RIB supporting transmission in the operating band.

The transmitter intermodulation level is the total radiated power of the intermodulation products when an interfering signal is injected into the CLTA.

For BS type 1-O, the transmitter intermodulation requirement is captured by the co-location transmitter intermodulation scenario case, in which the interfering signal is injected into the CLTA.

### 6.8.2	Minimum requirement

The minimum requirement for BS type 1-O operation is defined in TS 38.104 [2], clause 9.8.2.

The OTA transmitter intermodulation requirement is not applicable for BS type 2-O.

### 6.8.3	Test purpose

The test purpose is to verify the ability of the transmitter units associated with the RIB under test to restrict the generation of intermodulation products in its nonlinear elements caused by presence of the wanted signal and an interfering signal reaching the transmitter unit via the RDN and antenna array from a co-located base station to below specified levels.

### 6.8.4	Method of test

#### 6.8.4.1	Initial conditions

Test environment: normal; see annex B.2.

RF channels to be tested for single carrier:	M; see clause 4.9.1.

Base Station RF Bandwidth positions to be tested for multi-carrier:

-	MRFBW in single-band RIB, see clause 4.9.1;

-	BRFBW_T'RFBW and B'RFBW_TRFBW in multi-band RIB, see clause 4.9.1.

In addition, for multi-band RIB:

-	For BRFBW_T'RFBW, emission testing above the highest operating band may be omitted.

-	For B'RFBW_TRFBW, emission testing below the lowest operating band may be omitted.

Directions to be tested: The FR1 requirement is specified as co-location requirement. For general description of co-location requirements, refer to clause 4.12.

#### 6.8.4.2	Procedure

1)	Select a CLTA according to the description in clause 4.12 and parameters given in table 4.12.2.2-1.

2)	Place the CLTA according to the description in clause 4.12 and parameters given in table 4.12.2.3-1.

3)	The test antenna(s) shall be dual (or single) polarized covering the same frequency range as the NR BS and the emission frequencies.

4)	Several test antennas are required to cover both the NR BS and the whole emission frequency range.

5)	Connect test antenna and CLTA to the measurement equipment as shown in annex E.1.5.

6)	During the OTA emission measurements at the test antenna conducted output(s), both NR BS and CLTA are rotated around same axis.

7)	The OTA emission measurement method shall be TRP, according to the procedure described in annex I.

8)	The measurement device (signal analyzer) characteristics shall be:

-	Detection mode: True RMS.

9)	Set the BS type 1-O to transmit:

-	Set the NR BS to transmit maximum power according to the applicable test configuration in clause 4.8 using the corresponding test models or set of physical channels in clause 4.9.2.

-	For the NR BS declared to be capable of multi-carrier and/or CA operation, set the BS to transmit according to the applicable test configuration and corresponding power setting specified in clause 4.7.2 and 4.8 using the corresponding test models on all carriers configured.

10)	Generate the interfering signal

using test model as defined in clause 4.9.2, at a centre frequency offset according to the conditions in table 9.8.2-1 in TS 38.104 [2], but exclude interfering frequencies that are outside of the allocated downlink operating band or interfering frequencies that are not completely within the sub-block gap or within the Inter RF Bandwidth gap.

11)	Connect the interfering signal to the CLTA input interfaces, equally dividing the power among supported polarizations. Adjust the interfering signal level at the CLTA conducted input(s) as defined in table 6.8.5.1-1.

12)	If the interfering signal is applicable according to clause 4.7, perform the unwanted emission tests specified in clauses 6.7.3 (OTA ACLR) and 6.7.4 (OTA OBUE) for all third and fifth order intermodulation products which appear in the frequency ranges defined in clauses 6.7.3 and 6.7.4 (Note 2). The width of the intermodulation products shall be taken into account.

13)	If the interfering signal is applicable according to clause 4.7, perform the Transmitter spurious emissions test as specified in clause 6.7.5 (OTA spurious emission), except OTA co-location spurious emission, for all third and fifth order intermodulation products which appear in the frequency ranges defined in clause 6.7.5 (Note 2). The width of the intermodulation products shall be taken into account.

14)	Verify that the emission level does not exceed the required level in clause 6.8.5 (Test requirements) with the exception of interfering signal frequencies.

15)	Repeat the test for the remaining interfering signal centre frequency offsets

defined in table 6.8.5.1-1.

16)	Repeat the test for the remaining interfering signals defined in clause 4.7 for requirements 6.7.3 (OTA ACLR), 6.7.4 (OTA OBUE) and 6.7.5 (OTA spurious emission), except OTA co-location spurious emission.

In addition, for multi-band RIB, the following steps shall apply:

17)	For multi-band RIB and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

NOTE 1:	The third order intermodulation products are centred at 2F1F2 and 2F2F1. The fifth order intermodulation products are centred at 3F12F2, 3F22F1, 4F1F2, and 4F2F1 where F1 represents the test signal centre frequency or centre frequency of each sub-block and F2 represents the interfering signal centre frequency. The widths of intermodulation products are:

-	(n*BWF1 + m* BWF2) for the nF1mF2 products;

-	(n* BWF2 + m* BWF1) for the nF2mF1 products;

where BWF1 represents the test wanted signal RF bandwidth or channel bandwidth in case of single carrier, or sub-block bandwidth and BWF2 represents the interfering signal channel bandwidth.

NOTE 2:	During the conformance test the interferer signal can be applied on one side of the wanted signal, while the transmitter intermodulation emission is measured only on the opposite side of the wanted signal. This applies for intermodulation products which are within the operating band or OBUE region.

### 6.8.5	Test requirements

#### 6.8.5.1	Requirement for BS type 1-O

The transmitter intermodulation level shall not exceed the TRP unwanted emission limits specified for OTA transmitter spurious emission in clause 6.7.5 (except co-location with other base stations), OTA out-of-band emissions in clause 6.7.4 and OTA ACLR in clause 6.7.3 in the presence of a wanted signal and an interfering signal, defined in table 6.8.5.1-1.

The requirement is applicable outside the Base Station RF Bandwidth edges. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Radio Bandwidth edges.

For RIBs supporting operation in non-contiguous spectrum, the requirement is also applicable inside a sub-block gap for interfering signal offsets where the interfering signal falls completely within the sub-block gap. The interfering signal offset is defined relative to the sub-block edges.

For RIBs supporting operation in multiple operating bands, the requirement shall apply relative to the Base Station RF Bandwidth edges of each operating band. In case the inter RF Bandwidth gap is less than 3*BWChannel MHz (where BWChannel is the minimal BS channel bandwidth of the band), the requirement in the gap shall apply only for interfering signal offsets where the interfering signal falls completely within the inter RF Bandwidth gap.

Table 6.8.5.1-1: Interfering and wanted signals for the OTA transmitter intermodulation requirement

| Parameter | Value |
| --- | --- |
| Wanted signal | NR single or multi-carrier, or multiple intra-band contiguously or non-contiguously aggregated carriers |
| Interfering signal type | NR signal, the minimum BS channel bandwidth (BWChannel) with 15 kHz SCS of the band defined in clause 5.3.5 of TS 38.104 [2] |
| Interfering signal power level | min(46 dBm, Prated,t,TRP) |
| Interfering signal centre frequency offset from the lower (upper) edge of the wanted signal or edge of sub-block inside a gap | , for n=1, 2 and 3 |
| NOTE 1: Interfering signal positions that are partially or completely outside of any downlink operating band of the BS are excluded from the requirement, unless the interfering signal positions fall within the frequency range of adjacent downlink operating bands in the same geographical area. NOTE 2: In Japan, note 1 is not applied in Band n77, n78, n79. NOTE 3: For BS Type 1-O supporting dual polarization, the interfering signal power shall be equally divided between supported polarizations at the CLTA interfaces. |  |

<Endof change>