3GPP TSG-RAN WG4 Meeting # 103-e	R4-2210040

# Electronic Meeting, May 09 – May 20, 2022

Source: 	Huawei

Title: 	TP to TS 38.108: removal of extreme conditions requirements

Agenda Item:		9.12.4.1

Document for:	Approval

# Introduction

Based on related discussion paper in [1], TP to TS 38.108 is provided to remove the extreme conditions requirements from the NTN SAN specification.

# Conclusions

Based on the discussion above, the following proposals are formulated:

Proposal 1: Approve the attached TP to TS 38.108.

# References

[1]	R4-2210039	Further discussion on the Normal and Extreme conditions testing

# TP to TS 38.108

------------------------------ Modified section ------------------------------

### 6.2.2	Minimum requirement for SAN type 1-H

In normal conditions, Pmax,c,TABC shall remain within +2 dB and -2 dB of the rated carrier output power Prated,c,TABC for each TAB connector as declared by the manufacturer.

------------------------------  Next modified section ------------------------------

### 9.2.2	Minimum requirement for SAN type 1-H and SAN type 1-O

For each declared beam, in normal conditions, for any specific beam peak direction associated with a beam direction pair within the OTA peak directions set, a manufacturer claimed EIRP level in the corresponding beam peak direction shall be achievable to within ±2.2 dB of the claimed value.

Normal conditions are defined in TS 38.181, annex B [3].

In certain regions, the minimum requirement for normal conditions may apply also for some conditions outside the range of conditions defined as normal.

------------------------------ End of modified section ------------------------------