3GPP TSG-RAN WG4 Meeting # 103-e                     R4-2210042

Electronic Meeting, 9st of May – 20th of May, 2022

Agenda item:	     9.12.3.4

Source:	THALES

Title:	pCR for Clause 7.5 Out-of-band blocking - TS 38.108

Document for:	Approval

# Introduction

Draft TS skeleton for Non-terrestrial networks (NTN) related to NR; Satellite Access Node radio transmission and reception (TS 38.108) has been approved in R4-2203087 [1] and the work split has been summarized in R4-2203080 Way Forward on NTN_solutions_Part1 [2] approved in RAN4 101-bis-e meeting.

For further information, please also follow the discussions in R4-2203111 [3] and the updated TS 38.108 at RAN4 102-e meeting, and which can be found in R4-2207513 [4].

In this document, following changes have been proposed as a pCR to update TS 38.108:

- Section 7.5: Out-of-band blocking

# Reference

[1] R4-2203087 Skeleton for TS 38.108 NR Satellite Access Node radio transmission and reception v0.0.1, THALES.

[2] R4-2203080 Way Forward on NTN_solutions_Part1, THALES.

[3] R4-2203111 Email discussion summary for [101-bis-e][306] NTN_Solutions_Part1, THALES.

[4] R4-2207513 TS 38.108 v0.1.0, THALES.

# Text proposals for TS 38.108

------------------------------------------------<Start of TP>----------------------------------------------

## 7.5	Out-of-band blocking

### 7.5.1	General

The out-of-band blocking characteristics is a measure of the receiver ability to receive a wanted signal at its assigned channel at the TAB connector for SAN type 1-H in the presence of an unwanted interferer out of the operating band, which is a CW signal for out-of-band blocking.

### 7.5.2	Minimum requirements for Satellite Access Node

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel, with a wanted and an interfering signal coupled to SAN type 1-H TAB connector using the parameters in table 7.5.2-1.

The reference measurement channel for the wanted signal is identified in clause 7.2.2 for each SAN channel bandwidth and further specified in annex A.1.

The out-of-band blocking requirement apply from 1 MHz to FUL,low - ΔfOOB and from FUL,high + ΔfOOB up to 12750 MHz, including the downlink frequency range of the FDD operating band for SAN. The ΔfOOB for SAN type 1-H is defined in table 7.5.2-2.

Minimum conducted requirement is defined at the antenna connector at the TAB connector for SAN type 1-H.

Table 7.5.2-1: Out-of-band blocking requirement for NR

| Wanted Signal mean power (dBm) | Interfering Signal mean power (dBm) | Type of Interfering Signal |
| --- | --- | --- |
| PREFSENS +6 dB (NOTE 1) | -44 | CW carrier |
| NOTE 1: For SAN, PREFSENS depends on the SAN channel bandwidth. |  |  |

Table 7.5.2-2: ΔfOOB offset for NR operating bands

| SAN type | Operating band characteristics | ΔfOOB (MHz) |
| --- | --- | --- |
|  | FUL,high – FUL,low < 100 MHz | 20 |
| SAN type 1-H | 100 MHz ≤ FUL,high – FUL,low ≤ 900 MHz | 60 |

------------------------------------------------<End of TP>-----------------------------------------------