3GPP TSG-RAN WG4 Meeting # 103-e              									     R4-2210090

Electronic Meeting, 9st of May – 20th of May, 2022

Agenda item:			9.12.3.3

Source:	THALES

Title:	Discussion on SAN Spurious Emissions

Document for:	Discussion

# Introduction

During the RAN4#102-bis-e BS RF GTW discussion on 23/02/2022 it was decided to consider 2 classes (1 for LEO and 1 for GEO). The LEO class incorporates both LEO@600km and LEO@1200km. Moreover, during the BS RF GTW discussion on 28/02/2022, it was decided to further discuss the Way Forward with respect to SAN SEM (SAN OBUE and SAN spurious emissions). As a result of GTW decisions and “Email discussion summary for [102-e][310] NTN_Solutions_Part3” (see R4-2207440), the following agreements have been made for SAN Spurious Emissions in the approved Way Forward on SAN SEM and spurious emission (R4-2207456), with the following options:

- Option 1 (Ericsson):

| Spurious frequency range | Basic limit GEO class (dBm) | Basic limit LEO class (dBm) | Measurement bandwidth |
| --- | --- | --- | --- |
| 9kHz – 12.75 GHz | [-13] | [-13] | 4 kHz |

FFS how to handle limit when satellite output power > 50W.

- Option 2 (THALES):

For satellites operating in MSS S-Band n256 and MSS L-band n255, the frequency range for measurements shall be applied as in Table x.x-1.

Table x.x-1. Frequency range for measurements

| Fundamental frequency range | Frequency range for measurements |  |
| --- | --- | --- |
|  | Lower limit | Upper limit (The test should include the entire harmonic band and not be truncated at the precise upper frequency limit stated) |
| 600 MHz - 5.2 GHz | 800 MHz | 11 GHz (5th harmonic) |

SAN spurious emissions limits for GEO and LEO classes with respect to MSS S-Band n256 and MSS L-band n255 are defined as described in Table x.x-2.

Table x.x-2. Basic spurious emission limit for GEO and LEO SAN classes

| Spurious frequency range | Basic limit GEO class (dBm) | Basic limit LEO class (dBm) | Measurement bandwidth | Notes |
| --- | --- | --- | --- | --- |
| 800 MHz - 1508 MHz | -9 | -7 | 4 kHz | Notes 1,2,3 |
| 1508 MHz - 1518 MHz | Covered by OBUE requirement |  | 4 kHz | Note 1 |
| 1518 MHz - 1559 MHz | NA: L-Band transmit Bandwidth |  | 4 kHz | Note 1 |
| 1559 MHz - 1569 MHz | Covered by OBUE requirement |  | 4 kHz | Note 1 |
| 1569 MHz - 2160 MHz | -9 | -7 | 4 kHz | Notes 1,3 |
| 2160 MHz - 2170 MHz | Covered by OBUE requirement |  | 4 kHz | Note 1 |
| 2170 MHz - 2200 MHz | NA: S-Band transmit Bandwidth |  | 4 kHz | Note 1 |
| 2200 MHz - 2210 MHz | Covered by OBUE requirement |  | 4 kHz | Note 1 |
| 2210 MHz- 11000 MHz | -9 | -7 | 4 kHz | Notes 1, 2 |
| NOTE 1: Measurement bandwidths as in ITU-R SM.329 [x], s4.1. NOTE 2: Lower and Upper frequency as in ITU-R SM.329 [x], s2.5 table 1. NOTE 3: As per ITU-R SM.329 [x], Spurious requirement is -60 dBc with respect to useful operating power P: 51-60= -9 dBm for GEO class and 53-60= -7 dBm for LEO class. |  |  |  |  |

- Option 3: Other solutions are not precluded.

# Discussion

As a general information, the general principle is to follow ITU-R recommendation SM.329-12 (Unwanted emissions in the spurious domain), with the measurement bandwidth of 4 kHz.

The SAN spurious emissions limit requirements for GEO and LEO classes are defined in Table 1:

Table 1. Basic spurious emission limit for GEO and LEO SAN classes

| Spurious frequency range | Basic limit GEO class (dBm) | Basic limit LEO class (dBm) | Measurement bandwidth | Notes |
| --- | --- | --- | --- | --- |
| 30 MHz – 5th harmonic (NOTE 2, 4, 5) | When Prated,c,sys ≤ 47dBm : -13 When Prated,c,sys > 47dBm : Prated,c,sys-60 ( NOTE 3) |  | 4 kHz (NOTE 1) | See NOTES 1, 2, 3, 4 ,5 |
| NOTE 1: Measurement bandwidths as in ITU-R SM.329 [x], s4.1. NOTE 2: Lower and Upper frequency as in ITU-R SM.329 [x], s2.5, Table 1. Applies only for the fundamental frequency range 600 MHz - 5.2 GHz (e.g. for n255 and n256 frequency bands). NOTE 3: Values as in ITU-R SM.329 [x], Table 10. NOTE 4: Lower frequency limit is replaced by 800 MHz, according to ITU-R SM.329-12, when using waveguide section, spurious domain emission measurements below 0.7 times the waveguide cut-off frequency are not required. NOTE 5: This spurious frequency range applies only to SAN type 1-H. |  |  |  |  |

Measurement bandwidths are as in ITU-R recommendation SM.329-12, section 4.1. Lower and upper frequencies are defined as in ITU-R SM.329-12, section 2.5, Table 1. Provided limits for the values are as in ITU-R SM.329-12, Table 10.

Moreover, the equation is no longer depending on the SAN transmission power (in SAN case obtained from the system bandwidth and transmitted power density) and has a general expression, taking into account all the potential cases.

Please also find the following justifications and comments (as also described in R4-2207389):

- The lower limit of 30 MHz is replaced as per ITU-R SM.329-12: « Systems having an integral antenna incorporating a waveguide section, or with an antenna connection in such form, and of unperturbed length equal to at least twice the cut-off wavelength, do not require spurious domain emission measurements below 0.7 times the waveguide cut-off frequency. »

- This lower limit (800 MHz) covers the 0.7 times the standards cut-off frequency of WR510, WR340 and WR430 waveguides to be used for L-Band and S-Band SAN type 1-H. As an example, the cut-off frequency of lowest order mode for WR510 waveguide is 1.157 GHz, and therefore 0.7x1.157 GHz = 808.9 MHz > 800 MHz proposed lower limit.

- The upper limit of 11 GHz corresponds to the 5th harmonic of maximum frequency in S-Band (2200 MHz).

- Tx spurious limit value shall not be applied within the transmit L-Band and S-Band.

Please also note that in SM.329-12 the 43+log(P) attenuation is applicable only if transmitter power < 50W, and attenuation of 60 dBc is applicable for transmitter power > 50W. In order to avoid any confusion with respect to LEO satellite class (if defined based on LEO@600km, LEO@1200km), transmitted power and bandwidth, we therefore propose to have a more general expression as previously described in Table 1:

- When Prated,c,sys ≤ 50W : SAN spurious emission limit of -13 dBm

- When Prated,c,sys > 50W : SAN spurious emission limit of 10*log(Prated,c,sys[W])-30 dBm

Alternatively, these two conditions can be written as:

- When Prated,c,sys ≤ 47dBm : SAN spurious emission limit of -13 dBm

- When Prated,c,sys > 47dBm : SAN spurious emission limit of Prated,c,sys[dBm]-60 dBm

# Conclusion

Proposal 1: Agree with the following SAN spurious emission limit requirements (updated from Option 2 (see R4-2207456)):

Table 1. Basic spurious emission limit for GEO and LEO SAN classes

| Spurious frequency range | Basic limit GEO class (dBm) | Basic limit LEO class (dBm) | Measurement bandwidth | Notes |
| --- | --- | --- | --- | --- |
| 30 MHz – 5th harmonic (NOTE 2, 4, 5) | When Prated,c,sys ≤ 47dBm : -13 When Prated,c,sys > 47dBm : Prated,c,sys-60 ( NOTE 3) |  | 4 kHz (NOTE 1) | See NOTES 1, 2, 3, 4 ,5 |
| NOTE 1: Measurement bandwidths as in ITU-R SM.329 [x], s4.1. NOTE 2: Lower and Upper frequency as in ITU-R SM.329 [x], s2.5, Table 1. Applies only for the fundamental frequency range 600 MHz - 5.2 GHz (e.g. for n255 and n256 frequency bands). NOTE 3: Values as in ITU-R SM.329 [x], Table 10. NOTE 4: Lower frequency limit is replaced by 800 MHz, according to ITU-R SM.329-12, when using waveguide section, spurious domain emission measurements below 0.7 times the waveguide cut-off frequency are not required. NOTE 5: This spurious frequency range applies only to SAN type 1-H. |  |  |  |  |