3GPP TSG-RAN WG4 Meeting # 103-e	R4- 2210112

# Electronic Meeting, May 9 – May 20, 2022

Agenda Item:	9.24.2

Source: 		Qualcomm Incorporated

Title: 			RRM performance requirements for CG-SDT

Document for:	Discussion

# 1.	Introduction

In this paper we are discussing about the test scope of RRM performance requirement for CG-SDT.

# 2. 	Discussion

As the CG-SDT occasion is predefined by NW and TA validation is implemented by UE based on measured RSRP, TA validation is one of the key requirements for CG-SDT procedure. Although the requirement between TA validation at T2 and CG-SDT occasion at T3 is under discussion, verifying TA validation would be desirable to test the feature requirements. If UE is static state, then measured RSRP is same in general. Thus, the test scenario should be mobility case.

Proposal: TA validation test requirement for CG-SDT can contain the following configurations/procedures.

- RSRP should be increasing or decreasing from RSRP1

- RSRP2 should meet the RSRP threshold when UE measured RSRP2 within [640ms] from T3.

- RSRP2 should not meet the RSRP threshold when UE measured RSRP2 more than [640ms] prior to T3.

# 3. Conclusions

Proposal: Test requirement for CG-SDT should contain the following configurations/procedures.

- RSRP should be increasing or decreasing from RSRP1

- RSRP2 should meet the RSRP threshold when UE measured RSRP2 within [640ms] from T3.

- RSRP2 should not meet the RSRP threshold when UE measured RSRP2 more than [640ms] prior to T3.

# 4.    References